http://www.coryat.com/faa-pp-written/index01.html

With respect to the certification of airmen, which is a category of

aircraft?

ANSWER: Airplane, rotorcraft, glider, lighter-than-air.

Category of aircraft, as used with

respect to the certification, ratings, privileges, and

limitations of airmen, means a broad classification of aircraft.

Examples include: airplane, rotorcraft, glider, and

lighter-than-air.

With respect to the certification of airmen, which is a class of

aircraft?

ANSWER: Single-engine land and sea, multiengine land and sea.

Class of aircraft, as used with respect

to the certification, ratings, privileges, and limitations of

airmen, means a classification of aircraft within a category

having similar operating characteristics. Examples include

single engine, multiengine, land, water, gyroplane,

helicopter, airship, and free balloon.

With respect to the certification of aircraft, which is a category of

aircraft?

ANSWER: Normal, utility, acrobatic.

Category of aircraft, as used with

respect to the certification of aircraft, means a grouping of

aircraft based upon intended use or operating limitations.

Examples include transport, normal, utility, acrobatic, limited,

restricted, and provisional.

With respect to the certification of aircraft, which is a class of

aircraft?

ANSWER: Airplane, rotorcraft, glider, balloon.

Class of aircraft, as used with respect

to the certification of aircraft, means a broad grouping of

aircraft having similar characteristics of propulsion, flight, or

landing. Examples include airplane, rotorcraft, glider, balloon,

landplane, and seaplane.

The definition of nighttime is

ANSWER: the time between the end of evening civil twilight and the

beginning of morning civil twilight.

"Night" means the time between the

end of evening civil twilight and the beginning of morning

civil twilight, as published in the American Air Almanac,

converted to local time.

Which V-speed represents maximum flap extended speed?

ANSWER: VFE.

VFE means the maximum flap

extended speed.

Which V-speed represents maximum landing gear extended speed?

ANSWER: VLE.

VLE means the maximum landing gear

extended speed.

VNO is defined as the

ANSWER: maximum structural cruising speed.

VNO is defined as the maximum

structural cruising speed.

Which V-speed represents maneuvering speed?

ANSWER: VA.

VA means design maneuvering speed.

VS0 is defined as the

ANSWER: stalling speed or minimum steady flight speed in the

landing configuration.

VS0 is defined as the stalling speed or

minimum steady flight speed in the landing configuration.

Which would provide the greatest gain in altitude in the shortest

distance during climb after takeoff?

ANSWER: VX.

VX means the best angle of climb

airspeed (i.e., the airspeed which will provide the greatest

gain in altitude in the shortest distance).

After takeoff, which airspeed would the pilot use to gain the most

altitude in a given period of time?

ANSWER: VY.

VY means the airspeed for the best

rate of climb (i.e., the airspeed that you use to gain the most

altitude in a given period of time).

What should an owner or operator know about Airworthiness

Directives (AD's)?

ANSWER: They are mandatory.

Airworthiness Directives (ADs) are

issued under FAR Part 39 by the FAA to require correction

of unsafe conditions found in an airplane, an airplane

engine, a propeller, or an appliance when such conditions

exist and are likely to exist or develop in other products of

the same design. Since ADs are issued under FAR Part 39,

they are regulatory and must be complied with, unless a

specific exemption is granted.

May a pilot operate an aircraft that is not in compliance with an

Airworthiness Directive (AD)?

ANSWER: Yes, if allowed by the AD.

An AD is used to notify aircraft

owners and other interested persons of unsafe conditions

and prescribe the conditions under which the product (e.g.,

an aircraft) may continue to be operated. An AD may be one

of an emergency nature requiring immediate compliance

upon receipt or one of a less urgent nature requiring

compliance within a relatively longer period of time. You may

operate an airplane that is not in compliance with an AD, if

such operation is allowed by the AD.

What regulation allows a private pilot to perform preventive

maintenance?

ANSWER: 14 CFR Part 43.7.

Preventive maintenance means simple

or minor preservation operations and the replacement of

small standard parts not involving complex assembly

operations. Appendix A to Part 43 provides a list of work

that is considered preventive maintenance. Part 43 allows a

person who holds a pilot certificate to perform preventive

maintenance on any aircraft owned or operated by that pilot

which is not used in air carrier service.

Who may perform preventive maintenance on an aircraft and

approve it for return to service?

ANSWER: Private or Commercial pilot.

A person who holds a pilot certificate

issued under Part 61 may perform preventive maintenance

on any airplane owned or operated by that pilot which is not

used in air carrier service. To approve the airplane for return

to service after preventive maintenance is performed by a

pilot, the pilot must hold at least a private pilot certificate.

Preventive maintenance has been performed on an aircraft. What

paperwork is required?

ANSWER: The signature, certificate number, and kind of certificate

held by the person approving the work and a description of

the work must be entered in the aircraft maintenance records.

After preventive maintenance has

been performed, the signature, certificate number, and kind

of certificate held by the person approving the work and a

description of the work must be entered in the aircraft

maintenance records.

Which operation would be described as preventive maintenance?

ANSWER: Servicing landing gear wheel bearings.

Appendix A to Part 43 provides a list

of work that is considered preventive maintenance.

Preventive maintenance means simple or minor preservation

operations and the replacement of small standard parts not

involving complex assembly operations. Servicing landing

gear wheel bearings, such as cleaning and greasing, is

considered preventive maintenance.

Which operation would be described as preventive maintenance?

ANSWER: Replenishing hydraulic fluid.

Appendix A to Part 43 provides a list

of work that is considered preventive maintenance.

Preventive main tenance means simple or minor preservation

operations and the replacement of small standard parts not

involving complex assembly operations. An example of

preventive maintenance is replenishing hydraulic fluid.

When must a current pilot certificate be in the pilot's personal

possession or readily accessible in the aircraft?

ANSWER: Anytime when acting as pilot in command or as a required

crewmember.

Current and appropriate pilot and

medical certificates must be in your personal possession or

readily accessible in the aircraft when you act as pilot in

command (PIC) or as a required pilot flight crewmember.

A recreational or private pilot acting as pilot in command, or in any

other capacity as a required pilot flight crewmember, must have in

his or her personal possession or readily accessible in the aircraft a

current

ANSWER: medical certificate if required and an appropriate pilot

certificate.

Current and appropriate pilot and

medical certificates must be in your personal possession or

readily accessible in the aircraft when you act as pilot in

command (PIC) or as a required pilot flight crewmember.

What document(s) must be in your personal possession or readily

accessible in the aircraft while operating as pilot in command of an

aircraft?

ANSWER: An appropriate pilot certificate and an appropriate current

medical certificate if required.

Current and appropriate pilot and

medical certificates must be in your personal possession or

readily accessible in the aircraft when you act as pilot in

command (PIC) or as a required pilot flight crewmember.

Each person who holds a pilot certificate or a medical certificate

shall present it for inspection upon the request of the

Administrator, the National Transportation Safety Board, or any

ANSWER: federal, state, or local law enforcement officer.

Each person who holds a pilot

certificate, flight instructor certificate, medical certificate,

authorization, or license required by the FARs shall present

it for inspection upon the request of the Administrator (of

the FAA), an authorized representative of the National

Transportation Safety Board, or any Federal, State, or local law enforcement officer.

A Third-Class Medical Certificate is issued to a 36-year-old pilot on

August 10, this year. To exercise the privileges of a Private Pilot

Certificate, the medical certificate will be valid until midnight on

ANSWER: August 31, 3 years later.

A pilot may exercise the privileges of

a private pilot certificate under a third-class medical

certificate until it expires at the end of the last day of the

month 3 years after it was issued, for pilots less than 40

years old on the date of the medical examination. A

third-class medical certificate issued to a 36-year-old pilot on

Aug. 10 will be valid until midnight on Aug. 31, 3 years later.

A Third-Class Medical Certificate is issued to a 51-year-old pilot on

May 3, this year. To exercise the privileges of a Private Pilot

Certificate, the medical certificate will be valid until midnight on

ANSWER: May 31, 2 years later.

A pilot may exercise the privileges of

a private pilot certificate under a third-class medical

certificate until it expires at the end of the last day of the

month 2 years after it was issued, for pilots 40 years old or

older on the date of the medical examination. A third-class

medical certificate issued to a 51-year-old pilot on May 3 will

be valid until midnight on May 31, 2 years later.

For private pilot operations, a Second-Class Medical Certificate

issued to a 42-year-old pilot on July 15, this year, will expire at

midnight on

ANSWER: July 31, 2 years later.

For private pilot operations, a

second-class medical certificate will expire at the end of the

last day of the month, 2 years after it was issued, for pilots

40 years old or older on the date of the medical examination.

For private pilot operations, a second-class medical

certificate issued to a 42-year-old pilot on July 15 will be

valid until midnight on July 31, 2 years later.

For private pilot operations, a First-Class Medical Certificate issued

to a 23-year-old pilot on October 21, this year, will expire at

midnight on

ANSWER: October 31, 3 years later.

For private pilot operations, a

first-class medical certificate will expire at the end of the last

day of the month, 3 years after it was issued, for pilots less

than 40 years old on the date of the medical examination. For

private pilot operations, a first-class medical certificate

issued to a 23-year-old pilot on Oct. 21 will be valid until

midnight on Oct. 31, 3 years later.

A Third-Class Medical Certificate was issued to a 19-year-old pilot

on August 10, this year. To exercise the privileges of a recreational

or private pilot certificate, the medical certificate will expire at

midnight on

ANSWER: August 31, 3 years later.

A pilot may exercise the privileges of

a recreational or private pilot certificate under a third-class

medical certificate until it expires at the end of the last day of

the month 3 years after it was issued, for pilots less than 40

years old at the time of the medical examination. A

third-class medical certificate issued to a 19-year-old pilot on

Aug. 10 will expire at midnight on Aug. 31, 3 years later.

Before a person holding a private pilot certificate may act as pilot in

command of a high-performance airplane, that person must have

ANSWER: received ground and flight instruction from an authorized

flight instructor who then endorses that person's logbook.

A private pilot may not act as pilot in

command of a high-performance airplane (an airplane with an

engine of more than 200 horsepower) unless (s)he has

received and logged ground and flight training from an

authorized instructor who has certified in his/her logbook

that (s)he is proficient to operate a high-performance

airplane.

What is the definition of a high-performance airplane?

ANSWER: An airplane with an engine of more than 200 horsepower.

A high-performance airplane is

defined as an airplane with an engine of more than 200

horsepower.

The pilot in command is required to hold a type rating in which

aircraft?

ANSWER: Aircraft having a gross weight of more than 12,500

pounds.

A person may not act as pilot in

command of any of the following aircraft unless he holds a

type rating for that aircraft:

(1) A large aircraft (except lighter-than-air), i.e., over 12,500

lb. gross weight

(2) A turbojet-powered airplane

(3) Other aircraft specified by the FAA through aircraft type

certificate procedures

In order to act as pilot in command of a high-performance airplane,

a pilot must have

ANSWER: received and logged ground and flight instruction in an

airplane that has more than 200 horsepower.

Prior to acting as pilot in command of

an airplane with an engine of more than 200 horsepower, a

person is required to receive and log ground and flight

training in such an airplane from an authorized flight

instructor who has certified in the pilot's logbook that the

individual is proficient to operate a high-performance

airplane.

To act as pilot in command of an aircraft carrying passengers, a

pilot must show by logbook endorsement the satisfactory

completion of a flight review or completion of a pilot proficiency

check within the preceding

ANSWER: 24 calendar months.

To act as pilot in command of an

aircraft (whether carrying passengers or not), a pilot must

show by logbook endorsement the satisfactory completion

of a flight review or completion of a pilot proficiency check

within the preceding 24 calendar months.

If a recreational or private pilot had a flight review on August 8,

this year, when is the next flight review required?

ANSWER: August 31, 2 years later.

A pilot is required to have a flight

review within the preceding 24 calendar months before the

month in which the pilot acts as pilot in command. Thus, a

pilot who had a flight review on Aug. 8 of this year, must

have a flight review completed by Aug. 31, 2 years later.

Each recreational or private pilot is required to have

ANSWER: a biennial flight review.

Each recreational or private pilot is

required to have a biennial (every 2 years) flight review.

If a recreational or private pilot had a flight review on August 8,

this year, when is the next flight review required?

ANSWER: August 31, 2 years later.

A pilot is required to have a flight

review within the preceding 24 calendar months before the

month in which the pilot acts as pilot in command. Thus, a

recreational or private pilot who had a flight review on Aug.

8 of this year, must have a flight review completed by Aug.

31, 2 years later.

To act as pilot in command of an aircraft carrying passengers, the

pilot must have made at least three takeoffs and three landings in

an aircraft of the same category, class, and if a type rating is

required, of the same type, within the preceding

ANSWER: 90 days.

To act as pilot in command of an

airplane with passengers aboard, you must have made at

least three takeoffs and three landings (to a full stop if in a

tailwheel airplane) in an airplane of the same category, class,

and, if a type rating is required, of the same type within the

preceding 90 days. Category refers to airplane, rotorcraft,

etc.; class refers to single or multiengine, land or sea.

If recency of experience requirements for night flight are not met

and official sunset is 1830, the latest time passengers may be

carried is

ANSWER: 1929.

For the purpose of night recency

experience flight time, night is defined as the period

beginning 1 hr. after sunset and ending 1 hr. before sunrise.

If you have not met the night experience requirements, and

official sunset is 1830, a landing must be accomplished at or

before 1929 if passengers are carried.

To act as pilot in command of an aircraft carrying passengers, the

pilot must have made three takeoffs and three landings within the

preceding 90 days in an aircraft of the same

ANSWER: category, class, and type, if a type rating is required.

No one may act as pilot in command

of an airplane carrying passengers unless within the

preceding 90 days (s)he has made three takeoffs and three

landings as sole manipulator of the controls in an aircraft of

the same category and class and, if a type rating is required,

the same type. If the aircraft is a tailwheel airplane, the

landings must have been to a full stop.

The three takeoffs and landings that are required to act as pilot in

command at night must be done during the time period from

ANSWER: 1 hour after sunset to 1 hour before sunrise.

No one may act as pilot in command

of an aircraft carrying passengers at night (i.e., the period

from 1 hr. after sunset to 1 hr. before sunrise as published in

the American Air Almanac) unless (s)he has made three

takeoffs and three landings to a full stop within the

preceding 90 days, at night, in the category and class of

aircraft to be used.

To meet the recency of experience requirements to act as pilot in

command carrying passengers at night, a pilot must have made at

least three takeoffs and three landings to a full stop within the

preceding 90 days in

ANSWER: the same category and class of aircraft to be used.

No one may act as pilot in command

of an aircraft carrying passengers at night (i.e., the period

from 1 hr. after sunset to 1 hr. before sunrise) unless (s)he

has made three takeoffs and three landings to a full stop

within the preceding 90 days, at night, in the category and

class of aircraft to be used.

The takeoffs and landings required to meet the recency of

experience requirements for carrying passengers in a tailwheel

airplane

ANSWER: must be to a full stop.

To comply with recency requirements

for carrying passengers in a tailwheel airplane, one must

have made three takeoffs and landings to a full stop within

the past 90 days.

If a certificated pilot changes permanent mailing address and fails

to notify the FAA Airmen Certification Branch of the new address,

the pilot is entitled to exercise the privileges of the pilot certificate

for a period of only

ANSWER: 30 days after the date of the move.

If you have changed your permanent

mailing address, you may not exercise the privileges of your

pilot certificate after 30 days from the date of the address

change unless you have notified the FAA in writing of the

change. You are required to notify the Airman Certification

Branch at Box 25082, Oklahoma City, OK, 73125.

Note: While you must notify the FAA if your address

changes, you are not required to carry a certificate that

shows your current address. The FAA will not issue a new

certificate upon receipt of your new address unless you

send a written request and $2 to the address shown above.

When may a recreational pilot act as pilot in command on a

cross-country flight that exceeds 50 nautical miles from the

departure airport?

ANSWER: After receiving ground and flight instructions on

cross-country training and a logbook endorsement.

A recreational pilot may act as pilot in

command on a cross-country flight that exceeds 50 NM from

the departure airport, provided that person has received

ground and flight training from an authorized instructor on

the cross-country training requirements for a private pilot

certificate and has received a logbook endorsement, which is

in the person's possession in the aircraft, certifying the

person is proficient in cross-country flying.

A certificated private pilot may not act as pilot in command of an

aircraft towing a glider unless there is entered in the pilot's logbook

a minimum of

ANSWER: 100 hours of pilot-in-command time in the aircraft

category, class, and type, if required, that the pilot is using

to tow a glider.

As a private pilot, you may not act as

pilot in command of an aircraft towing a glider unless you

have had, and entered in your logbook, at least 100 hr. of

pilot-in-command time in the aircraft category, class, and

type, if required, that you are using to tow a glider.

To act as pilot in command of an aircraft towing a glider, a pilot is

required to have made within the preceding 12 months

ANSWER: at least three actual or simulated glider tows while

accompanied by a qualified pilot.

To act as pilot in command of an

aircraft towing a glider, you are required to have made, in the

preceding 12 months,

(1) At least three actual or simulated glider tows while

accompanied by a qualified pilot, or

(2) At least three flights as pilot in command of a glider

towed by an aircraft.

In regard to privileges and limitations, a private pilot may

ANSWER: not pay less than the pro rata share of the operating

expenses of a flight with passengers provided the expenses

involve only fuel, oil, airport expenditures, or rental fees.

A private pilot may not pay less than

an equal (pro rata) share of the operating expenses of a flight

with passengers. These expenses may involve only fuel, oil,

airport expenditures (e.g., landing fees, tie-down fees, etc.),

or rental fees.

According to regulations pertaining to privileges and limitations, a

private pilot may

ANSWER: not pay less than the pro rata share of the operating

expenses of a flight with passengers provided the expenses

involve only fuel, oil, airport expenditures, or rental fees.

A private pilot may not pay less than

an equal (pro rata) share of the operating expenses of a flight

with passengers. These expenses may involve only fuel, oil,

airport expenditures (e.g., landing fees, tie-down fees, etc.),

or rental fees.

What exception, if any, permits a private pilot to act as pilot in

command of an aircraft carrying passengers who pay for the flight?

ANSWER: If a donation is made to a charitable organization for the

flight.

A private pilot may act as pilot in

command of an airplane used in a passenger-carrying airlift

sponsored by a charitable organization for which

passengers make donations to the organization, provided

the following requirements are met: the local FSDO is

notified at least 7 days before the flight, the flight is

conducted from an adequate public airport, the pilot has

logged at least 200 hr., no acrobatic or formation flights are

performed, the 100-hr. inspection of the airplane requirement

is complied with, and the flight is day-VFR.

A recreational pilot acting as pilot in command must have in his or

her personal possession while aboard the aircraft

ANSWER: a current logbook endorsement that permits flight within

50 nautical miles from the departure airport.

A recreational pilot acting as pilot in

command must have in his/her personal possession while

aboard the airplane a current logbook endorsement that

permits flight within 50 NM from the departure airport.

How many passengers is a recreational pilot allowed to carry on

board?

ANSWER: One.

Recreational pilots may carry not

more than one passenger.

According to regulations pertaining to privileges and limitations, a recreational pilot may

ANSWER: not pay less than the pro rata share of the operating

expenses of a flight with a passenger.

A recreational pilot may not pay less

than an equal (pro rata) share of the operating expenses of

the flight with a passenger. These expenses may involve

only fuel, oil, airport expenditures (e.g., landing fees,

tie-down fees, etc.), or rental fees.

In regard to privileges and limitations, a recreational pilot may

ANSWER: not pay less than the pro rata share of the operating

expenses of a flight with a passenger.

A recreational pilot may not pay less

than an equal (pro rata) share of the operating expenses of

the flight with a passenger. These expenses may involve

only fuel, oil, airport expenditures (e.g., landing fees,

tie-down fees, etc.), or rental fees.

A recreational pilot may act as pilot in command of an aircraft that

is certificated for a maximum of how many occupants?

ANSWER: Four.

Recreational pilots may not act as

pilot in command of an aircraft that is certificated for more

than four occupants. Note, however, that only two

occupants are permitted, the recreational pilot and a

passenger.

A recreational pilot may act as pilot in command of an aircraft with

a maximum engine horsepower of

ANSWER: 180.

A recreational pilot may act as pilot in

command of an aircraft with a maximum engine horsepower

of 180.

With respect to daylight hours, what is the earliest time a

recreational pilot may take off?

ANSWER: At sunrise.

A recreational pilot may not act as

pilot in command of an airplane between sunset and sunrise.

Thus, the earliest time a recreational pilot may take off is at

sunrise.

What exception, if any, permits a recreational pilot to act as pilot in

command of an aircraft carrying a passenger for hire?

ANSWER: There is no exception.

Recreational pilots may not act as

pilot in command of an aircraft for compensation or hire.

There is no exception.

May a recreational pilot act as pilot in command of an aircraft in

furtherance of a business?

ANSWER: No, it is not allowed.

Recreational pilots may not act as

pilot in command of an aircraft that is used in furtherance of

a business. There is no exception.

When may a recreational pilot operate to or from an airport that lies

within Class C airspace?

ANSWER: For the purpose of obtaining an additional certificate or

rating while under the supervision of an authorized flight

instructor.

For the purpose of obtaining an

additional certificate or rating while under the supervision of

an authorized flight instructor, a recreational pilot may fly as

sole occupant of an airplane within airspace that requires

communication with ATC, such as Class C airspace. [Note

that in this situation, (s)he is active as a student pilot, not a

recreational pilot.]

If sunset is 2021 and the end of evening civil twilight is 2043, when

must a recreational pilot terminate the flight?

ANSWER: 2021.

A recreational pilot may not act as

pilot in command of an airplane between sunset and sunrise.

Thus, if sunset is 2021, the recreational pilot must terminate

the flight at 2021.

Under what conditions may a recreational pilot operate at an airport

that lies within Class D airspace and that has a part-time control

tower in operation?

ANSWER: Between sunrise and sunset when the tower is closed, the

ceiling is at least 1,000 feet, and the visibility is at least 3

miles.

A recreational pilot may not operate in

airspace in which communication with ATC is required, e.g.,

Class D airspace. When a part-time control tower at an

airport in Class D airspace is closed, the Class D airspace is

classified as either Class E or Class G airspace, which does

not require communication with ATC. A recreational pilot

must maintain flight or surface visibility of 3 SM or greater,

and the flight must be during the day. To operate at an

airport in Class E airspace, the ceiling must be at least 1,000

ft. and the visibility at least 3 SM (FAR 91.155).

When may a recreational pilot fly above 10,000 feet MSL?

ANSWER: When 2,000 feet AGL or below.

Recreational pilots may not act as

pilot in command of an aircraft at an altitude of more than

10,000 ft. MSL or 2,000 ft. AGL, whichever is higher. Thus,

an airplane may fly above 10,000 ft. MSL only if below 2,000

ft. AGL.

During daytime, what is the minimum flight or surface visibility

required for recreational pilots in Class G airspace below 10,000 feet

MSL?

ANSWER: 3 miles.

The minimum flight or surface

visibility required for recreational pilots in Class G airspace

below 10,000 ft. MSL during the day is 3 SM.

During daytime, what is the minimum flight visibility required for

recreational pilots in controlled airspace below 10,000 feet MSL?

ANSWER: 3 miles.

The minimum flight visibility for

recreational pilots in Class E airspace below 10,000 ft. MSL

during the day is 3 SM.

Under what conditions, if any, may a recreational pilot demonstrate

an aircraft in flight to a prospective buyer?

ANSWER: None.

Recreational pilots may not act as

pilot in command of an aircraft to demonstrate that aircraft in

flight to a prospective buyer.

When must a recreational pilot have a pilot-in-command flight

check?

ANSWER: If the pilot has less than 400 total flight hours and has not

flown as pilot in command in an aircraft within the preceding

180 days.

The recreational pilot who has logged

fewer than 400 flight hr. and has not logged pilot in

command time in an aircraft within the preceding 180 days

may not act as pilot in command of an aircraft until the pilot

has received flight instruction from an authorized flight

instructor who certifies in the pilot's logbook that the pilot is

competent to act as pilot in command of the aircraft.

When, if ever, may a recreational pilot act as pilot in command in an

aircraft towing a banner?

ANSWER: It is not allowed.

Recreational pilots may not act as

pilot in command of an aircraft that is towing any object.

A recreational pilot may fly as sole occupant of an aircraft at night

while under the supervision of a flight instructor provided the

flight or surface visibility is at least

ANSWER: 5 miles.

For the purposes of obtaining

additional certificates or ratings, a recreational pilot may fly

as sole occupant in the aircraft between sunset and sunrise

while under the supervision of an authorized flight

instructor, providing the flight or surface visibility is at least

5 SM.

The width of a Federal Airway from either side of the centerline is

ANSWER: 4 nautical miles.

The width of a Federal Airway from

either side of the centerline is 4 NM.

Unless otherwise specified, Federal Airways include that Class E

airspace extending upward from

ANSWER: 1,200 feet above the surface up to and including 17,999

feet MSL.

Unless otherwise specified, Federal

Airways include that Class E airspace extending from 1,200

ft. above the surface up to and including 17,999 ft.

Normal VFR operations in Class D airspace with an operating

control tower require the visibility and ceiling to be at least

ANSWER: 1,000 feet and 3 miles.

The basic VFR weather minimums for

operating an aircraft within Class D airspace are a 1,000-ft.

ceiling and 3 SM visibility.

The final authority as to the operation of an aircraft is the

ANSWER: pilot in command.

The final authority as to the operation

of an aircraft is the pilot in command.

Who is responsible for determining if an aircraft is in condition for

safe flight?

ANSWER: The pilot in command.

The pilot in command of an aircraft is

directly responsible for, and is the final authority for,

determining whether the airplane is in condition for safe

flight.

Where may an aircraft's operating limitations be found?

ANSWER: In the current, FAA-approved flight manual, approved

manual material, markings, and placards, or any combination

thereof.

An aircraft's operating limitations may

be found in the current, FAA-approved flight manual,

approved manual material, markings, and placards, or any

combination thereof.

Under what conditions may objects be dropped from an aircraft?

ANSWER: If precautions are taken to avoid injury or damage to

persons or property on the surface.

No pilot in command of a civil aircraft

may allow any object to be dropped from that aircraft in

flight that creates a hazard to persons or property. However,

this section does not prohibit the dropping of any object if

reasonable precautions are taken to avoid injury or damage

to persons or property.

No person may attempt to act as a crewmember of a civil aircraft

with

ANSWER: 04 percent by weight or more alcohol in the blood.

No person may act or attempt to act

as a crewmember of a civil aircraft, while having a .04% by

weight or more alcohol in the blood.

A person may not act as a crewmember of a civil aircraft if alcoholic

beverages have been consumed by that person within the

preceding

ANSWER: 8 hours.

No person may act as a crewmember

of a civil aircraft if alcoholic beverages have been consumed

by that person within the preceding 8 hr.

Under what condition, if any, may a pilot allow a person who is

obviously under the influence of drugs to be carried aboard an

aircraft?

ANSWER: In an emergency or if the person is a medical patient

under proper care.

No pilot of a civil aircraft may allow a

person who demonstrates by manner or physical indications

that the individual is under the influence of drugs to be

carried in that aircraft, except in an emergency or if the

person is a medical patient under proper care.

Preflight action, as required for all flights away from the vicinity of

an airport, shall include

ANSWER: an alternate course of action if the flight cannot be

completed as planned.

Preflight actions for flights not in the

vicinity of an airport include checking weather reports and

forecasts, fuel requirements, alternatives available if the

planned flight cannot be completed, and any known traffic  delays.

In addition to other preflight actions for a VFR flight away from the

vicinity of the departure airport, regulations specifically require the

pilot in command to

ANSWER: determine runway lengths at airports of intended use and

the aircraft's takeoff and landing distance data.

Preflight actions for a VFR flight away

from the vicinity of the departure airport specifically require

the pilot in command to determine runway lengths at airports

of intended use and the aircraft's takeoff and landing

distance data.

Which preflight action is specifically required of the pilot prior to

each flight?

ANSWER: Become familiar with all available information concerning

the flight.

Each pilot in command will, before

beginning a flight, become familiar with all available

information concerning that flight.

Flight crewmembers are required to keep their safety belts and

shoulder harnesses fastened during

ANSWER: takeoffs and landings.

During takeoff and landing, and while

en route, each required flight crewmember shall keep his/her

safety belt fastened while at the crewmember station. If

shoulder harnesses are available, they must be used by crew

members during takeoff and landing.

Which best describes the flight conditions under which flight

crewmembers are specifically required to keep their safety belts and

shoulder harnesses fastened?

ANSWER: Safety belts during takeoff and landing and while en

route; shoulder harnesses during takeoff and landing.

During takeoff and landing, and while

en route, each required flight crewmember shall keep his/her

safety belt fastened while at the crewmember station. If

shoulder harnesses are available, they must be used by

crewmembers during takeoff and landing.

Safety belts are required to be properly secured about which

persons in an aircraft and when?

ANSWER: Passengers, during taxi, takeoffs, and landings only.

Regulations require that safety belts

in an airplane be properly secured about all passengers

during taxi, takeoffs, and landings.

With respect to passengers, what obligation, if any, does a pilot in

command have concerning the use of safety belts?

ANSWER: The pilot in command must brief the passengers on the

use of safety belts and notify them to fasten their safety

belts during taxi, takeoff, and landing.

The pilot in command is required to

brief the passengers on the use of safety belts and notify

them to fasten their safety belts during taxi, takeoff, and

landing.

With certain exceptions, safety belts are required to be secured

about passengers during

ANSWER: taxi, takeoffs, and landings.

During the taxi, takeoff, and landing

of U.S. registered civil aircraft, each person on board that

aircraft must occupy a seat or berth with a safety belt and

shoulder harness, if installed, properly secured about

him/her. However, a person who has not reached his/her

second birthday may be held by an adult who is occupying

a seat or berth, and a person on board for the purpose of

engaging in sport parachuting may use the floor of the

aircraft as a seat (but is still required to use approved safety

belts for takeoff).

When must a pilot who deviates from a regulation during an

emergency send a written report of that deviation to the

Administrator?

ANSWER: Upon request.

A pilot who deviates from a regulation

during an emergency must send a written report of that

deviation to the Administrator of the FAA only upon

request.

If an in-flight emergency requires immediate action, the pilot in

command may

ANSWER: deviate from the FAR's to the extent required to meet that

emergency.

In an in-flight emergency requiring

immediate action, the pilot in command may deviate from the

FARs to the extent required to meet that emergency. A

written report of the deviation must be sent to the

Administrator of the FAA only if requested.

Which is the correct traffic pattern departure procedure to use at a

noncontrolled airport?

ANSWER: Comply with any FAA traffic pattern established for the

airport.

Each person operating an airplane to

or from an airport without an operating control tower shall

(1) in the case of an airplane approaching to land, make all

turns of that airplane to the left unless the airport displays

approved light signals or visual markings indicating that

turns should be made to the right, in which case the pilot

shall make all turns to the right, and (2) in the case of an

airplane departing the airport, comply with any FAA traffic

pattern for that airport.

When approaching to land on a runway served by a visual

approach slope indicator (VASI), the pilot shall

ANSWER: maintain an altitude at or above the glide slope.

An airplane approaching to land on a

runway served by a VASI shall maintain an altitude at or

above the glide slope until a lower altitude is necessary for a

safe landing.

Each pilot of an aircraft approaching to land on a runway served by

a visual approach slope indicator (VASI) shall

ANSWER: maintain an altitude at or above the glide slope.

When approaching to land on a

runway served by a VASI, each pilot of an airplane must fly

at or above the VASI glide path until a lower altitude is

necessary for a safe landing.

A blue segmented circle on a Sectional Chart depicts which class

airspace?

ANSWER: Class D.

A blue segmented circle on a

sectional chart depicts Class D airspace.

Airspace at an airport with a part-time control tower is classified as

Class D airspace only

ANSWER: when the associated control tower is in operation.

A Class D airspace area is

automatically in effect when and only when the associated

part-time control tower is in operation regardless of weather

conditions, availability of radar services, or time of day.

Airports with part-time operating towers only have a

part-time Class D airspace area.

Unless otherwise authorized, two-way radio communications with

Air Traffic Control are required for landings or takeoffs.

ANSWER: at all tower controlled airports regardless of weather

conditions.

Two-way radio communications with

air traffic control (ATC) are required for landing and taking

off at all tower controlled airports, regardless of weather

conditions. However, light signals from the tower may be

used during radio failure.

While on final approach for landing, an alternating green and red

light followed by a flashing red light is received from the control

tower. Under these circumstances, the pilot should

ANSWER: exercise extreme caution and abandon the approach,

realizing the airport is unsafe for landing.

An alternating red and green light

signaled from a control tower means "exercise extreme

caution" whether to an airplane on the ground or in the air.

The flashing red light received while in the air indicates the

airport is not safe and the pilot should not land.

A steady green light signal directed from the control tower to an

aircraft in flight is a signal that the pilot

ANSWER: is cleared to land.

A steady green light signal from the

tower to an airplane in flight means cleared to land.

A flashing white light signal from the control tower to a taxiing

aircraft is an indication to

ANSWER: return to the starting point on the airport.

A flashing white light given to an

aircraft taxiing along the ground means to return to the

aircraft's starting point.

If the control tower uses a light signal to direct a pilot to give way

to other aircraft and continue circling, the light will be

ANSWER: steady red.

A steady red light signal given to an

aircraft in the air means to give way to other aircraft and

continue circling.

Which light signal from the control tower clears a pilot to taxi?

ANSWER: Flashing green.

A flashing green gives the pilot

permission to taxi.

An alternating red and green light signal directed from the control

tower to an aircraft in flight is a signal to

ANSWER: exercise extreme caution.

A flashing red and green light given

anytime means exercise extreme caution.

No person may operate an aircraft in formation flight

ANSWER: except by prior arrangement with the pilot in command of

each aircraft.

No person may operate in formation

flight except by arrangement with the pilot in command of

each aircraft in formation.

An airplane and an airship are converging. If the airship is left of

the airplane's position, which aircraft has the right-of-way?

ANSWER: The airship.

When aircraft of different categories

are converging, the less maneuverable aircraft has the

right-of-way. Thus, the airship has the right-of-way in this

question.

When two or more aircraft are approaching an airport for the

purpose of landing, the right-of-way belongs to the aircraft

ANSWER: at the lower altitude, but it shall not take advantage of this

rule to cut in front of or to overtake another.

When two or more aircraft are

approaching an airport for the purpose of landing, the

aircraft at the lower altitude has the right-of-way, but it shall

not take advantage of this rule to cut in front of or to

overtake another aircraft.

Which aircraft has the right-of-way over the other aircraft listed?

ANSWER: Glider.

If aircraft of different categories are

converging, the right-of-way depends upon who has the

least maneuverability. A glider has right-of-way over an

airship, airplane or rotorcraft.

What action should the pilots of a glider and an airplane take if on

a head-on collision course?

ANSWER: Both pilots should give way to the right.

When aircraft are approaching

head-on, or nearly so (regardless of category), each aircraft

shall alter course to the right.

What action is required when two aircraft of the same category

converge, but not head-on?

ANSWER: The aircraft on the left shall give way.

When two aircraft of the same

category converge (but not head-on), the aircraft to the

other's right has the right-of-way. Thus, an airplane on the

left gives way to the airplane on the right.

Which aircraft has the right-of-way over the other aircraft listed?

ANSWER: Aircraft towing other aircraft.

An aircraft towing or refueling

another aircraft has the right-of-way over all engine-driven

aircraft. An airship is an engine-driven, lighter-than-air

aircraft that can be steered.

Which aircraft has the right-of-way over all other air traffic?

ANSWER: An aircraft in distress.

An aircraft in distress has the

right-of-way over all other aircraft.

A seaplane and a motorboat are on crossing courses. If the

motorboat is to the left of the seaplane, which has the

right-of-way?

ANSWER: The seaplane.

When aircraft, or an aircraft and a

vessel (e.g., a motorboat), are on crossing courses, the

aircraft or vessel to the other's right has the right-of-way.

Since the seaplane is to the motorboat's right, the seaplane

has the right-of-way.

When flying in a VFR corridor designated through Class B

airspace, the maximum speed authorized is

ANSWER: 200 knots.

No person may operate an airplane in

a VFR corridor designated through Class B airspace at an

indicated airspeed of more than 200 kt. (230 MPH).

Unless otherwise authorized, what is the maximum indicated

airspeed at which a person may operate an aircraft below 10,000

feet MSL?

ANSWER: 250 knots.

Unless otherwise authorized by ATC,

no person may operate an aircraft below 10,000 ft. MSL at an

indicated airspeed of more than 250 kt. (288 MPH).

When flying in the airspace underlying Class B airspace, the

maximum speed authorized is

ANSWER: 200 knots.

No person may operate an airplane in

the airspace underlying Class B airspace at an indicated

airspeed of more than 200 kt. (230 MPH).

Unless otherwise authorized, the maximum indicated airspeed at

which aircraft may be flown when at or below 2,500 feet AGL and

within 4 nautical miles of the primary airport of Class C airspace is

ANSWER: 200 knots.

Unless otherwise authorized, the

maximum indicated airspeed at which an airplane may be

flown when at or below 2,500 ft. AGL and within 4 NM of the

primary airport of Class C airspace is 200 kt. (230 mph).

Except when necessary for takeoff or landing, an aircraft may not

be operated closer than what distance from any person, vessel,

vehicle, or structure?

ANSWER: 500 feet.

Over other than congested areas, an

altitude of 500 ft. above the surface is required. Over open

water and sparsely populated areas, a distance of 500 ft.

from any person, vessel, vehicle, or structure must be

maintained.

Except when necessary for takeoff or landing, what is the minimum

safe altitude for a pilot to operate an aircraft anywhere?

ANSWER: An altitude allowing, if a power unit fails, an emergency

landing without undue hazard to persons or property on the

surface.

Except when necessary for takeoff or

landing, no person may operate an aircraft anywhere below

an altitude allowing, if a power unit fails, an emergency

landing without undue hazard to persons or property on the

surface.

Except when necessary for takeoff or landing, what is the minimum

safe altitude required for a pilot to operate an aircraft over

congested areas?

ANSWER: An altitude of 1,000 feet above the highest obstacle

within a horizontal radius of 2,000 feet of the aircraft.

When operating an aircraft over any

congested area of a city, town, or settlement, or over an

open air assembly of persons, a pilot must remain at an

altitude of 1,000 ft. above the highest obstacle within a

horizontal radius of 2,000 ft. of the aircraft.

Except when necessary for takeoff or landing, what is the minimum

safe altitude required for a pilot to operate an aircraft over other

than a congested area?

ANSWER: An altitude of 500 feet AGL, except over open water or a

sparsely populated area, which requires 500 feet from any

person, vessel, vehicle, or structure.

Over other than congested areas, an

altitude of 500 ft. above the surface is required. Over open

water and sparsely populated areas, a distance of 500 ft.

from any person, vessel, vehicle, or structure must be

maintained.

Prior to takeoff, the altimeter should be set to which altitude or

altimeter setting?

ANSWER: The current local altimeter setting, if available, or the

departure airport elevation.

Prior to takeoff, the altimeter should

be set to the local altimeter setting, or to the departure

airport elevation.

If an altimeter setting is not available before flight, to which altitude

should the pilot adjust the altimeter?

ANSWER: The elevation of the departure area.

When the local altimeter setting is not

available at takeoff, the pilot should adjust the altimeter to

the elevation of the departure area.

At what altitude shall the altimeter be set to 29.92, when climbing to

cruising flight level?

ANSWER: 18,000 feet MSL.

Pressure altitude is the altitude used

for all flights at and above 18,000 ft. MSL, i.e., in Class A

airspace. When climbing to or above 18,000 ft. MSL, one

does not use local altimeter settings, but rather 29.92" Hg

after reaching 18,000 ft. MSL.

When would a pilot be required to submit a detailed report of an

emergency which caused the pilot to deviate from an ATC

clearance?

ANSWER: When requested by ATC.

Each pilot in command who is given

priority by ATC in an emergency shall, if requested by ATC,

submit a detailed report within 48 hrs. to the manager of that

ATC facility.

When an ATC clearance has been obtained, no pilot in command

may deviate from that clearance, unless that pilot obtains an

amended clearance. The one exception to this regulation is

ANSWER: an emergency.

When an ATC clearance has been

obtained, no pilot in command may deviate from that

clearance, except in an emergency, unless an amended

clearance is obtained.

What action, if any, is appropriate if the pilot deviates from an ATC

instruction during an emergency and is given priority?

ANSWER: File a detailed report within 48 hours to the chief of the

appropriate ATC facility, if requested.

Each pilot in command who is given

priority by ATC in an emergency shall, if requested by ATC,

submit a detailed report within 48 hrs. to the manager of that

ATC facility.

Two-way radio communication must be established with the Air

Traffic Control facility having jurisdiction over the area prior to

entering which class airspace?

ANSWER: Class C.

No person may operate an aircraft in

Class C airspace unless two-way radio communication is

established with the ATC facility having jurisdiction over

the airspace prior to entering that area.

What minimum pilot certification is required for operation within

Class B airspace?

ANSWER: Private Pilot Certificate or Student Pilot Certificate with

appropriate logbook endorsements.

No person may take off or land aircraft

at an airport within Class B airspace or operate an aircraft

within Class B airspace unless they are at least a private

pilot or, if a student pilot, they have the appropriate logbook

endorsement required by FAR 61.95.

What minimum pilot certification is required for operation within

Class B airspace?

ANSWER: Private Pilot Certificate or Student Pilot Certificate with

appropriate logbook endorsements.

No person may take off or land

aircraft at an airport within Class B airspace or operate an

aircraft within Class B airspace unless they are at least a

private pilot or, if a student pilot, they have the appropriate

logbook endorsement required by FAR 61.95.

In which type of airspace are VFR flights prohibited?

ANSWER: Class A.

Class A airspace (from 18,000 ft. MSL

up to and including FL 600) require operation under IFR at

specific flight levels assigned by ATC. Accordingly, VFR

flights are prohibited.

An operable 4096-code transponder and Mode C encoding

altimeter are required in

ANSWER: Class B airspace and within 30 miles of the Class B

primary airport.

An operable 4096-code transponder

and Mode C encoding altimeter are required in Class B

airspace and within 30 NM of the Class B primary airport.

What minimum radio equipment is required for operation within

Class C airspace?

ANSWER: Two-way radio communications equipment, a 4096-code

transponder, and an encoding altimeter.

To operate within Class C airspace,

the aircraft must have

1. Two-way radio communications equipment,

2. A 4096-code transponder, and

3. An encoding altimeter.

What minimum radio equipment is required for VFR operation

within Class B airspace?

ANSWER: Two-way radio communications equipment, a 4096-code

transponder, and an encoding altimeter.

To operate within Class B airspace,

the aircraft must have

1. Two-way radio communications equipment,

2. A 4096-code transponder, and

3. An encoding altimeter.

What minimum visibility and clearance from clouds are required for

a recreational pilot in Class G airspace at 1,200 feet AGL or below

during daylight hours?

ANSWER: 3 miles visibility and clear of clouds.

Recreational pilots may not act as

pilot in command of an aircraft when the visibility is less

than 3 SM. Additionally, FAR 91.155 specifies basic VFR

weather minimums which permit pilots to fly in Class G

airspace 1,200 ft. AGL or below at 1 SM clear of clouds.

Thus, the 3-SM recreational pilot limitation and the clear of

clouds situation apply.

Outside controlled airspace, the minimum flight visibility

requirement for a recreational pilot flying VFR above 1,200 feet

AGL and below 10,000 feet MSL during daylight hours is

ANSWER: 3 miles.

Recreational pilots may not act as

pilot in command of an aircraft when the visibility is less

than 3 SM.

What is the specific fuel requirement for flight under VFR at night

in an airplane?

ANSWER: Enough to fly to the first point of intended landing and to

fly after that for 45 minutes at normal cruising speed.

The night VFR requirement is enough

fuel to fly to the first point of intended landing and to fly

thereafter for 45 min. at normal cruising speed given forecast

conditions.

What is the specific fuel requirement for flight under VFR during

daylight hours in an airplane?

ANSWER: Enough to fly to the first point of intended landing and to

fly after that for 30 minutes at normal cruising speed.

The day-VFR requirement is enough

fuel to fly to the first point of intended landing and

thereafter for 30 min. at normal cruising speed.

The minimum flight visibility required for VFR flights above 10,000

feet MSL and more than 1,200 feet AGL in controlled airspace is

ANSWER: 5 miles.

Controlled airspace is the generic term

for Class A, B, C, D, or E airspace. Of these, only in Class E

airspace is the minimum flight visibility 5 SM for VFR flights

at or above 10,000 ft. MSL.

Note: AGL altitudes are not used in controlled airspace. In

Class E airspace, the visibility and distance from clouds are

given for (1) below 10,000 ft. MSL and (2) at or above 10,000

ft. MSL.

VFR flight in controlled airspace above 1,200 feet AGL and below

10,000 feet MSL requires a minimum visibility and vertical cloud

clearance of

ANSWER: 3 miles, and 500 feet below or 1,000 feet above the clouds

in controlled airspace.

Controlled airspace is the generic

term for Class A, B, C, D, or E airspace. Only in Class C, D, or

below 10,000 ft. MSL in Class E airspace are the minimum

flight visibility and vertical distance from cloud for VFR

flight required to be 3 SM, and 500 ft. below or 1,000 ft.

above the clouds.

Note: AGL altitudes are not used in controlled airspace. In

Class E airspace, the visibility and distance from clouds are

given for (1) below 10,000 ft. MSL and (2) at or above 10,000

ft. MSL.

For VFR flight operations above 10,000 feet MSL and more than

1,200 feet AGL, the minimum horizontal distance from clouds

required is

ANSWER: 1 mile.

For VFR flight operations in Class G

airspace at altitudes more than 1,200 ft. AGL and at or above

10,000 ft. MSL, the minimum horizontal distance from clouds

required is 1 SM.

Note: The FAA question fails to specify what type of

airspace. Since AGL altitudes are not used in controlled

airspace (Class A, B, C, D, or E), that implies Class G

airspace.

The basic VFR weather minimums for operating an aircraft within

Class D airspace are

ANSWER: 1,000-foot ceiling and 3 miles visibility.

The basic VFR weather minimums for

operating an aircraft within Class D airspace are 1,000-ft.

ceiling and 3 SM visibility.

The minimum distance from clouds required for VFR operations on

an airway below 10,000 feet MSL is

ANSWER: 500 feet below, 1,000 feet above, and 2,000 feet

horizontally.

An airway includes that Class E

airspace extending upward from 1,200 ft. AGL to, but not

including, 18,000 ft. MSL. The minimum distance from clouds

below 10,000 ft. MSL in Class E airspace is 500 ft. below,

1,000 ft. above, and 2,000 ft. horizontally.

What minimum visibility and clearance from clouds are required for

VFR operations in Class G airspace at 700 feet AGL or below during

daylight hours?

ANSWER: 1 mile visibility and clear of clouds.

Below 1,200 ft. AGL in Class G

airspace during daylight hours, the VFR weather minimum is

1 SM visibility and clear of clouds.

What minimum flight visibility is required for VFR flight operations

on an airway below 10,000 feet MSL?

ANSWER: 3 miles.

An airway includes that Class E

airspace extending upward from 1,200 ft. AGL to, but not

including, 18,000 ft. MSL. The minimum flight visibility for

VFR flight operations in Class E airspace less than 10,000 ft.

MSL is 3 SM.

During operations outside controlled airspace at altitudes of more

than 1,200 feet AGL, but less than 10,000 feet MSL, the minimum

flight visibility for VFR flight at night is

ANSWER: 3 miles.

When operating outside controlled

airspace (i.e., Class G airspace) at night at altitudes of more

than 1,200 ft. AGL, but less than 10,000 ft. MSL, the minimum

flight visibility is 3 SM.

During operations within controlled airspace at altitudes of more

than 1,200 feet AGL, but less than 10,000 feet MSL, the minimum

distance above clouds requirement for VFR flight is

ANSWER: 1,000 feet.

Controlled airspace is the generic term

for Class A, B, C, D, or E airspace. Only in Class C, D, or

below 10,000 ft. MSL in Class E airspace are the minimum

flight visibility and vertical distance from cloud for VFR

flight required to be 3 SM, and 500 ft. below or 1,000 ft.

above the clouds.

Note: AGL altitudes are not used in controlled airspace. In

Class E airspace, the visibility and distance from clouds are

given for (1) below 10,000 ft. MSL and (2) at or above 10,000

ft. MSL.

No person may take off or land an aircraft under basic VFR at an

airport that lies within Class D airspace unless the

ANSWER: ground visibility at that airport is at least 3 miles.

No person may take off or land an

aircraft at any airport that lies within Class D airspace under

basic VFR unless the ground visibility is 3 SM. If ground

visibility is not reported, flight visibility during landing or

takeoff, or while operating in the traffic pattern, must be at

least 3 SM.

During operations at altitudes of more than 1,200 feet AGL and at

or above 10,000 feet MSL, the minimum distance above clouds

requirement for VFR flight is

ANSWER: 1,000 feet.

During operations in Class G airspace

at altitudes of more than 1,200 ft. AGL and at or above 10,000

ft. MSL, the minimum distance above clouds requirement for

VFR flight is 1,000 ft.

Note: The FAA question fails to specify what type of

airspace. Since AGL altitudes are not used in controlled

airspace (Class A, B, C, D, and E), that implies Class G

airspace.

Outside controlled airspace, the minimum flight visibility

requirement for VFR flight above 1,200 feet AGL and below 10,000

feet MSL during daylight hours is

ANSWER: 1 mile.

Outside controlled airspace (i.e.,

Class G airspace) at altitudes above 1,200 ft. AGL and below

10,000 ft. MSL, the minimum flight visibility requirement for

VFR flight during the day is 1 SM.

During operations outside controlled airspace at altitudes of more

than 1,200 feet AGL, but less than 10,000 feet MSL, the minimum

distance below clouds requirement for VFR flight at night is

ANSWER: 500 feet.

Outside controlled airspace (i.e.,

Class G airspace) at altitudes above 1,200 ft. AGL and less

than 10,000 ft. MSL, the minimum distance below clouds

requirement for VFR flight at night is 500 ft.

During operations within controlled airspace at altitudes of less

than 1,200 feet AGL, the minimum horizontal distance from clouds

requirement for VFR flight is

ANSWER: 2,000 feet.

Controlled airspace is the generic term

for Class A, B, C, D, or E airspace. Only in Class C, D, or

below 10,000 ft. MSL in Class E airspace is the minimum

horizontal distance from clouds for VFR flight required to be

2,000 ft.

Note: AGL altitudes are not used in controlled airspace. In

Class E airspace, the visibility and distance from clouds are

given for (1) below 10,000 ft. MSL and (2) at or above 10,000

ft. MSL.

A special VFR clearance authorizes the pilot of an aircraft to

operate VFR while within Class D airspace when the visibility is

ANSWER: at least 1 mile and the aircraft can remain clear of clouds.

To operate within Class D airspace

under special VFR clearance, visibility must be at least 1 SM.

There is no ceiling requirement, but the aircraft must remain

clear of clouds.

No person may operate an airplane within Class D airspace at night

under special VFR unless the

ANSWER: airplane is equipped for instrument flight.

To operate under special VFR within

Class D airspace at night, the pilot must be instrument rated

and the airplane equipped for instrument flight.

What are the minimum requirements for airplane operations under

special VFR in Class D airspace at night?

ANSWER: The pilot must be instrument rated, and the airplane must

be IFR equipped.

To operate under special VFR within

Class D airspace at night, the pilot must be instrument rated

and the airplane must be IFR equipped.

What is the minimum weather condition required for airplanes

operating under special VFR in Class D airspace?

ANSWER: 1 mile flight visibility.

To operate within Class D airspace

under special VFR clearance, visibility must be at least 1 SM.

There is no ceiling requirement, but the aircraft must remain

clear of clouds.

Which VFR cruising altitude is acceptable for a flight on a Victor

Airway with a magnetic course of 175�? The terrain is less than

1,000 feet.

ANSWER: 5,500 feet.

When operating a VFR flight above

3,000 ft. AGL on a magnetic course of 0� through 179�, fly

any odd thousand-ft. MSL altitude plus 500 ft. Thus, on a

magnetic course of 175�, an appropriate VFR cruising

altitude is 5,500 ft.

Which cruising altitude is appropriate for a VFR flight on a

magnetic course of 135�?

ANSWER: Odd thousand plus 500 feet.

When operating a VFR flight above

3,000 ft. AGL on a magnetic course of 0� through 179�, fly

any odd thousand-ft. MSL altitude plus 500 ft. Thus, on a

magnetic course of 135�, an appropriate VFR cruising

altitude is an odd thousand plus 500 ft.

Which VFR cruising altitude is appropriate when flying above

3,000 feet AGL on a magnetic course of 185�?

ANSWER: 4,500 feet.

When operating a VFR flight above

3,000 ft. AGL on a magnetic course of 180� through 359�, fly

any even thousand-ft. MSL altitude, plus 500 ft. Thus, on a

magnetic course of 185�, an appropriate VFR cruising

altitude is 4,500 ft.

Each person operating an aircraft at a VFR cruising altitude shall

maintain an odd-thousand plus 500-foot altitude while on a

ANSWER: magnetic course of 0� through 179�.

When operating above 3,000 ft. AGL

but less than 18,000 ft. MSL on a magnetic course of 0� to

179�, fly at an odd thousand-ft. MSL altitude plus 500 ft.

With certain exceptions, all aircraft within 30 miles of a Class B

primary airport from the surface upward to 10,000 feet MSL must be

equipped with

ANSWER: an operable transponder having either Mode S or

4096-code capability with Mode C automatic altitude

reporting capability.

All aircraft within 30 NM of a Class B

primary airport must be equipped with an operable

transponder having either Mode S or 4096-code capability

with Mode C automatic altitude reporting capability. The

exception is any aircraft which was not originally certificated

with an engine-driven electrical system or which has not

subsequently been certified with such a system installed,

balloon, or glider may conduct operations in the airspace

within 30 NM of a Class B airspace primary airport provided

such operations are conducted (1) outside any Class A,

Class B, or Class C airspace area; and (2) below the altitude

of the ceiling of a Class B or Class C airspace area or 10,000

ft. MSL, whichever is lower.

In addition to a valid Airworthiness Certificate, what documents or

records must be aboard an aircraft during flight?

ANSWER: Operating limitations and Registration Certificate.

FAR 91.203 requires both an

Airworthiness Certificate and a Registration Certificate to be

aboard aircraft during flight. FAR 91.9 requires that

operating limitations be available in the aircraft in an

approved Airplane Flight Manual, approved manual material,

markings, and placards, or any combination thereof.

When must batteries in an emergency locator transmitter (ELT) be

replaced or recharged, if rechargeable?

ANSWER: When the ELT has been in use for more than 1 cumulative

hour.

ELT batteries must be replaced or

recharged (if rechargeable) when the transmitter has been in

use for more than 1 cumulative hr. or when 50% of their

useful life (or useful life of charge) has expired.

When are non-rechargeable batteries of an emergency locator

transmitter (ELT) required to be replaced?

ANSWER: When 50 percent of their useful life expires.

Non-rechargeable batteries of an ELT

must be replaced when 50% of their useful life expires or

after the transmitter has been in use for more than 1

cumulative hr.

Except in Alaska, during what time period should lighted position

lights be displayed on an aircraft?

ANSWER: Sunset to sunrise.

Except in Alaska, no person may

operate an aircraft during the period from sunset to sunrise

unless the aircraft's lighted position lights are on.

Unless each occupant is provided with supplemental oxygen, no

person may operate a civil aircraft of U.S. registry above a maximum

cabin pressure altitude of

ANSWER: 15,000 feet MSL.

No person may operate a civil aircraft

of U.S. registry at cabin pressure altitudes above 15,000 ft.

MSL unless each occupant is provided with supplemental

oxygen.

When operating an aircraft at cabin pressure altitudes above 12,500

feet MSL up to and including 14,000 feet MSL, supplemental

oxygen shall be used during

ANSWER: that flight time in excess of 30 minutes at those altitudes.

At cabin pressure altitudes above

12,500 ft. MSL, up to and including 14,000 ft. MSL, the

required minimum flight crew must use supplemental oxygen

only after 30 min. at those altitudes.

An operable 4096-code transponder with an encoding altimeter is

required in which airspace?

ANSWER: Class A, Class B (and within 30 miles of the Class B

primary airport), and Class C.

An operable transponder with an

encoding altimeter (Mode C) is required in Class A, Class B

(and within 30 NM of the Class B primary airport), and Class

C airspace, and at or above 10,000 ft. MSL excluding that

airspace below 2,500 ft. AGL.

In which class of airspace is acrobatic flight prohibited?

ANSWER: Class E airspace below 1,500 feet AGL.

No person may operate an aircraft in

acrobatic flight below an altitude of 1,500 ft. AGL.

No person may operate an aircraft in acrobatic flight when the

flight visibility is less than

ANSWER: 3 miles.

No person may operate an aircraft in

acrobatic flight when the flight visibility is less than 3 SM.

What is the lowest altitude permitted for acrobatic flight?

ANSWER: 1,500 feet AGL.

No person may operate an aircraft in

acrobatic flight below 1,500 ft. AGL.

No person may operate an aircraft in acrobatic flight when

ANSWER: over any congested area of a city, town, or settlement.

No person may operate an aircraft in

acrobatic flight over any congested area of a city, town, or

settlement.

With certain exceptions, when must each occupant of an aircraft

wear an approved parachute?

ANSWER: When intentionally pitching the nose of the aircraft up or

down 30� or more.

Unless each occupant of an airplane

is wearing an approved parachute, no pilot carrying any

other person (other than a crewmember) may execute any

intentional maneuver that exceeds a bank of 60� or a nose-up

or nose-down attitude of 30� relative to the horizon.

A chair-type parachute must have been packed by a certificated

and appropriately rated parachute rigger within the preceding

ANSWER: 120 days.

No pilot of a civil aircraft may allow a

parachute that is available for emergency use to be carried in

that aircraft unless it is an approved type and, if a chair type,

it has been packed by a certificated and appropriately rated

parachute rigger within the preceding 120 days.

An approved chair-type parachute may be carried in an aircraft for

emergency use if it has been packed by an appropriately rated

parachute rigger within the preceding

ANSWER: 120 days.

No pilot of a civil aircraft may allow a

parachute that is available for emergency use to be carried in

that aircraft unless it is an approved type and, if a chair type,

it has been packed by a certificated and appropriately rated

parachute rigger within the preceding 120 days.

Which is normally prohibited when operating a restricted category

civil aircraft?

ANSWER: Flight over a densely populated area.

Normally, no person may operate a

restricted category civil aircraft over a densely populated

area.

Unless otherwise specifically authorized, no person may operate an

aircraft that has an experimental certificate

ANSWER: over a densely populated area or in a congested airway.

Unless otherwise specifically

authorized, no person may operate an aircraft that has an

experimental certificate over a densely populated area or

along a congested airway.

How long does the Airworthiness Certificate of an aircraft remain

valid?

ANSWER: As long as the aircraft is maintained and operated as

required by Federal Aviation Regulations.

The airworthiness certificate of an

airplane remains valid as long as the airplane is in an

airworthy condition, i.e., operated and maintained as

required by the FARs.

The responsibility for ensuring that an aircraft is maintained in an

airworthy condition is primarily that of the

ANSWER: owner or operator.

The owner or operator of an aircraft is

primarily responsible for maintaining that aircraft in an

airworthy condition. The term "operator" includes the pilot

in command.

Who is responsible for ensuring Airworthiness Directives (AD's)

are complied with?

ANSWER: Owner or operator.

Airworthiness Directives (ADs) are

regulatory and must be complied with, unless a specific

exemption is granted. It is the responsibility of the owner or

operator to assure compliance with all pertinent ADs,

including those ADs that require recurrent or continuing

action.

The responsibility for ensuring that maintenance personnel make

the appropriate entries in the aircraft maintenance records

indicating the aircraft has been approved for return to service lies

with the

ANSWER: owner or operator.

Each owner or operator of an aircraft

shall ensure that maintenance personnel make the

appropriate entries in the aircraft maintenance records

indicating the aircraft has been approved for return to

service.

Who is responsible for ensuring appropriate entries are made in

maintenance records indicating the aircraft has been approved for

return to service?

ANSWER: Owner or operator.

It is the responsibility of the owner or

operator of an aircraft to ensure that appropriate entries are

made in maintenance records by maintenance personnel

indicating the aircraft has been approved for return to

service.

If an alteration or repair substantially affects an aircraft's operation

in flight, that aircraft must be test flown by an appropriately-rated

pilot and approved for return to service prior to being operated

ANSWER: with passengers aboard.

If an alteration or repair has been

made that substantially affects the airplane's flight

characteristics, the airplane must be test flown and approved

for return to service by an appropriately rated pilot prior to

being operated with passengers aboard. The test pilot must

be at least a private pilot and appropriately rated for the

airplane being tested and must make an operational check of

the alteration or repair made, and log the flight in the aircraft

records.

Before passengers can be carried in an aircraft that has been

altered in a manner that may have appreciably changed its flight

characteristics, it must be flight tested by an appropriately-rated

pilot who holds at least a

ANSWER: Private Pilot Certificate.

If an alteration or repair has been

made that may have changed an airplane's flight

characteristics, the airplane must be test flown and approved

for return to service by an appropriately rated pilot prior to

being operated with passengers aboard. The test pilot must

be at least a private pilot and appropriately rated for the

airplane being tested.

A 100-hour inspection was due at 3302.5 hours. The 100-hour

inspection was actually done at 3309.5 hours. When is the next

100-hour inspection due?

ANSWER: 3402.5 hours.

Since the 100-hr. inspection was due

at 3302.5 hr., the next 100-hr. inspection is due at 3402.5

(3302.5 + 100). The excess time used before the 100-hr.

inspection was done must be included in computing the next

100 hr. of time in service.

An aircraft's annual inspection was performed on July 12, this year.

The next annual inspection will be due no later than

ANSWER: July 31, next year.

Annual inspections expire on the last

day of the 12th calendar month after the previous annual

inspection. If an annual inspection is performed on July 12

of this year, it will expire at midnight on July 31 next year.

What aircraft inspections are required for rental aircraft that are

also used for flight instruction?

ANSWER: Annual and 100-hour inspections.

All aircraft that are used for hire (e.g.,

rental) and flight instruction must be inspected on a 100-hr.

basis. Also an annual inspection must be completed.

An aircraft had a 100-hour inspection when the tachometer read

1259.6. When is the next 100-hour inspection due?

ANSWER: 1359.6 hours.

The next 100-hr. inspection is due

within 100 hr. of time in service. The 100-hr. may be exceeded

by 10 hr. in order to get to a place where the work can be

done. Add 100 hr. to 1259.6 to get the next inspection, due at

1359.6.

No person may use an ATC transponder unless it has been tested

and inspected within at least the preceding

ANSWER: 24 calendar months.

No person may use an ATC

transponder that is specified in the regulations unless within

the preceding 24 calendar months it has been tested and

found to comply with its operating specifications.

Maintenance records show the last transponder inspection was

performed on September 1, 1993. The next inspection will be due no

later than

ANSWER: September 30, 1995.

No person may use an ATC

transponder that is specified in the regulations unless within

the preceding 24 calendar months it has been tested and

found to comply with its operating specifications. Thus, if

the last inspection was performed on September 1, 1993, the

next inspection will be due no later than September 30, 1995.

Completion of an annual inspection and the return of the aircraft to

service should always be indicated by

ANSWER: an appropriate notation in the aircraft maintenance

records.

Completion of an annual inspection

and the return of the aircraft to service should always be

indicated by an appropriate notation in the aircraft's

maintenance records.

To determine the expiration date of the last annual aircraft

inspection, a person should refer to the

ANSWER: aircraft maintenance records.

After maintenance inspections have

been completed, maintenance personnel should make the

appropriate entries in the aircraft maintenance records or

logbooks. This is where the date of the last annual

inspection can be found.

Which records or documents shall the owner or operator of an

aircraft keep to show compliance with an applicable Airworthiness

Directive?

ANSWER: Aircraft maintenance records.

Aircraft maintenance records must

show the current status of applicable airworthiness

directives (ADs) including, for each, the method of

compliance, the AD number, and revision date. If the AD

involves recurring action, the time and date when the next

action is required.

The airworthiness of an aircraft can be determined by a preflight

inspection and a

ANSWER: review of the maintenance records.

As pilot in command, you are

responsible for determining whether your aircraft is in

condition for safe flight. Only by conducting a preflight

inspection and a review of the maintenance records can you

determine whether all required maintenance has been

performed and, thus, whether the aircraft is airworthy.

If an aircraft is involved in an accident which results in substantial

damage to the aircraft, the nearest NTSB field office should be

notified

ANSWER: immediately.

The NTSB must be notified

immediately and by the most expeditious means possible

when an aircraft accident or any of various listed incidents

occurs or when an aircraft is overdue and is believed to have

been in an accident.

Which incident would necessitate an immediate notification to the

nearest NTSB field office?

ANSWER: An in-flight fire.

The NTSB must be notified

immediately and by the most expeditious means possible

when an aircraft accident or any of various listed incidents

occurs or when an aircraft is overdue and believed to have

been in an accident. The following are considered incidents:

1. Flight control system malfunction or failure;

2. Inability of any required flight crewmember to perform

normal flight duties as a result of injury or illness;

3. Failure of structural components of a turbine engine,

excluding compressor and turbine blades and vanes;

4. In-flight fire; or

5. Aircraft collision in flight.

Which incident requires an immediate notification to the nearest

NTSB field office?

ANSWER: Flight control system malfunction or failure.

The NTSB must be notified

immediately and by the most expeditious means possible

when an aircraft accident or any of various listed incidents

occurs or when an aircraft is overdue and believed to have

been in an accident. The following are considered incidents:

1. Flight control system malfunction or failure;

2. Inability of any required flight crewmember to perform

normal flight duties as a result of injury or illness;

3. Failure of structural components of a turbine engine,

excluding compressor and turbine blades and vanes;

4. In-flight fire; or

5. Aircraft collision in flight.

Which incident requires an immediate notification be made to the

nearest NTSB field office?

ANSWER: An overdue aircraft that is believed to be involved in an

accident.

The NTSB must be notified

immediately and by the most expeditious means possible

when an aircraft is overdue and is believed to have been

involved in an accident.

May aircraft wreckage be moved prior to the time the NTSB takes

custody?

ANSWER: Yes, but only to protect the wreckage from further

damage.

Prior to the time the Board or its

authorized representative takes custody of aircraft wreckage,

mail, or cargo, such wreckage, mail, or cargo may not be

disturbed or moved except to the extent necessary:

1. To remove persons injured or trapped;

2. To protect the wreckage from further damage; or

3. To protect the public from injury.

The operator of an aircraft that has been involved in an accident is

required to file an accident report within how many days?

ANSWER: 10.

The operator of an aircraft shall file a

report on NTSB Form 6120.1/2 within 10 days after an

accident, or after 7 days if an overdue aircraft is still missing.

A report on an incident for which notification is required

shall be filed only as required.

The operator of an aircraft that has been involved in an incident is

required to submit a report to the nearest field office of the NTSB

ANSWER: when requested.

The operator of an aircraft shall file a

report on NTSB Form 6120.1/2 only when requested. A

report is required within 10 days of an accident, or after 7

days if an overdue aircraft is still missing.

The four forces acting on an airplane in flight are

ANSWER: lift, weight, thrust, and drag.

Lift is produced by the wings and

opposes weight, which is the result of gravity. Thrust is

produced by the engine/propeller and opposes drag, which

is the resistance of the air as the airplane moves through it.

When are the four forces that act on an airplane in equilibrium?

ANSWER: During unaccelerated flight.

The four forces (lift, weight, thrust,

and drag) that act on an airplane are in equilibrium during

unaccelerated flight.

What is the relationship of lift, drag, thrust, and weight when the

airplane is in straight-and-level flight?

ANSWER: Lift equals weight and thrust equals drag.

When the airplane is in

straight-and-level flight (assuming no change of airspeed), it

is not accelerating, and therefore lift equals weight and

thrust equals drag.

The term "angle of attack" is defined as the angle

ANSWER: between the wing chord line and the relative wind.

The angle of attack is the angle

between the wing chord line and the direction of the relative

wind. The wing chord line is a straight line from the leading

edge to the trailing edge of the wing. The relative wind is the

direction of airflow relative to the wing when the wing is

moving through the air.

Figure 1

(Refer to figure 1.) The acute angle A is the angle of

ANSWER: attack.

The angle between the relative wind

and the wing chord line is the angle of attack. The wing

chord line is a straight line from the leading edge to the

trailing edge of the wing.

How will frost on the wings of an airplane affect takeoff

performance?

ANSWER: Frost will disrupt the smooth flow of air over the wing,

adversely affecting its lifting capability.

Frost does not change the basic

aerodynamic shape of the wing, but the roughness of its

surface spoils the smooth flow of air, thus causing an

increase in drag and an early airflow separation over the

wing, resulting in a loss of lift.

In what flight condition is torque effect the greatest in a

single-engine airplane?

ANSWER: Low airspeed, high power, high angle of attack.

The effect of torque increases in

direct proportion to engine power and inversely to airspeed.

Thus, at low airspeeds, high angles of attack, and high

power settings, torque is the greatest.

The left turning tendency of an airplane caused by P-factor is the

result of the

ANSWER: propeller blade descending on the right, producing more

thrust than the ascending blade on the left.

Asymmetric propeller loading

(P-factor) occurs when the airplane is flown at a high angle

of attack. The downward-moving blade on the right side of

the propeller (as seen from the rear) has a higher angle of

attack, which creates higher thrust than the upward moving

blade on the left. Thus, the airplane yaws around the vertical

axis to the left.

When does P-factor cause the airplane to yaw to the left?

ANSWER: When at high angles of attack.

P-factor or asymmetric propeller

loading occurs when an airplane is flown at a high angle of

attack because the downward-moving blade on the right

side of the propeller (as seen from the rear) has a higher

angle of attack, which creates higher thrust than the upward

moving blade on the left. Thus, the airplane yaws around the

vertical axis to the left.

What is the purpose of the rudder on an airplane?

ANSWER: To control yaw.

The rudder is used to control yaw,

which is rotation about the airplane's vertical axis.

An airplane said to be inherently stable will

ANSWER: require less effort to control.

An inherently stable airplane will

usually return to the original condition of flight (except when

in a bank) if disturbed by a force such as air turbulence.

Thus, an inherently stable airplane will require less effort to

control than an inherently unstable one.

What determines the longitudinal stability of an airplane?

ANSWER: The location of the CG with respect to the center of lift.

The location of the center of gravity

with respect to the center of lift determines, to a great extent,

the longitudinal stability of the airplane. Positive stability is

attained by having the center of lift behind the center of

gravity. Then the tail provides negative lift, creating a

downward tail force, which counteracts the nose's tendency

to pitch down.

What causes an airplane (except a T-tail) to pitch nosedown when

power is reduced and controls are not adjusted?

ANSWER: The downwash on the elevators from the propeller

slipstream is reduced and elevator effectiveness is reduced.

The relative wind on the tail is the

result of the airplane's movement through the air and the

propeller slipstream. When that slipstream is reduced, the

horizontal stabilizer (except a T-tail) will produce less

negative lift and the nose will pitch down.

The angle of attack at which an airplane wing stalls will

ANSWER: remain the same regardless of gross weight.

A given airplane wing will always stall

at the same angle of attack regardless of airspeed, weight,

load factor, or density altitude. Each wing has a particular

angle of attack (the critical angle of attack) at which the

airflow separates from the upper surface of the wing and the

stall occurs.

The amount of excess load that can be imposed on the wing of an

airplane depends upon the

ANSWER: speed of the airplane.

The amount of excess load that can be

imposed on the wing depends upon how fast the airplane is

flying. At low speeds, the maximum available lifting force of

the wing is only slightly greater than the amount necessary

to support the weight of the airplane. Thus, any excess load

would simply cause the airplane to stall. At high speeds, the

lifting capacity of the wing is so great (as a result of the

greater flow of air over the wings) that a sudden movement

of the elevator controls (strong gust of wind) may increase

the load factor beyond safe limits. This is why maximum

speeds are established by airplane manufacturers.

Which basic flight maneuver increases the load factor on an

airplane as compared to straight-and-level flight?

ANSWER: Turns.

Turns increase the load factor

because the lift from the wings is used to pull the airplane

around a corner as well as to offset the force of gravity. The

wings must carry the airplane's weight plus offset centrifugal

force during the turn. For example, a 60� bank results in a

load factor of 2; i.e., the wings must support twice the

weight they do in level flight.

During an approach to a stall, an increased load factor will cause

the airplane to

ANSWER: stall at a higher airspeed.

The greater the load (whether from

gross weight or from centrifugal force), the more lift is

required. Therefore, an airplane will stall at higher airspeeds

when the load and/or load factor is increased.

Figure 2

(Refer to figure 2.) If an airplane weighs 2,300 pounds, what

approximate weight would the airplane structure be required to

support during a 60� banked turn while maintaining altitude?

ANSWER: 4,600 pounds.

Note on Fig. 2 that, at a 60� bank

angle, the load factor is 2. Thus, a 2,300-lb. airplane in a 60�

bank would require its wings to support 4,600 lb. (2,300 x 2).

Figure 2

(Refer to figure 2.) If an airplane weighs 3,300 pounds, what

approximate weight would the airplane structure be required to

support during a 30� banked turn while maintaining altitude?

ANSWER: 3,960 pounds.

Look on the left side of the chart in

Fig. 2 to see that, at a 30� bank angle, the load factor is 1.154.

Thus, a 3,300-lb. airplane in a 30� bank would require its

wings to support 3,808.2 lb. (3,300 x 1.154). This answer is

closest to this value.

Figure 2

(Refer to figure 2.) If an airplane weighs 4,500 pounds, what

approximate weight would the airplane structure be required to

support during a 45� banked turn while maintaining altitude?

ANSWER: 6,750 pounds.

Look on the left side of the chart

under 45� and note that the load factor curve is 1.414. Thus,

a 4,500-lb. airplane in a 45� bank would require its wings to

support 6,363 lb. (4,500 x 1.414). This answer is closest to

this value.

What is one purpose of wing flaps?

ANSWER: To enable the pilot to make steeper approaches to a

landing without increasing the airspeed.

Extending the flaps increases the

wing camber and the angle of attack of the wing. This

increases wing lift and induced drag, which enables the pilot

to make steeper approaches to a landing without an increase

in airspeed.

One of the main functions of flaps during approach and landing is

to

ANSWER: increase the angle of descent without increasing the

airspeed.

Extending the flaps increases the wing

camber and the angle of attack of the wing. This increases

wing lift and induced drag, which enables the pilot to

increase the angle of descent without increasing the

airspeed.

An abnormally high engine oil temperature indication may be

caused by

ANSWER: the oil level being too low.

Operating with an excessively low oil

level prevents the oil from being cooled adequately; i.e., an

inadequate supply of oil will not be able to transfer engine

heat to the engine's oil cooler (similar to a car engine's water

radiator). Insufficient oil may also damage an engine from

excessive friction within the cylinders and on other

metal-to-metal contact parts.

Excessively high engine temperatures will

ANSWER: cause loss of power, excessive oil consumption, and

possible permanent internal engine damage.

Excessively high engine temperatures

will result in loss of power, excessive oil consumption, and

possible permanent internal engine damage.

For internal cooling, reciprocating aircraft engines are especially

dependent on

ANSWER: the circulation of lubricating oil.

An engine accomplishes much of its

cooling by the flow of oil through the lubrication system.

The lubrication system aids in cooling by reducing friction

and absorbing heat from internal engine parts. Many

airplane engines use an oil cooler, a small radiator device

that will cool the oil before it is recirculated through the

engine.

If the engine oil temperature and cylinder head temperature gauges

have exceeded their normal operating range, the pilot may have

been operating with

ANSWER: too much power and with the mixture set too lean.

If the engine oil temperature and

cylinder head temperature gauges exceed their normal

operating range, it is possible that the power setting is too

high and the fuel/air mixture is set excessively lean. These

conditions may cause engine overheating.

What action can a pilot take to aid in cooling an engine that is

overheating during a climb?

ANSWER: Reduce rate of climb and increase airspeed.

If an airplane is overheating during a

climb, the engine temperature will be decreased if the

airspeed is increased. Airspeed will increase if the rate of

climb is reduced.

What is one procedure to aid in cooling an engine that is

overheating?

ANSWER: Enrich the fuel mixture.

Enriched fuel mixtures have a cooling

effect on an engine.

One purpose of the dual ignition system on an aircraft engine is to

provide for

ANSWER: improved engine performance.

Most airplane engines are equipped

with dual ignition systems, which have two magnetos to

supply the electrical current to two spark plugs for each

combustion chamber. The main advantages of the dual

system are increased safety and improved burning and

combustion of the mixture, which results in improved

performance.

With regard to carburetor ice, float-type carburetor systems in

comparison to fuel injection systems are generally considered to be

ANSWER: more susceptible to icing.

Float-type carburetor systems are

generally more susceptible to icing than fuel-injected

engines. When there is visible moisture or high humidity

and the temperature is between 20�F and 70�F, icing is

possible, particularly at low power settings.

The operating principle of float-type carburetors is based on the

ANSWER: difference in air pressure at the venturi throat and the air

inlet.

In a float-type carburetor, air flows

into the carburetor and through a venturi tube (a narrow

throat in the carburetor). As the air flows more rapidly

through the venturi, a low pressure area is created which

draws the fuel from a main fuel jet located at the throat of the

carburetor and into the airstream, where it is mixed with

flowing air. It is called a float-type carburetor in that a ready

supply of gasoline is kept in the float bowl by a float, which

activates a fuel inlet valve.

If an aircraft is equipped with a fixed-pitch propeller and a

float-type carburetor, the first indication of carburetor ice would most likely be

ANSWER: loss of RPM.

In an airplane equipped with a

fixed-pitch propeller and float-type carburetor, the first

indication of carburetor ice would be a loss in RPM.

The presence of carburetor ice in an aircraft equipped with a

fixed-pitch propeller can be verified by applying carburetor heat

and noting

ANSWER: a decrease in RPM and then a gradual increase in RPM.

The presence of carburetor ice in an

airplane equipped with a fixed-pitch propeller can be verified

by applying carburetor heat and noting a decrease in RPM

and then a gradual increase. The decrease in RPM as heat is

applied is caused by less dense hot air entering the engine

and reducing power output. Also, if ice is present, melting

water entering the engine may also cause a loss in

performance. As the carburetor ice melts, however, the RPM

gradually increases until it stabilizes when the ice is

completely removed.

Which condition is most favorable to the development of

carburetor icing?

ANSWER: Temperature between 20 and 70�F and high humidity.

When the temperature is between

20�F and 70�F with visible moisture or high humidity, one

should be on the alert for carburetor ice. During low or

closed throttle settings, an engine is particularly susceptible

to carburetor icing.

The possibility of carburetor icing exists even when the ambient air

temperature is as

ANSWER: high as 70�F and the relative humidity is high.

When the temperature is between

20�F and 70�F with visible moisture or high humidity, one

should be on the alert for carburetor ice. During low or

closed throttle settings, an engine is particularly susceptible

to carburetor icing.

Generally speaking, the use of carburetor heat tends to

ANSWER: decrease engine performance.

Use of carburetor heat tends to

decrease the engine performance and also to increase the

operating temperature. Warmer air is less dense, and engine

performance decreases with density. Thus, carburetor heat

should not be used when full power is required (as during

takeoff) or during normal engine operation except as a check

for the presence or removal of carburetor ice.

Applying carburetor heat will

ANSWER: enrich the fuel/air mixture.

Applying carburetor heat will enrich

the fuel/air mixture. Warm air is less dense than cold air,

hence the application of heat increases the fuel-to-air ratio.

What change occurs in the fuel/air mixture when carburetor heat is

applied?

ANSWER: The fuel/air mixture becomes richer.

When carburetor heat is applied, hot

air is introduced into the carburetor. Hot air is less dense

than cold air; therefore, the decrease in air density with a

constant amount of fuel makes a richer mixture.

During the run-up at a high-elevation airport, a pilot notes a slight

engine roughness that is not affected by the magneto check but

grows worse during the carburetor heat check. Under these

circumstances, what would be the most logical initial action?

ANSWER: Check the results obtained with a leaner setting of the

mixture.

If, during a run-up at a high-elevation

airport, you notice a slight roughness that is not affected by

a magneto check but grows worse during the carburetor heat

check, you should check the results obtained with a leaner

setting of the mixture control. At a high-elevation field, the

air is less dense and the application of carburetor heat

increases the already too rich fuel-to-air mixture. By leaning

the mixture during the run-up, the condition should improve.

The basic purpose of adjusting the fuel/air mixture at altitude is to

ANSWER: decrease the fuel flow in order to compensate for

decreased air density.

At higher altitudes the air density is

decreased. Thus the mixture control must be adjusted to

decrease the fuel flow in order to maintain a constant fuel/air

ratio.

While cruising at 9,500 feet MSL, the fuel/air mixture is properly

adjusted. What will occur if a descent to 4,500 feet MSL is made

without readjusting the mixture?

ANSWER: The fuel/air mixture may become excessively lean.

At 9,500 ft., the mixture control is

adjusted to provide the proper fuel/air ratio. As the airplane

descends, the density of the air increases and there will be

less fuel to air in the ratio, causing a leaner running engine.

This excessively lean mixture will create higher cylinder

temperature and may cause detonation.

Detonation occurs in a reciprocating aircraft engine when

ANSWER: the unburned charge in the cylinders explodes instead of

burning normally.

Detonation occurs when the fuel/air

mixture in the cylinders explodes instead of burning

normally. This more rapid force slams the piston down

instead of pushing it.

If a pilot suspects that the engine (with a fixed-pitch propeller) is

detonating during climb-out after takeoff, the initial corrective

action to take would be to

ANSWER: lower the nose slightly to increase airspeed.

If you suspect engine detonation

during climb-out after takeoff, you would normally decrease

the pitch to increase airspeed (more cooling) and decrease

the load on the engine. Detonation is usually caused by a

poor grade of fuel or an excessive engine temperature.

If the grade of fuel used in an aircraft engine is lower than specified

for the engine, it will most likely cause

ANSWER: detonation.

If the grade of fuel used in an airplane

engine is lower than specified for the engine, it will probably

cause detonation. Lower grades of fuel ignite at lower

temperatures. A higher temperature engine (which should

use a higher grade of fuel) may cause lower grade fuel to

explode (detonate) rather than burn evenly.

The uncontrolled firing of the fuel/air charge in advance of normal

spark ignition is known as

ANSWER: pre-ignition.

Pre-ignition is the ignition of the fuel

prior to normal ignition or ignition before the electrical arcing

occurs at the spark plug. Pre-ignition may be caused by

excessively hot exhaust valves, carbon particles, or spark

plugs and electrodes heated to an incandescent, or glowing,

state. These hot spots are usually caused by high

temperatures encountered during detonation. A significant

difference between pre-ignition and detonation is that if the

conditions for detonation exist in one cylinder they usually

exist in all cylinders, but pre-ignition often takes place in

only one or two cylinders.

What type fuel can be substituted for an aircraft if the

recommended octane is not available?

ANSWER: The next higher octane aviation gas.

If the recommended octane is not

available for an airplane, the next higher octane aviation gas

should be used.

Filling the fuel tanks after the last flight of the day is considered a

good operating procedure because this will

ANSWER: prevent moisture condensation by eliminating airspace in

the tanks.

Filling the fuel tanks after the last

flight of the day is considered good operating practice

because it prevents moisture condensation by eliminating

airspace in the tanks. Humid air may result in condensation

at night when the airplane cools.

On aircraft equipped with fuel pumps, when is the auxiliary electric

driven pump used?

ANSWER: In the event engine-driven fuel pump fails.

In a fuel pump system, two fuel

pumps are used on most airplanes. The main fuel pump is

engine-driven and an auxiliary electric-driven pump is

provided for use in the event the engine pump fails.

Which would most likely cause the cylinder head temperature and

engine oil temperature gauges to exceed their normal operating

ranges?

ANSWER: Using fuel that has a lower-than-specified fuel rating.

Use of fuel with lower-than-specified

fuel ratings, e.g., 80 octane instead of 100, can cause many

problems, including higher operating temperatures,

detonation, etc.

How is engine operation controlled on an engine equipped with a

constant-speed propeller?

ANSWER: The throttle controls power output as registered on the

manifold pressure gauge and the propeller control regulates

engine RPM.

Airplanes equipped with

controllable-pitch propellers have both a throttle control and

a propeller control. The throttle controls the power output of

the engine, which is registered on the manifold pressure

gauge. This is a simple barometer that measures the air

pressure in the engine intake manifold in inches of mercury.

The propeller control regulates the engine RPM, which is

registered on a tachometer.

A precaution for the operation of an engine equipped with a

constant-speed propeller is to

ANSWER: avoid high manifold pressure settings with low RPM.

For any given RPM, there is a

manifold pressure that should not be exceeded. Manifold

pressure is excessive for a given RPM when the cylinder

design pressure is exceeded, placing undue stress on them.

If repeated or extended, the stress would weaken the

cylinder components and eventually cause engine failure.

What is an advantage of a constant-speed propeller?

ANSWER: Permits the pilot to select the blade angle for the most

efficient performance.

A controllable-pitch propeller

(constant-speed) permits the pilot to select the blade angle

that will result in the most efficient performance given the

flight conditions. A low blade angle and a decreased pitch

reduces the propeller drag and allows more engine RPM

(power) for takeoffs. After airspeed is attained during

cruising flight, the propeller blade is changed to a higher

angle to increase pitch. The blade takes a larger bite of air at

a lower RPM and consequently increases the efficiency of

the flight. This process is similar to shifting gears in an

automobile from low to high gear.

What effect does high density altitude, as compared to low density

altitude, have on propeller efficiency and why?

ANSWER: Efficiency is reduced because the propeller exerts less

force at high density altitudes than at low density altitudes.

The propeller produces thrust in

proportion to the mass of air being accelerated through the

rotating propeller. If the air is less dense, the propeller

efficiency is decreased. Remember, higher density altitude

refers to less dense air.

What should be the first action after starting an aircraft engine?

ANSWER: Adjust for proper RPM and check for desired indications

on the engine gauges.

After the engine starts, the engine

speed should be adjusted to the proper RPM. Then the

engine gauges should be reviewed, with the oil pressure

being the most important gauge initially.

Should it become necessary to handprop an airplane engine, it is

extremely important that a competent pilot

ANSWER: be at the controls in the cockpit.

Because of the hazards involved in

handstarting airplane engines, every precaution should be

exercised. It is extremely important that a competent pilot be

at the controls in the cockpit. Also, the person turning the

propeller should be thoroughly familiar with the technique.

During the preflight inspection who is responsible for determining

the aircraft as safe for flight?

ANSWER: The pilot in command.

During the preflight inspection, the

pilot in command is responsible for determining whether the

airplane is in condition for safe flight.

Who is primarily responsible for maintaining an aircraft in

airworthy condition?

ANSWER: Owner or operator.

The owner or operator of an airplane

is primarily responsible for maintaining an airplane in an

airworthy condition, including compliance with all applicable

Airworthiness Directives (ADs).

How should an aircraft preflight inspection be accomplished for the

first flight of the day?

ANSWER: Thorough and systematic means recommended by the

manufacturer.

For the first flight of the day, the

preflight inspection should be accomplished by a thorough

and systematic means recommended by the manufacturer.

As altitude increases, the indicated airspeed at which a given

airplane stalls in a particular configuration will

ANSWER: remain the same regardless of altitude.

All the performance factors of an

airplane are dependent upon air density. As air density

decreases, the airplane stalls at a higher true airspeed.

However, you cannot detect the effect of high density

altitude on your airspeed indicator. Accordingly, an airplane

will stall in a particular configuration at the same indicated

airspeed regardless of altitude.

The pitot system provides impact pressure for which instrument?

ANSWER: Airspeed indicator.

The pitot system provides impact

pressure, or ram pressure, for only the airspeed indicator.

Which instrument will become inoperative if the pitot tube

becomes clogged?

ANSWER: Airspeed.

The pitot-static system is a source of

pressure for the altimeter, vertical-speed indicator, and

airspeed indicator. The pitot tube is connected directly to

the airspeed indicator and provides impact pressure for it

alone. Thus, if the pitot tube becomes clogged, only the

airspeed indicator will become inoperative.

If the pitot tube and outside static vents become clogged, which

instruments would be affected?

ANSWER: The altimeter, airspeed indicator, and vertical speed

indicator.

The pitot-static system is a source of

air pressure for the operation of the altimeter, airspeed

indicator, and vertical speed indicator. Thus, if the pitot and

outside static vents become clogged, all of these

instruments will be affected.

Which instrument(s) will become inoperative if the static vents

become clogged?

ANSWER: Airspeed, altimeter, and vertical speed.

The pitot-static system is a source of

air pressure for the operation of the airspeed indicator,

altimeter, and vertical speed indicator. Thus, if the static

vents become clogged, all three instruments will become

inoperative.

What does the red line on an airspeed indicator represent?

ANSWER: Never-exceed speed.

The red line on an airspeed indicator

indicates the maximum speed at which the airplane can be

operated in smooth air, which should never be exceeded

intentionally. This speed is known as the never-exceed

speed.

What is an important airspeed limitation that is not color coded on

airspeed indicators?

ANSWER: Maneuvering speed.

The maneuvering speed of an airplane

is an important airspeed limitation not color-coded on the

airspeed indicator. It is found in the airplane manual (Pilot's

Operating Handbook) or placarded in the cockpit.

Maneuvering speed is the maximum speed at which full

deflection of the airplane controls can be made without

incurring structural damage. Maneuvering speed or less

should be held in turbulent air to prevent structural damage

due to excessive loads.

Figure 4

(Refer to figure 4.) What is the caution range of the airplane?

ANSWER: 165 to 208 MPH.

The caution range is indicated by the

yellow arc on the airspeed indicator. Operation within this

range is safe only in smooth air. The airspeed indicator in

Fig. 4 indicates the caution range from 165 to 208 MPH.

Figure 4

(Refer to figure 4.) The maximum speed at which the airplane can be

operated in smooth air is

ANSWER: 208 MPH.

The maximum speed at which the

airplane can be operated in smooth air is indicated by the red

radial line. The airspeed indicator in Fig. 4 indicates the red

line is at 208 MPH.

Figure 4

(Refer to figure 4.) What is the full flap operating range for the

airplane?

ANSWER: 60 to 100 MPH.

The full flap operating range is

indicated by the white arc on the airspeed indicator. The

airspeed indicator in Fig. 4 indicates the full flap operating

range is from 60 to 100 MPH.

Figure 4

(Refer to figure 4.) Which color identifies the never-exceed speed?

ANSWER: The red radial line.

The never-exceed speed is indicated

by a red line and is found at the upper limit of the yellow arc.

Operating above this speed may result in structural damage.

Figure 4

(Refer to figure 4.) Which color identifies the power-off stalling

speed in a specified configuration?

ANSWER: Lower limit of the green arc.

The lower airspeed limit of the green

arc indicates the power-off stalling speed in a specified

configuration. "Specified configuration" refers to flaps up

and landing gear retracted.

Figure 4

(Refer to figure 4.) What is the maximum flaps-extended speed?

ANSWER: 100 MPH.

The maximum flaps-extended speed is

indicated by the upper limit of the white arc. This is the

highest air speed at which a pilot should extend full flaps. At

higher airspeeds, severe strain or structural failure could

result. The upper limit of the white arc on the airspeed

indicator shown in Fig. 4 indicates 100 MPH.

Figure 4

(Refer to figure 4.) Which color identifies the normal flap operating

range?

ANSWER: The white arc.

The normal flap operating range is

indicated by the white arc. The power-off stall speed with

flaps extended is at the lower limit of the arc, and the

maximum speed at which flaps can be extended without

damage to them is the upper limit of the arc.

Figure 4

(Refer to figure 4.) Which color identifies the power-off stalling

speed with wing flaps and landing gear in the landing

configuration?

ANSWER: Lower limit of the white arc.

The lower limit of the white arc

indicates the power-off stalling speed with wing flaps and

landing gear in the landing position.

Figure 4

(Refer to figure 4.) What is the maximum structural cruising speed?

ANSWER: 165 MPH.

The maximum structural cruising

speed is the maximum speed for normal operation and is

indicated as the upper limit of the green arc on an airspeed

indicator. The upper limit of the green arc on the airspeed

indicator shown in Fig. 4 indicates 165 MPH.

Figure 3

(Refer to figure 3.) Altimeter 2 indicates

ANSWER: 14,500 feet.

Altimeter 2 indicates 14,500 ft.

because the shortest needle is between the 1 and the 2,

indicating about 15,000 ft; the middle needle is between 4

and 5, indicating 4,500 ft; and the long needle is on 5,

indicating 500 ft., i.e., 14,500 ft.

Figure 3

(Refer to figure 3.) Altimeter 1 indicates

ANSWER: 10,500 feet.

The altimeter has three needles. The

short needle indicates 10,000-ft. intervals, the middle-length

needle indicates 1,000-ft. intervals, and the long needle

indicates 100-ft. intervals. In altimeter 1, the shortest needle

is on 1, which indicates about 10,000 ft. The middle-length

needle indicates half-way between zero and 1, which is 500

ft. This is confirmed by the longest needle on 5, indicating

500 ft., i.e., 10,500 ft.

Figure 3

(Refer to figure 3.) Altimeter 3 indicates

ANSWER: 9,500 feet.

Altimeter 3 indicates 9,500 ft. because

the shortest needle is near 1 (i.e., about 10,000 ft.), the middle

needle is between 9 and the 0, indicating between 9,000 and

10,000 ft., and the long needle is on 5, indicating 500 ft.

Figure 3

(Refer to figure 3.) Which altimeter(s) indicate(s) more than 10,000

feet?

ANSWER: 1 and 2 only.

Altimeters 1 and 2 indicate over 10,000

ft. because 1 indicates 10,500 ft. and 2 indicates 14,500 ft.

The short needle on 3 points just below 1, i.e., below 10,000

ft.

What is absolute altitude?

ANSWER: The vertical distance of the aircraft above the surface.

Absolute altitude is altitude above the

surface, i.e., AGL.

What is true altitude?

ANSWER: The vertical distance of the aircraft above sea level.

True altitude is the actual altitude

above mean sea level, i.e., MSL.

What is density altitude?

ANSWER: The pressure altitude corrected for nonstandard

temperature.

Density altitude is the pressure

altitude corrected for nonstandard temperature.

Under what condition is indicated altitude the same as true

altitude?

ANSWER: When at sea level under standard conditions.

Indicated altitude (what you read on

your altimeter) approximates the true altitude (distance

above mean sea level) when standard conditions exist and

your altimeter is properly calibrated.

What is pressure altitude?

ANSWER: The altitude indicated when the barometric pressure scale

is set to 29.92.

Pressure altitude is the airplane's

height above the standard datum plane of 29.92" Hg. If the

altimeter is set to 29.92" Hg, the indicated altitude is the

pressure altitude.

Altimeter setting is the value to which the barometric pressure

scale of the altimeter is set so the altimeter indicates

ANSWER: true altitude at field elevation.

Altimeter setting is the value to which

the scale of the pressure altimeter is set so that the altimeter

indicates true altitude at field elevation.

If it is necessary to set the altimeter from 29.15 to 29.85, what

change occurs?

ANSWER: 700-foot increase in indicated altitude.

When increasing the altimeter setting

from 29.15 to 29.85, the indicated altitude increases by 700 ft.

The altimeter-indicated altitude moves in the same direction

as the altimeter setting and changes about 1,000 ft. for every

change of 1" Hg in the altimeter setting.

How do variations in temperature affect the altimeter?

ANSWER: Pressure levels are raised on warm days and the indicated

altitude is lower than true altitude.

On warm days, the atmospheric

pressure levels are higher than on cold days. Your altimeter

will indicate a lower than true altitude. Remember, "low to

high, clear the sky."

If the outside air temperature (OAT) at a given altitude is warmer

than standard, the density altitude is

ANSWER: higher than pressure altitude.

When temperature increases, the air

expands and therefore becomes less dense. This decrease in

density means a higher density altitude. Pressure altitude is

based on standard temperature. Thus, density altitude

exceeds pressure altitude when the temperature is warmer

than standard.

Figure 7

(Refer to figure 7.) The proper adjustment to make on the attitude

indicator during level flight is to align the

ANSWER: miniature airplane to the horizon bar.

The horizon bar (marked as B) on Fig.

7 represents the true horizon. This bar is fixed to the gyro

and remains on a horizontal plane as the airplane is pitched

or banked about its lateral or longitudinal axis, indicating the

attitude of the airplane relative to the true horizon. An

adjustment knob is provided, with which the pilot may move

the miniature airplane (marked as C) up or down to align the

miniature airplane with the horizontal bar to suit the pilot's

line of vision.

Figure 7

(Refer to figure 7.) How should a pilot determine the direction of

bank from an attitude indicator such as the one illustrated?

ANSWER: By the relationship of the miniature airplane (C) to the

deflected horizon bar (B).

The direction of bank on the attitude

indicator (AI) is indicated by the relationship of the

miniature airplane to the deflecting horizon bar. The

miniature airplane's relative position to the horizon indicates

its attitude: nose high, nose low, left bank, right bank. As

you look at the attitude indicator, you see your airplane as it

is positioned with respect to the actual horizon. The attitude

indicator in Fig. 7 indicates a level right turn.

Figure 5

(Refer to figure 5.) A turn coordinator provides an indication of the

ANSWER: movement of the aircraft about the yaw and roll axes.

There really are no yaw and roll axes,

i.e., an airplane yaws about its vertical axis and rolls about

its longitudinal axis. However, this is the best answer since

the turn coordinator does indicate the roll and yaw

movement of the airplane. The movement of the miniature

airplane is proportional to the roll rate of the airplane. When

the roll rate is reduced to zero (i.e., when the bank is held

constant) the instrument provides an indication of the rate

of turn.

Figure 6

(Refer to figure 6.) To receive accurate indications during flight

from a heading indicator, the instrument must be

ANSWER: periodically realigned with the magnetic compass as the

gyro precesses.

Due to gyroscopic precession,

directional gyros must be periodically realigned with a

magnetic compass. Friction is the major cause of its drifting

from the correct heading.

In the Northern Hemisphere, a magnetic compass will normally

indicate a turn toward the north if

ANSWER: an aircraft is accelerated while on an east or west heading.

In the Northern Hemisphere, a

magnetic compass will normally indicate a turn toward the

north if an airplane is accelerated while on an east or west

heading.

During flight, when are the indications of a magnetic compass

accurate?

ANSWER: Only in straight-and-level unaccelerated flight.

During flight, the magnetic compass

indications can be considered accurate only when in

straight-and-level, unaccelerated flight. During acceleration,

deceleration, or turns, the compass card will dip and cause

false readings.

Deviation in a magnetic compass is caused by the

ANSWER: magnetic fields within the aircraft distorting the lines of

magnetic force.

Magnetic fields produced by metals

and electrical accessories in the airplane disturb the

compass needle and produce errors. These errors are

referred to as compass deviation.

In the Northern Hemisphere, if an aircraft is accelerated or

decelerated, the magnetic compass will normally indicate

ANSWER: correctly when on a north or south heading.

Acceleration and deceleration errors

on magnetic compasses do not occur when on a north or

south heading in the Northern Hemisphere. They occur on

east and west headings.

In the Northern Hemisphere, a magnetic compass will normally

indicate initially a turn toward the west if

ANSWER: a right turn is entered from a north heading.

Due to the northerly turn error in the

Northern Hemisphere, a magnetic compass will initially

indicate a turn toward the west if a right (east) turn is

entered from a north heading.

In the Northern Hemisphere, the magnetic compass will normally

indicate a turn toward the south when

ANSWER: the aircraft is decelerated while on a west heading.

In the Northern Hemisphere, a

magnetic compass will normally indicate a turn toward the

south if an airplane is decelerated while on an east or west

heading.

In the Northern Hemisphere, a magnetic compass will normally

indicate initially a turn toward the east if

ANSWER: a left turn is entered from a north heading.

In the Northern Hemisphere, a

magnetic compass normally initially indicates a turn toward

the east if a left (west) turn is entered from a north heading.

An airplane has been loaded in such a manner that the CG is

located aft of the aft CG limit. One undesirable flight characteristic a

pilot might experience with this airplane would be

ANSWER: difficulty in recovering from a stalled condition.

The recovery from a stall in any

airplane becomes progressively more difficult as its center of

gravity moves backward. Generally, airplanes become less

controllable, especially at slow flight speeds, as the center of

gravity is moved backward.

Loading an airplane to the most aft CG will cause the airplane to be

ANSWER: less stable at all speeds.

Airplanes become less stable at all

speeds as the center of gravity is moved backward. The

rearward center of gravity limit is determined largely by

considerations of stability.

Which items are included in the empty weight of an aircraft?

ANSWER: Unusable fuel and undrainable oil.

The empty weight of an airplane

includes airframe, engines, and all items of operating

equipment that have fixed locations and are permanently

installed. It includes optional and special equipment, fixed

ballast, hydraulic fluid, unusable fuel, and undrainable oil.

An aircraft is loaded 110 pounds over maximum certificated gross

weight. If fuel (gasoline) is drained to bring the aircraft weight

within limits, how much fuel should be drained?

ANSWER: 18.4 gallons.

Fuel weighs 6 lb./gal. If an airplane is

110 lb. over maximum gross weight, 18.4 gal. (110 lb./6) must

be drained to bring the airplane weight within limits.

If an aircraft is loaded 90 pounds over maximum certificated gross

weight and fuel (gasoline) is drained to bring the aircraft weight

within limits, how much fuel should be drained?

ANSWER: 15 gallons.

Since fuel weighs 6 lb./gal., draining

15 gal. (90 lb./6) will reduce the weight of an airplane that is

90 lb. over maximum gross weight to the acceptable amount.

GIVEN:

WEIGHT        ARM         MOMENT

(LB)         (IN)       (LB-IN)

Empty weight            1,495.0      101.4      151,593.0

Pilot and passengers      380.0       64.0         ---

Fuel (30 gal

usable no reserve)      ---         96.0         ---

The CG is located how far aft of datum?

ANSWER: CG 94.01.

To compute the CG you must first

multiply each weight by the arm to get the moment. Note

that the fuel is given as 30 gal. To get the weight multiply

the 30 by 6 lb. per gal. (30 x 6) = 180 lb.

Weight       Arm       Moment

(lb.)      (in.)     (lb.-in.)

Empty weight           1,495.0     101.4     151,593.0

Pilot and passengers     380.0      64.0      24,320.0

Fuel (30 x 6)            180.0      96.0      17,280.0

2,055.0               193,193.0

Now add the weights and moments. To get CG, you divide

total moment by total weight (193,193 � 2,055.0) = a CG of

94.01 in.

Figure 35

(Refer to figure 35.) What is the maximum amount of baggage that

may be loaded aboard the airplane for the CG to remain within the

moment envelope?

WEIGHT (LB)     MOM/1000

Empty weight                   1,350          51.5

Pilot and front passenger        250           ---

Rear passengers                  400           ---

Baggage                          ---           ---

Fuel, 30 gal.                    ---           ---

Oil, 8 qt.                       ---          -0.2

ANSWER: 105 pounds.

To compute the amount of weight left

for baggage, compute each individual moment by using the

loading graph and add them up. First, compute the moment

for the pilot and front seat passenger with a weight of 250 lb.

Refer to the loading graph and the vertical scale at the left

side and find the value of 250. From this position, move to

the right horizontally across the graph until you intersect the

diagonal line that represents pilot and front passenger. From

this point, move vertically down to the bottom scale, which

indicates a moment of about 9.2.

To compute rear passenger moment, measure up the vertical

scale of the loading graph to a value of 400, horizontally

across to intersect the rear passenger diagonal line, and

down vertically to the moment scale, which indicates

approximately 29.0.

To compute the moment of the fuel, you must recall that fuel

weighs 6 lb. per gal. The question gives 30 gal., for a total

fuel weight of 180 lb. Now move up the weight scale on the

loading graph to 180, then horizontally across to intersect

the diagonal line that represents fuel, then vertically down to

the moment scale, which indicates approximately 8.7.

To get the weight of the oil, see Note (2) at the bottom of the

loading graph section of Fig. 35. It gives 15 lb. as the weight

with a moment of -.2.

Now total the weights (2,195 lb. including 15 lb. of engine

oil). Also total the moments (98.2 including engine oil with a

negative 0.2 moment).

With this information, refer to the center of gravity moment

envelope chart. Note that the maximum weight in the

envelope is 2,300 lb. 2,300 lb. - 2,195 lb. already totaled

leaves a maximum possible 105 lb. for baggage. However,

you must be sure 105 lb. of baggage does not exceed the 109

moments allowed at the top of the envelope. On the loading

graph, 105 lb. of baggage indicates approximately 10

moments.

Thus, a total of 108.2 moments (98.2 + 10) is within the 109

moments allowed on the envelope for 2,300 lb. of weight.

Therefore, baggage of 105 lb. can be loaded.

Moment/1000

Weight        lb.-in.

Empty weight                      1,350          51.5

Pilot and front seat passenger      250           9.2

Rear passengers                     400          29.0

Baggage                              ?              ?

Fuel (30 gal. x 6 lb./gal.)         180           8.7

Oil                                  15          -0.2

2,195          98.2

(without baggage)

Figure 35

(Refer to figure 35.) Calculate the moment of the airplane and

determine which category is applicable.

WEIGHT (LB)     MOM/1000

Empty weight                   1,350          51.5

Pilot and front passenger        310           ---

Rear passengers                   96           ---

Baggage                          ---           ---

Fuel, 38 gal.                    ---           ---

Oil, 8 qt.                       ---          -0.2

ANSWER: 80.8, utility category.

First, total the weight and get 1,999 lb.

Note that the 38 gal. of fuel weighs 228 lb. (38 gal. x 6 lb./gal.).

Find the moments for the pilot and front seat passengers,

rear passengers, and fuel by using the loading graph in Fig.

35. Find the oil weight and moment by consulting Note (2)

on Fig. 35. It is 15 lb. and -0.2 moments. Total the moments

as shown in the schedule below.

Now refer to the center of gravity moment envelope. Find

the gross weight of 1,999 on the vertical scale, and move

horizontally across the chart until intersecting the vertical

line that represents the 80.8 moment. Note that a moment of

80.8 lb.-in. falls into the utility category envelope.

Moment/1000

Weight      lb.-in.

Empty weight                      1,350         51.5

Pilot and front seat passenger      310         11.5

Rear passengers                      96          7.0

Fuel (38 gal. x 6 lb./gal.)         228         11.0

Oil                                  15         -0.2

1,999         80.8

Figure 35

(Refer to figure 35.) What is the maximum amount of fuel that may

be aboard the airplane on takeoff if loaded as follows?

WEIGHT (LB)     MOM/1000

Empty weight                   1,350          51.5

Pilot and front passenger        340           ---

Rear passengers                  310           ---

Baggage                           45           ---

Oil, 8 qt.                       ---           ---

ANSWER: 40 gallons.

To find the maximum amount of fuel

this airplane can carry, add the empty weight (1,350), pilot

and front passenger weight (340), rear passengers (310),

baggage (45), and oil (15), for a total of 2,060 lb. (Find the oil

weight and moment by consulting Note (2) on Fig. 35. It is 15

lb. and -0.2 moments.) Gross weight maximum on the center

of gravity moment envelope chart is 2,300. Thus, 240 lb. of

weight (2,300 - 2,060) is available for fuel. Since each gallon

of fuel weighs 6 lb., this airplane can carry 40 gal. of fuel

(240/6 lb. per gal.) if its center of gravity moments do not

exceed the limit. Note that long-range tanks were not

mentioned; assume they exist.

Compute the moments for each item. The empty weight

moment is given as 51.5. Calculate the moment for the pilot

and front passenger as 12.5, the rear passengers as 22.5, the

fuel as 11.5, the baggage as 4.0, and the oil as -0.2. These

total to 101.8, which is within the envelope, so 40 gal. of fuel

may be carried.

Moment/1000

Weight      lb.-in.

Empty weight                      1,350         51.5

Pilot and front seat passenger      340         12.5

Rear passengers                     310         22.5

Baggage                              45          4.0

Fuel (40 gal. x 6 lb./gal.)         240         11.5

Oil                                  15         -0.2

2,300        101.8

Figure 35

(Refer to figure 35.) Determine the moment with the following data:

WEIGHT (LB)     MOM/1000

Empty weight                   1,350          51.5

Pilot and front passenger        340           ---

Fuel (std tanks)             Capacity          ---

Oil, 8 qt.                       ---           ---

ANSWER: 74.9 pound-inches.

To find the CG moment/1000, find the

moments for each item and total the moments as shown in

the schedule below. For the fuel, the loading graph shows

the maximum as 38 gal. for standard tanks (38 gal. x 6 lb. =

228 lb.). (Find the oil weight and moment by consulting Note

(2) on Fig. 35; it is 15 lb. and -0.2. moments.) These total 74.9,

so this answer is correct.

Moment/1000

Weight      lb.-in.

Empty weight                      1,350         51.5

Pilot and front seat passenger      340         12.6

Fuel                                228         11.0

Oil                                  15         -0.2

1,933         74.9

Figure 35

(Refer to figure 35.) Determine the aircraft loaded moment and the

aircraft category.

WEIGHT (LB)     MOM/1000

Empty weight                   1,350          51.5

Pilot and front passenger        380           ---

Fuel, 48 gal                     288           ---

Oil, 8 qt.                       ---           ---

ANSWER: 79.2, normal category.

The moments for the pilot, front

passenger, fuel, and oil must be found on the loading graph

in Fig. 35. Total all the moments and the weight as shown in

the schedule below.

Now refer to the center of gravity moment envelope graph.

Find the gross weight of 2,033 on the vertical scale, and

move horizontally across the graph until intersecting the

vertical line that represents the 79.2 moment. A moment of

79.2 lb.-in. falls into the normal category envelope.

Moment/1000

Weight      lb.-in.

Empty weight                      1,350         51.5

Pilot and front seat passenger      380         14.2

Fuel (capacity)                     288         13.7

Oil                                  15         -0.2

2,033         79.2

Figure 33 Figure 34

(Refer to figures 33 and 34.) Determine if the airplane weight and

balance is within limits.

Front seat occupants

. . . . . . . . . . . . . . . . . . . . . . . . . . 340 lb

Rear seat occupants

. . . . . . . . . . . . . . . . . . . . . . . . . . 295 lb

Fuel (main wing tanks)

. . . . . . . . . . . . . . . . . . . . . . . . . . 44 gal

Baggage

. . . . . . . . . . . . . . . . . . . . . . . . . . 56 lb

ANSWER: 20 pounds overweight, CG within limits.

Both the total weight and the total

moment must be calculated. As in most weight and balance

problems, you should begin by setting up a schedule as

below. Note that the empty weight in Fig. 33 is given as

2,015 with a moment/100 in. of 1,554 (note the change to

moment/100 on this chart), and that empty weight includes

the oil.

The next step is to compute the moment/100 for each item.

The front seat occupants' moment/100 is 289 (340 x 85 � 100).

The rear seat occupants' moment/100 is 357 (295 x 121 � 100).

The fuel (main tanks) weight of 264 lb. and moment/100 of

198 is read directly from the table. The baggage moment/100

is 78 (56 x 140 � 100).

The last step is to go to the Moment Limits versus Weight

chart (Fig. 34), and note that the maximum weight allowed is

2,950, which means that the plane is 20 lb. over. At a

moment/100 of 2,476, the plane is within the CG limits

because the moments/100 may be from 2,422 to 2,499 at 2,950 lb.

Moment/1000

Weight       lb.-in.

Empty weight w/oil                2,015         1,554

Front seat                          340           289

Rear seat                           295           357

Fuel (44 gal. x 6 lb/gal)           264           198

Oil                                  56            78

2,970         2,476

Figure 33 Figure 34

(Refer to figures 33 and 34.) Calculate the weight and balance and

determine if the CG and the weight of the airplane are within limits.

Front seat occupants

. . . . . . . . . . . . . . . . . . . . . . . . . . 350 lb

Rear seat occupants

. . . . . . . . . . . . . . . . . . . . . . . . . . 325 lb

Baggage

. . . . . . . . . . . . . . . . . . . . . . . . . . 27 lb

Fuel

. . . . . . . . . . . . . . . . . . . . . . . . . . 35 gal

ANSWER: CG 83.4, within limits.

Total weight, total moment, and CG

must all be calculated. As in most weight and balance

problems, you should begin by setting up the schedule as

shown below.

Next, go to the Moment Limits vs. Weight chart (Fig. 34),

and note that the maximum weight allowed is 2,950, which

means that this airplane is 23 lb. under maximum weight. At a

total moment of 2,441, it is also within the CG limits (2,399 to

2,483) at that weight.

Finally, compute the CG. Recall that Fig. 33 gives moment

per 100 in. The total moment is therefore 244,100 (2,441 x

100). The CG is 244,100/2,927 = 83.4.

Moment/1000

Weight       lb.-in.

Empty weight w/oil                2,015         1,554

Front seat                          350           298

Rear seat                           325           393

Fuel, main (35 gal.)                210           158

Baggage                              27            38

2,927         2,441

Figure 33 Figure 34

(Refer to figures 33 and 34.) What is the maximum amount of

baggage that can be carried when the airplane is loaded as follows?

Front seat occupants

. . . . . . . . . . . . . . . . . . . . . . . . . . 387 lb

Rear seat occupants

. . . . . . . . . . . . . . . . . . . . . . . . . . 293 lb

Fuel

. . . . . . . . . . . . . . . . . . . . . . . . . . 35 gal

ANSWER: 45 pounds.

The maximum allowable weight on the

Moment Limits vs. Weight chart (Fig. 34) is 2,950 lb. The

total of the given weights is 2,905 lb. (including the empty

weight of the airplane at 2,015 lb. and the fuel at 6 lb./gal.),

so baggage cannot weigh more than 45 lb.

It is still necessary to compute total moments to verify that

the position of these weights does not move the CG out of

CG limits.

The total moment of 2,460 lies safely between the moment

limits of 2,422 and 2,499 on Fig. 34, at the maximum weight,

so this airplane can carry as much as 45 lb. of baggage when

loaded in this manner.

Moment/1000

Weight       lb.-in.

Empty weight w/oil                2,015         1,554

Front seat                          387           330

Rear seat                           293           355

Fuel, main (35 gal.)                210           158

Baggage                              45            63

2,950         2,460

Figure 33 Figure 34

(Refer to figures 33 and 34.) Determine if the airplane weight and

balance is within limits.

Front seat occupants

. . . . . . . . . . . . . . . . . . . . . . . . . . 415 lb

Rear seat occupants

. . . . . . . . . . . . . . . . . . . . . . . . . . 110 lb

Fuel, main tanks

. . . . . . . . . . . . . . . . . . . . . . . . . . 44 gal

Fuel, aux. tanks

. . . . . . . . . . . . . . . . . . . . . . . . . . 19 gal

Baggage

. . . . . . . . . . . . . . . . . . . . . . . . . . 32 lb

ANSWER: Weight within limits, CG out of limits.

Both the weight and the total moment

must be calculated. Begin by setting up the schedule shown

below. The fuel must be separated into main and auxiliary

tanks, but weights and moments for both tanks are provided

in Fig. 33.

Since 415 lb. is not shown on the front seat table, simply

multiply the weight by the arm shown at the top of the table

(415 lb. x 85 in. = 35,275 lb.-in.) and divide by 100 for

moment/100 of 353 (35,275 � 100 = 352.75). The rear seat

moment must also be multiplied (110 lb. x 121 in. = 13,310

lb.-in.). Divide by 100 to get 133.1, or 133 lb.-in./100. The last

step is to go to the Moment Limits vs. Weight chart (Fig.

34). The maximum weight allowed is 2,950, which means that

the airplane weight is within the limits. However, the CG is

out of limits because the minimum moment/100 for a weight

of 2,950 lb. is 2,422.

Moment/1000

Weight       lb.-in.

Empty weight w/oil                2,015         1,554

Front seat                          415           353

Rear seat                           110           133

Fuel, main                          264           198

Fuel, aux.                          114           107

Baggage                              32            45

2,950         2,390

Figure 33 Figure 34

(Refer to figures 33 and 34.) Which action can adjust the airplane's

weight to maximum gross weight and the CG within limits for

takeoff?

Front seat occupants

. . . . . . . . . . . . . . . . . . . . . . . . . . 425 lb

Rear seat occupants

. . . . . . . . . . . . . . . . . . . . . . . . . . 300 lb

Fuel, main tanks

. . . . . . . . . . . . . . . . . . . . . . . . . . 44 gal

ANSWER: Drain 9 gallons of fuel.

First, determine the total weight to see

how much must be reduced. As shown below, this original

weight is 3,004 lb. Fig. 34 shows the maximum weight as

2,950 lb. Thus, you must adjust the total weight by removing

54 lb. (3,004 - 2,950). Since fuel weighs 6 lb./gal., you must

drain at least 9 gal. To check for CG, recompute the total

moment using a new fuel moment of 158 (from the chart) for

210 lb. The plane now weighs 2,950 lb. with a total moment

of 2,437, which falls within the moment limits on Fig. 34.

Original      Adjusted    Moment/100

Weight        Weight       lb.-in.

Empty weight with oil      2,015         2,015         1,554

Front seat                   425           425           362

Rear seat                    300           300           363

Fuel                         264           210           158

3,004         2,950         2,437

Figure 33 Figure 34

(Refer to figures 33 and 34.) With the airplane loaded as follows,

what action can be taken to balance the airplane?

Front seat occupants

. . . . . . . . . . . . . . . . . . . . . . . . . . 411 lb

Rear seat occupants

. . . . . . . . . . . . . . . . . . . . . . . . . . 100 lb

Main wing tanks

. . . . . . . . . . . . . . . . . . . . . . . . . . 44 gal

ANSWER: Add a 100-pound weight to the baggage compartment.

You need to calculate the weight and

moment. The weight of the empty plane including oil is

2,015, with a moment of 1,554. The 411 lb. in the front seats

has a total moment of 349.35 [411 x 85 (ARM) = 34,935/100 =

349.35]. The rear seat occupants have a weight of 100 lb. and

a moment of 121.0 [100 x 121 (ARM) = 12,100/100 = 121.0].

The fuel weight is given on the chart as 264 lb. with a

moment of 198.

Moment/100

Weight        lb.-in.

Empty weight w/oil                2,015         1,554

Front seat                          411           349.35

Rear seat                           100           121.0

Fuel                                264           198.0

2,790         2,222.35

On the Fig. 34 chart, the minimum moment for 2,790 lb. is

2,243. Thus, the CG of 2,222.35 is forward. Evaluate A, B, and

C to see which puts the CG within limits.

Weight     Moment/100

A                +114         +107

B                +100         +140

C                 +60          +56

-60          -45

0          +11

At 2,890 lb. (2,790 + 100), moment/100 of 2,362.35 (2,222.35 +

140) is over the minimum moment/100 of 2,354.

Figure 33 Figure 34

(Refer to figures 33 and 34.) Upon landing, the front passenger (180

pounds) departs the airplane. A rear passenger (204 pounds)

moves to the front passenger position. What effect does this have

on the CG if the airplane weighed 2,690 pounds and the MOM/100

was 2,260 just prior to the passenger transfer?

ANSWER: The CG moves forward approximately 3 inches.

The requirement is the effect of a

change in loading. Look at Fig. 33 for occupants. Losing the

180-lb. passenger from the front seat reduces the MOM/100

by 153. Moving the 204-lb. passenger from the rear seat to

the front reduces the MOM/100 by about 74 (247 - 173). The

total moment reduction is thus about 227 (153 + 74). As

calculated below, the CG moves forward from 84.01 to 81.00

in.

Old CG = 226,000 lb.-in.

2,690 lb. = 84.01 in.

New CG = 203,300 lb.-in.

2,510 lb. = 81.00 in.

Figure 33 Figure 34

(Refer to figures 33 and 34.) What effect does a 35-gallon fuel burn

(main tanks) have on the weight and balance if the airplane

weighed 2,890 pounds and the MOM/100 was 2,452 at takeoff?

ANSWER: Weight is reduced by 210 pounds and the CG is aft of

limits.

The effect of a 35-gal. fuel burn on

weight balance is required. Burning 35 gal. of fuel will reduce

weight by 210 lb. and moment by 158. At 2,680 lb. (2,890 -

210), the 2,294 MOM/100 (2,452 - 158) is above the maximum

moment of 2,287; i.e., CG is aft of limits. This is why weight

and balance should always be computed for the beginning

and end of each flight.

What is ground effect?

ANSWER: The result of the interference of the surface of the Earth

with the airflow patterns about an airplane.

Ground effect is due to the

interference of the ground (or water) surface with the airflow

patterns about the airplane in flight. As the wing encounters

ground effect, there is a reduction in the upwash,

downwash, and the wingtip vortices. The result is a

reduction in induced drag. Thus, for a given angle of attack,

the wing will produce more lift in ground effect than it does

out of ground effect.

Floating caused by the phenomenon of ground effect will be most

realized during an approach to land when at

ANSWER: less than the length of the wingspan above the surface.

Ground effect is most usually

recognized when the airplane is within one-half of the length

of its wingspan above the surface. It may extend as high as a

full wingspan length above the surface. Due to an alteration

of the airflow about the wings, induced drag decreases,

which reduces the thrust required at low airspeeds. Thus,

any excess speed during the landing flare may result in

considerable floating.

What must a pilot be aware of as a result of ground effect?

ANSWER: Induced drag decreases; therefore, any excess speed at

the point of flare may cause considerable floating.

Ground effect reduces the upwash,

downwash, and vortices caused by the wings, resulting in a

decrease in induced drag. Thus, thrust required at low

airspeeds will be reduced and any excess speed at the point

of flare may cause considerable floating.

Ground effect is most likely to result in which problem?

ANSWER: Becoming airborne before reaching recommended takeoff

speed.

Due to the reduction of induced drag

in ground effect, the airplane may seem capable of becoming

airborne well below the recommended takeoff speed.

However, as the airplane rises out of ground effect (a height

greater than the wingspan) with a deficiency of speed, the

increase in induced drag may result in very marginal initial

climb performance. In extreme cases, the airplane may

become airborne initially, with a deficiency of airspeed, only

to settle back on the runway when attempting to fly out of

the ground effect area.

What effect, if any, does high humidity have on aircraft

performance?

ANSWER: It decreases performance.

As the air becomes more humid, it

becomes less dense. This is because a given volume of

moist air weighs less than the same volume of dry air. Less

dense air reduces aircraft performance.

What effect does high density altitude have on aircraft

performance?

ANSWER: It reduces climb performance.

High density altitude reduces all

aspects of an airplane's performance, including takeoff and

climb performance.

Which combination of atmospheric conditions will reduce aircraft

takeoff and climb performance?

ANSWER: High temperature, high relative humidity, and high

density altitude.

Takeoff and climb performance are

reduced by high density altitude. High density altitude is a

result of high temperatures and high relative humidity.

Figure 8

(Refer to figure 8.) Determine the density altitude for these

conditions:

Altimeter setting

. . . . . . . . . . . . . . . . . . . . . . .   30.35

Runway temperature

. . . . . . . . . . . . . . . . . . . . . . .   +25�F

Airport elevation

. . . . . . . . . . . . . . . . . . . . . . . 3,894 ft MSL

ANSWER: 2,000 feet MSL.

With an altimeter setting of 30.35"

Hg, 394 ft. must be subtracted from a field elevation of 3,894

to obtain a pressure altitude of 3,500 ft. Note that the

higher-than-normal pressure of 30.35 means the pressure

altitude will be less than true altitude. The 394 ft. was found

by interpolation: 30.3 on the graph is -348, and 30.4 was -440

ft. Adding one-half the -92 ft. difference (-46 ft.) to -348 ft.

results in -394 ft. Once you have found the pressure altitude,

use the chart to plot 3,500 ft. pressure altitude at 25�F, to

reach 2,000 ft. density altitude. Note that since the

temperature is lower than standard, the density altitude is

lower than the pressure altitude.

Figure 8

(Refer to figure 8.) What is the effect of a temperature increase from

30 to 50�F on the density altitude if the pressure altitude remains at

3,000 feet MSL?

ANSWER: 1,300-foot increase.

Increasing the temperature from 30�F

to 50�F, given a constant pressure altitude of 3,000 ft.,

requires you to find the 3,000-ft. line on the density altitude

chart at the 30�F level. At this point, the density altitude is

approximately 1,650 ft. Then move up the 3,000-ft. line to

50�F, where the density altitude is approximately 2,950 ft.

There is an approximate 1,300-ft. increase (2,950 - 1,650 ft.).

Note that 50�F is just about standard and pressure altitude is

very close to density altitude.

Figure 8

(Refer to figure 8.) Determine the pressure altitude at an airport that

is 3,563 feet MSL with an altimeter setting of 29.96.

ANSWER: 3,527 feet MSL.

Note that the question asks only for

pressure altitude, not density altitude. Pressure altitude is

determined by adjusting the altimeter setting to 29.92" Hg,

i.e., adjusting for nonstandard pressure. This is the true

altitude plus or minus the pressure altitude conversion

factor (based on current altimeter setting). On the chart, an

altimeter setting of 30.0 requires you to subtract 73 ft. to

determine pressure altitude (note that at 29.92, nothing is

subtracted because that is pressure altitude). Since 29.96 is

half way between 29.92 and 30.0, you need only subtract 36

(-73/2) from 3,563 ft. to obtain a pressure altitude of 3,527 ft.

(3,563 - 36). Note that a higher-than-standard barometric

pressure means pressure altitude is lower than true altitude.

Figure 8

(Refer to figure 8.) What is the effect of a temperature decrease and

a pressure altitude increase on the density altitude from 90�F and

1,250 feet pressure altitude to 55�F and 1,750 feet pressure altitude?

ANSWER: 1,700-foot decrease.

The requirement is the effect of a

temperature decrease and a pressure altitude increase on

density alti tude. First, find the density altitude at 90�F and

1,250 ft. (approximately 3,600 ft.). Then find the density

altitude at 55�F and 1,750 ft. pressure altitude (approximately

1,900 ft.). Next, subtract the two numbers. 3,600 ft. minus

1,900 ft. equals a 1,700-ft. decrease in density altitude.

Figure 8

(Refer to figure 8.) Determine the pressure altitude at an airport that

is 1,386 feet MSL with an altimeter setting of 29.97.

ANSWER: 1,341 feet MSL.

Pressure altitude is determined by

adjusting the altimeter setting to 29.92" Hg. This is the true

altitude plus or minus the pressure altitude conversion

factor (based on current altimeter setting). Since 29.97 is not

a number given on the conversion chart, you must

interpolate. Compute 5/8 of -73 (since 29.97 is 5/8 of the way

between 29.92 and 30.0), which is 45. Subtract 45 ft. from

1,386 ft. to obtain a pressure altitude of 1,341 ft. Note if the

altimeter setting is greater than standard (e.g., 29.97), the

pressure altitude (i.e., altimeter set to 29.92) will be less than

true altitude.

Figure 8

(Refer to figure 8.) What is the effect of a temperature increase from

25 to 50�F on the density altitude if the pressure altitude remains at

5,000 feet?

ANSWER: 1,650-foot increase.

Increasing the temperature from 25�F

to 50�F, given a pressure altitude of 5,000 ft., requires you to

find the 5,000-ft. line on the density altitude chart at the 25�F

level. At this point, the density altitude is approximately

3,850 ft. Then move up the 5,000-ft. line to 50�F, where the

density altitude is approximately 5,500 ft. There is about a

1,650-ft. increase (5,500 - 3,850 ft.). As temperature increases,

so does density altitude; i.e., the atmosphere becomes

thinner (less dense).

Figure 8

(Refer to figure 8.) Determine the pressure altitude with an

indicated altitude of 1,380 feet MSL with an altimeter setting of

28.22 at standard temperature.

ANSWER: 2,991 feet MSL.

Pressure altitude is determined by

adjusting the altimeter setting to 29.92" Hg, i.e., adjusting for

nonstandard pressure. This is the indicated altitude of 1,380

ft. plus or minus the pressure altitude conversion factor

(based on the current altimeter setting).

On the right side of Fig. 8 is a pressure altitude conversion

factor schedule. Add 1,533 ft. for an altimeter setting of 28.30

and 1,630 ft. for an altimeter setting of 28.20. Using

interpolation, you must subtract 20% of the difference

between 28.3 and 28.2 from 1,630 ft. (1,630 - 1,533 = 97 x .2 =

19). Then, 1,630 - 19 = 1,611 and add 1,611 ft. to 1,380 ft. to

get the pressure altitude of 2,991 ft.

Figure 8

(Refer to figure 8.) Determine the density altitude for these

conditions:

Altimeter setting

. . . . . . . . . . . . . . . . . . . . . . . . 29.25

Runway temperature

. . . . . . . . . . . . . . . . . . . . . . . . +81�F

Airport elevation

. . . . . . . . . . . . . . . . . . . . . . . . 5,250 ft MSL

ANSWER: 8,500 feet MSL.

With an altimeter setting of 29.25" Hg,

about 626 ft. (579 plus � the 94-ft. pressure altitude

conversion factor difference between 29.2 and 29.3) must be

added to the field elevation of 5,250 ft. to obtain the pressure

altitude, or 5,876 ft. Note barometric pressure is less than

standard and pressure altitude is greater than true altitude.

Next convert pressure altitude to density altitude. On the

chart, find the point at which the pressure altitude line for

5,876 ft. crosses the 81�F line. The density altitude at that

spot shows somewhere in the mid-8,000s ft. The closest

answer choice is 8,500 ft. Note that, when temperature is

higher than standard, density altitude exceeds pressure

altitude.

Figure 41

(Refer to figure 41.) Determine the total distance required for

takeoff to clear a 50-foot obstacle.

OAT

. . . . . . . . . . . . . . . . . . . . . . . . . Std

Pressure altitude

. . . . . . . . . . . . . . . . . . . . . . . . . 4,000 ft

Takeoff weight

. . . . . . . . . . . . . . . . . . . . . . . . . 2,800 lb

Headwind component

. . . . . . . . . . . . . . . . . . . . . . . . . Calm

ANSWER: 1,750 feet.

The takeoff distance to clear a 50-ft.

obstacle is required. Begin on the left side of the graph at

standard temperature (as represented by the curved line

labeled "ISA"). From the intersection of the standard

temperature line and the 4,000-ft. pressure altitude, proceed

horizon tally to the right to the first reference line, and then

move parallel to the closest guideline to 2,800 lb. From there,

proceed horizontally to the right to the third reference line

(skip the second reference line because there is no wind),

and move parallel to the closest guideline all the way to the

far right. You are at 1,750 ft., which is the takeoff distance to

clear a 50-ft. obstacle.

Figure 41

(Refer to figure 41.) Determine the approximate ground roll distance

required for takeoff.

OAT

. . . . . . . . . . . . . . . . . . . . . . . . . 100�F

Pressure altitude

. . . . . . . . . . . . . . . . . . . . . . . . . 2,000 ft

Takeoff weight

. . . . . . . . . . . . . . . . . . . . . . . . . 2,750 lb

Headwind component

. . . . . . . . . . . . . . . . . . . . . . . . . Calm

ANSWER: 1,150 feet.

Begin on the left section of Fig. 41 at

100�F (see outside air temperature at the bottom). Move up

vertically to the pressure altitude of 2,000 ft. Then proceed

horizontally to the first reference line. Since takeoff weight is

2,750, move parallel to the closest guideline to 2,750 lb. Then

proceed horizontally to the second reference line. Since the

wind is calm, proceed again horizontally to the right-hand

margin of the diagram (ignore the third reference line

because there is no obstacle, i.e., ground roll is desired),

which will be at 1,150 ft.

Figure 41

(Refer to figure 41.) Determine the total distance required for

takeoff to clear a 50-foot obstacle.

OAT

. . . . . . . . . . . . . . . . . . . . . . . . . . Std

Pressure altitude

. . . . . . . . . . . . . . . . . . . . . . . . . . Sea level

Takeoff weight

. . . . . . . . . . . . . . . . . . . . . . . . . . 2,700 lb

Headwind component

. . . . . . . . . . . . . . . . . . . . . . . . . . Calm

ANSWER: 1,400 feet.

Begin in the left section of Fig. 41 by

finding the intersection of the sea level pressure altitude and

standard temperature (59�F) and proceed horizontally to the

right to the first reference line. Then proceed parallel to the

closest guideline, to 2,700 lb. From there, proceed

horizontally to the right to the third reference line. You skip

the second reference line because the wind is calm. Then

proceed upward parallel to the closest guideline to the far

right side. To clear the 50-ft. obstacle, you need a takeoff

distance of about 1,400 ft.

Figure 41

(Refer to figure 41.) Determine the approximate ground roll distance

required for takeoff.

OAT

. . . . . . . . . . . . . . . . . . . . . . . . . . 90�F

Pressure altitude

. . . . . . . . . . . . . . . . . . . . . . . . . . 2,000 ft

Takeoff weight

. . . . . . . . . . . . . . . . . . . . . . . . . . 2,500 lb

Headwind component

. . . . . . . . . . . . . . . . . . . . . . . . . . 20 kts

ANSWER: 650 feet.

Begin with the intersection of the

2,000-ft. pressure altitude curve and 90�F in the left section

of Fig. 41. Move horizontally to the right to the first

reference line, and then parallel to the closest guideline to

2,500 lb. Then move horizontally to the right to the second

reference line, and then parallel to the closest guideline to

the right to 20 kt. Then move horizontally to the right,

directly to the right margin because there is no obstacle

clearance. You should end up at about 650 ft., which is the

required ground roll when there is no obstacle to clear.

Figure 36

(Refer to figure 36.) What fuel flow should a pilot expect at 11,000

feet on a standard day with 65 percent maximum continuous

power?

ANSWER: 11.2 gallons per hour.

Note that the entire chart applies to

65% maximum continuous power (regardless of the throttle),

so use the middle section of the chart which is labeled a

standard day.

The fuel flow at 11,000 ft. on a standard day would be 1/2 of

the way between the fuel flow at 10,000 ft. (11.5 GPH) and

the fuel flow at 12,000 ft. (10.9 GPH). Thus, the fuel flow at

11,000 ft. would be 11.5 - 0.3, or 11.2 GPH.

Figure 36

(Refer to figure 36.) What is the expected fuel consumption for a

1,000-nautical mile flight under the following conditions?

Pressure altitude

. . . . . . . . . . . . . . . . . . . . . . . . . 8,000 ft

Temperature

. . . . . . . . . . . . . . . . . . . . . . . . . 22�C

Manifold pressure

. . . . . . . . . . . . . . . . . . . . . . . . . 20.8" Hg

Wind

. . . . . . . . . . . . . . . . . . . . . . . . . Calm

ANSWER: 70.1 gallons.

To determine the fuel consumption,

you need to know the number of hours the flight will last

and the gallons per hour the airplane will use. The chart is

divided into three sections. They differ based on air

temperature. Use the right section of the chart as the

temperature at 8,000 ft. is 22�C.

At a pressure altitude of 8,000 ft., 20.8" Hg manifold

pressure, and 22�C, the fuel flow is 11.5 GPH and the true

airspeed is 164 kt. Given a calm wind, the 1,000-NM trip will

take 6.09 hr. (1,000 NM � 164 kt).

6.09 hr. x 11.5 GPH = 70.1 gal.

Figure 36

(Refer to figure 36.) What is the expected fuel consumption for a

500-nautical mile flight under the following conditions?

Pressure altitude

. . . . . . . . . . . . . . . . . . . . . . . . . 4,000 ft

Temperature

. . . . . . . . . . . . . . . . . . . . . . . . . +29�C

Manifold pressure

. . . . . . . . . . . . . . . . . . . . . . . . . 21.3" Hg

Wind

. . . . . . . . . . . . . . . . . . . . . . . . . Calm

ANSWER: 36.1 gallons.

At 4,000 ft., 21.3" Hg manifold

pressure, and 29�C (use the section on the right), the fuel

flow will be 11.5 GPH, and the true airspeed will be 159 kt.

The 500-NM trip will take 3.14 hr. (500 NM � 159 kt).

3.14 hr. x 11.5 GPH = 36.1 gal.

Figure 36

(Refer to figure 36.) Determine the approximate manifold pressure

setting with 2,450 RPM to achieve 65 percent maximum continuous

power at 6,500 feet with a temperature of 36�F higher than standard.

ANSWER: 21.0" Hg.

The part of the chart on the right is for

temperatures 36�F greater than standard. At 6,500 ft. with a

temperature of 36�F higher than standard, the required

manifold pressure change is 1/4 of the difference between

the 21.0" Hg at 6,000 ft. and the 20.8" Hg at 8,000 ft., or

slightly less than 21.0. Thus, 21.0 is the best answer given.

The manifold pressure is closer to 21.0 than 20.8.

Figure 36

(Refer to figure 36.) Approximately what true airspeed should a

pilot expect with 65 percent maximum continuous power at 9,500

feet with a temperature of 36�F below standard?

ANSWER: 183 MPH.

The left part of the chart applies to

36�F below standard. At 8,000 ft., TAS is 181 MPH. At

10,000 ft., TAS is 184 MPH. At 9,500 ft., with a temperature

36�F below standard, the expected true airspeed is 75%

above the 181 MPH at 8,000 ft. toward the 184 MPH at 10,000

ft., i.e., approximately 183 MPH.

Figure 37

(Refer to figure 37.) What is the crosswind component for a landing

on Runway 18 if the tower reports the wind as 220� at 30 knots?

ANSWER: 19 knots.

The requirement is the crosswind

component, which is found on the horizontal axis of the

graph. You are given a 30-kt. wind speed (the wind speed is

shown on the circular lines or arcs). First, calculate the angle

between the wind and the runway (220� - 180� = 40�). Next,

find the intersection of the 40� line and the 30-kt. headwind

arc. Then, proceed downward to determine a crosswind

component of 19 kt.

Note the crosswind component is on the horizontal axis and

the headwind component is on the vertical axis.

Figure 37

(Refer to figure 37.) What is the headwind component for a landing

on Runway 18 if the tower reports the wind as 220� at 30 knots?

ANSWER: 23 knots.

The headwind component is on the

vertical axis (left-hand side of the graph). Find the same

intersection as in the preceding question, i.e., the 30-kt. wind

speed arc, and the 40� angle between wind direction and

flight path (220� - 180�). Then move horizontally to the left

and read approximately 23 kt.

Figure 37

(Refer to figure 37.) Determine the maximum wind velocity for a 45�

crosswind if the maximum crosswind component for the airplane is

25 knots.

ANSWER: 35 knots.

Start on the bottom of the graph's

horizontal axis at 25 kt. and move straight upward to the 45�

angle between wind direction and flight path line (half-way

between the 40� and 50� lines). Note that you are half-way

between the 30 and 40 arc-shaped wind speed lines, which

means that the maximum wind velocity for a 45� crosswind is

35 kt. if the airplane is limited to a 25-kt. crosswind

component.

Figure 37

(Refer to figure 37.) With a reported wind of north at 20 knots,

which runway (6, 29, or 32) is acceptable for use for an airplane

with a 13-knot maximum crosswind component?

ANSWER: Runway 32.

If the wind is from the north (i.e.,

either 360� or 0�) at 20 kt., runway 32, i.e., 320�, would

provide a 40� crosswind component (360� - 320�). Given a

20-kt. wind, find the intersection between the 20-kt. arc and

the angle between wind direction and the flight path of 40�.

Dropping straight downward to the horizontal axis gives 13

kt., which is the maximum crosswind component of the

example airplane.

Figure 37

(Refer to figure 37.) What is the maximum wind velocity for a 30�

crosswind if the maximum crosswind component for the airplane is

12 knots?

ANSWER: 24 knots.

Start on the graph's horizontal axis at

12 kt. and move upward to the 30� angle between wind

direction and flight path line. Note that you are almost

half-way between the 20 and 30 arc-shaped wind speed

lines, which means that the maximum wind velocity for a 30�

crosswind is approximately 24 kt. if the airplane is limited to a

12-kt. crosswind component.

Figure 37

(Refer to figure 37.) With a reported wind of south at 20 knots,

which runway (10, 14, or 24) is appropriate for an airplane with a

13-knot maximum crosswind component?

ANSWER: Runway 14.

If the wind is from the south at 20 kt.,

runway 14, i.e., 140�, would provide a 40� crosswind

component (180� - 140�). Given a 20-kt. wind, find the

intersection between the 20-kt. arc and the angle between

wind direction and the flight path of 40�. Dropping straight

downward to the horizontal axis gives 13 kt., which is the

maximum crosswind component of the example airplane.

Figure 38

(Refer to figure 38.) Determine the total distance required to land.

OAT

. . . . . . . . . . . . . . . . . . . . . . . . . . Std

Pressure altitude

. . . . . . . . . . . . . . . . . . . . . . . . . . 10,000 ft

Weight

. . . . . . . . . . . . . . . . . . . . . . . . . . 2,400 lb

Wind component

. . . . . . . . . . . . . . . . . . . . . . . . . . Calm

Obstacle

. . . . . . . . . . . . . . . . . . . . . . . . . . 50 ft.

ANSWER: 1,925 feet.

The landing distance graphs are very

similar to the takeoff distance graphs. Begin with the

pressure altitude line of 10,000 ft. and the intersection with

the standard temperature line which begins at 20�C and

slopes up and to the left; i.e., standard temperature

decreases as pressure altitude increases. Then move

horizontally to the right to the first reference line. Proceed

parallel to the closest guideline to 2,400 lb. Proceed

horizontally to the right to the second reference line. Since

the wind is calm, proceed horizontally to the third reference

line. Given a 50-ft. obstacle, proceed parallel to the closest

guideline to the right margin to determine a distance of

approximately 1,900 ft.

Figure 38

(Refer to figure 38.) Determine the approximate total distance

required to land over a 50-ft. obstacle.

OAT

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 90�F

Pressure altitude

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 4,000 ft

Weight

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 2,800 lb

Headwind component

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 10 kts

ANSWER: 1,775 feet.

To determine the total landing

distance, begin at the left side of Fig. 38 on the 4,000-ft.

pressure altitude line at the intersection of 90�F. Proceed

horizontally to the right to the first reference line. Proceed

parallel to the closest guideline to 2,800 lb., and then straight

across to the second reference line. Since the headwind

component is 10 kt., proceed parallel to the closest headwind

guideline to the 10-kt. line. Then move directly to the right,

to the third reference line. Given a 50-ft. obstacle, proceed

parallel to the closest guideline for obstacles to find the total

distance of approximately 1,775 ft.

Figure 38

(Refer to figure 38.) Determine the total distance required to land.

OAT

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 90�F

Pressure altitude

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 3,000 ft

Weight

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 2,900 lb

Headwind component

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 10 kts

Obstacle

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 50 ft

ANSWER: 1,725 feet.

To determine the total landing

distance, begin with pressure altitude of 3,000 ft. (between

the 2,000- and 4,000-ft. lines) at its intersection with 90�F.

Proceed horizontally to the right to the first reference line,

and then parallel to the closest guideline to 2,900 lb. From

that point, proceed horizontally to the second reference line.

Since there is a headwind component of 10 kt., proceed

parallel to the closest headwind guideline down to 10 kt. and

then horizontally to the right to the third reference line.

Given a 50-ft. obstacle, proceed parallel to the closest

guideline for obstacles to find the landing distance of

approximately 1,725 ft.

Figure 38

(Refer to figure 38.) Determine the total distance required to land.

OAT

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 32�F

Pressure altitude

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 8,000 ft

Weight

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 2,600 lb

Headwind component

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 20 kts

Obstacle

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 50 ft

ANSWER: 1,400 feet.

To determine the total landing

distance, begin with the pressure altitude of 8,000 ft. at its

intersection with 32�F (0�C). Proceed horizontally to the first

reference line, and then parallel to the closest guideline to

2,600 lb. From that point, proceed horizontally to the second

reference line. Since there is a headwind component of 20 kt.,

follow parallel to the closest headwind guideline down to 20

kt., and then horizontally to the right to the third reference

line. Given a 50-ft. obstacle, proceed parallel to the closest

guideline for obstacles to find the landing distance of

approximately 1,400 ft.

Figure 39

(Refer to figure 39.) Determine the approximate landing ground roll

distance.

Pressure altitude

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . Sea level

Headwind

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 4 kts

Temperature

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . Std

ANSWER: 401 feet.

At sea level, the ground roll is 445 ft.

The standard temperature needs no adjustment. According

to Note 1 in Fig. 39, the distance should be decreased 10%

for each 4 kt. of headwind, so the headwind of 4 kt. means

that the landing distance is reduced by 10%. The result is

401 ft. (445 ft. x 90%).

Figure 39

(Refer to figure 39.) Determine the total distance required to land

over a 50-ft. obstacle.

Pressure altitude

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 3,750 ft

Headwind

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 12 kts

Temperature

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . Std

ANSWER: 816 feet.

The total distance to clear a 50-ft.

obstacle for a 3,750-ft. pressure altitude is required. Note

that this altitude lies halfway between 2,500 ft. and 5,000 ft.

Halfway between the total distance at 2,500 ft. of 1,135 ft.

and the total distance at 5,000 ft. of 1,195 ft. is 1,165 ft. Since

the headwind is 12 kt., the total distance must be reduced by

30% (10% for each 4 kt.).

70% x 1,165 = 816 ft.

Figure 39

(Refer to figure 39.) Determine the approximate landing ground roll

distance.

Pressure altitude

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 5,000 ft

Headwind

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . Calm

Temperature

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 101�F

ANSWER: 545 feet.

The ground roll distance at 5,000 ft. is

495 ft. According to Note 2 in Fig. 39, since the temperature

is 60�F above standard, the distance should be increased by

10%.

495 ft. x 110% = 545 ft.

Figure 39

(Refer to figure 39.) Determine the approximate landing ground roll

distance.

Pressure altitude

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 1,250 ft

Headwind

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 8 kts

Temperature

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . Std

ANSWER: 366 feet.

The landing ground roll at a pressure

altitude of 1,250 ft. is required. The difference between

landing distance at sea level and 2,500 ft. is 25 ft. (470 - 445).

One-half of this distance (12) plus the 445 ft. at sea level is

457 ft. The temperature is standard, requiring no adjustment.

The headwind of 8 kt. requires the distance to be decreased

by 20%. Thus, the distance required will be 366 ft. (457 x

80%).

Figure 39

(Refer to figure 39.) Determine the total distance required to land

over a 50-foot obstacle.

Pressure altitude

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 7,500 ft

Headwind

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 8 kts

Temperature

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 32�F

Runway

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . Hard surface

ANSWER: 1,004 feet.

Under normal conditions, the total

landing distance required to clear a 50-ft. obstacle is 1,255 ft.

The temperature is standard (32�F), requiring no adjustment.

The headwind of 8 kt. reduces the 1,255 by 20% (10% for

each 4 kt.). Thus, the total distance required will be 1,004 ft.

(1,255 x 80%).

Figure 39

(Refer to figure 39.) Determine the total distance required to land

over a 50-foot obstacle.

Pressure altitude

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 5,000 ft

Headwind

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 8 kts

Temperature

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 41�F

Runway

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . Hard surface

ANSWER: 956 feet.

Under standard conditions, the

distance to land over a 50-ft. obstacle at 5,000 ft. is 1,195 ft.

The temperature is standard, requiring no adjustment. The

headwind of 8 kt., however, requires that the distance be

decreased by 20% (10% for each 4 kt. headwind). Thus, the

landing ground roll will be 956 ft. (80% of 1,195).

What should pilots state initially when telephoning a weather

briefing facility for preflight weather information?

ANSWER: Identify yourself as a pilot.

When telephoning for a weather

briefing, you should identify yourself as a pilot so the

person can give you an aviation-oriented briefing. Many

nonpilots call weather briefing facilities to get the weather

for other activities.

What should pilots state initially when telephoning a weather

briefing facility for preflight weather information?

ANSWER: The intended route of flight and destination.

By telling the briefer your intended

route and destination, the briefer will be able to provide you

a more relevant briefing.

When telephoning a weather briefing facility for preflight weather

information, pilots should state

ANSWER: whether they intend to fly VFR only.

When telephoning for a weather

briefing, one should identify oneself as a pilot, the route,

destination, type of airplane, and whether one intends to fly

VFR or IFR to permit the weather briefer to give you the

most complete briefing.

Figure 21

(Refer to figure 21, area 3.) Determine the approximate latitude and

longitude of Currituck County Airport.

ANSWER: 36�24'N - 76�01'W.

On Fig. 21, find the Currituck County

Airport, which is northeast of area 3. Note that the airport

symbol is just to the west of 76� longitude (find 76� just

north of Virginia Beach and at the bottom of the chart).

There are 60 min. between the 76� and 77� lines of longitude,

with each tick mark depicting 1 min. The airport is one tick to

the west of the 76� line, or 76�01'W.

The latitude is below the 30-min. latitude line across the

center of the chart. See the numbered latitude lines at the top

(37�) and bottom (36�) of the chart. Since each tick mark

represents 1 min. of latitude, and the airport is approximately

six ticks south of the 36�30'N latitude, the airport is at

36�24'N latitude. Thus, Currituck County Airport is located

at approximately 36�24'N - 76�01'W.

Figure 22

(Refer to figure 22, area 2.) Which airport is located at

approximately 47�39'30"N latitude and 100�53'00"W longitude?

ANSWER: Crooked Lake.

On Fig. 22, you are asked to locate an

airport at 47�39'30"N latitude and 100�53' longitude. Note

that the 101� longitude line runs down the middle of the

page. Accordingly, the airport you are seeking is 7 min. to

the east of that line.

Each crossline is 1 min. on the latitude and longitude lines.

The 48� latitude line is approximately two-thirds of the way

up the chart. The 47�30N latitude line is about one-fourth of

the way up. One-third up from 47�30' to 48� latitude would be

47�39'. At this spot is Crooked Lake Airport.

Figure 22

(Refer to figure 22, area 3.) Which airport is located at

approximately 47�21'N latitude and 101�01'W longitude?

ANSWER: Washburn.

See Fig. 22. Find the 48� line of

latitude (2/3 up the page). Start at the 47�30' line of latitude

(the line below the 48� line) and count down nine ticks to the

47�21' mark and draw a horizontal line on the chart. Next find

the 101� line of longitude and go left one tick and draw a

vertical line. The closest airport is Washburn.

Figure 23

(Refer to figure 23, area 3.) Determine the approximate latitude and

longitude of Shoshone County Airport.

ANSWER: 47�33'N - 116�11'W.

See Fig. 23, just below 3. Shoshone

County Airport is just west of the 116� line of longitude (find

116� in the 8,000 MSL northwest of Shoshone). There are 60

min. between the 116� line and the 117� line. These are

depicted in 1-min. ticks. Shoshone is 11 ticks or 11 min. past

the 116� line.

Note that the 48 line of latitude is labeled. Find 48 just

northeast of the 116�. The latitude and longitude lines are

presented each 30 min. Since lines of latitude are also

divided into 1-min. ticks the airport is three ticks above the

47�30' line or 47�33'. The correct latitude and longitude is

thus 47�33'N - 116�11'W.

Figure 27

(Refer to figure 27, area 2.) What is the approximate latitude and

longitude of Cooperstown Airport?

ANSWER: 47�25'N - 98�06'W.

First locate the Cooperstown Airport

on Fig. 27. It is just above 2, middle right of chart. Note that

it is to the left (west) of the 98� line of longitude. The line of

longitude on the left side of the chart is 99�. Thus, the

longitude is a little bit more than 98�, but not near 99�.

With respect to latitude, note that Cooperstown Airport is

just below a line of latitude that is not marked in terms of

degrees. However, the next line of latitude below is 47� (see

the left side of the chart, northwest of the Jamestown

Airport). As with longitude, there are two lines of latitude for

every degree of latitude; i.e., each line is 30 min. Thus,

latitude of the Cooperstown Airport is almost 47�30', but not

quite. Accordingly, Cooperstown

Airport's latitude is 47�25'N and longitude is 98�06'W.

Figure 28

(Refer to figure 28.) An aircraft departs an airport in the eastern

daylight time zone at 0945 EDT for a 2-hour flight to an airport

located in the central daylight time zone. The landing should be at

what coordinated universal time?

ANSWER: 1545Z.

First convert the departure time to

coordinated universal time (Z) by using the time conversion

table in Fig. 28. To convert from eastern daylight time (EDT),

add 4 hr. to get 1345Z (0945 + 4 hr). A 2-hr. flight would have

you arriving at your destination airport at 1545Z.

Figure 28

(Refer to figure 28.) An aircraft departs an airport in the central

standard time zone at 0930 CST for a 2-hour flight to an airport

located in the mountain standard time zone. The landing should be

at what time?

ANSWER: 1030 MST.

Flying from the Central Standard Time

Zone to the Mountain Standard Time Zone results in a 1-hr.

gain due to time zone changes. A 2-hr. flight leaving at 0930

CST will arrive in the Mountain Standard Time Zone at 1130

CST, which is 1030 MST.

Figure 28

(Refer to figure 28.) An aircraft departs an airport in the central

standard time zone at 0845 CST for a 2-hour flight to an airport

located in the mountain standard time zone. The landing should be

at what coordinated universal time?

ANSWER: 1645Z.

First convert the departure time to

coordinated universal time (Z) by using the time conversion

table in Fig. 28. To convert from CST to Z, you must add 6

hr., thus 0845 CST is 1445Z (0845 + 6 hr.). A 2-hr. flight

would make the estimated landing time at 1645Z (1445 + 2

hr.).

Figure 28

(Refer to figure 28.) An aircraft departs an airport in the mountain

standard time zone at 1615 MST for a 2-hour 15-minute flight to an

airport located in the Pacific standard time zone. The estimated time

of arrival at the destination airport should be

ANSWER: 1730 PST.

Departing the Mountain Standard

Time Zone at 1615 MST for a 2-hr. 15-min. flight would result

in arrival in the Pacific Standard Time Zone at 1830 MST.

Because there is a 1-hr. difference between Mountain

Standard Time and Pacific Standard Time, 1 hr. must be

subtracted from the 1830 MST arrival to determine the 1730 PST arrival.

Figure 28

(Refer to figure 28.) An aircraft departs an airport in the Pacific

standard time zone at 1030 PST for a 4-hour flight to an airport

located in the central standard time zone. The landing should be at

what coordinated universal time?

ANSWER: 2230Z.

First, convert the departure time to

coordinated universal time (Z) by using the time conversion

table in Fig. 28. To convert from PST to Z, you must add 8

hr., thus 1030 PST is 1830Z (1030 + 8 hr.). A 4-hr. flight

would make the proposed landing time at 2230Z (1830 + 4

hr.).

Figure 28

(Refer to figure 28.) An aircraft departs an airport in the mountain

standard time zone at 1515 MST for a 2-hour 30-minute flight to an

airport located in the Pacific standard time zone. What is the

estimated time of arrival at the destination airport?

ANSWER: 1645 PST.

Departing the Mountain Standard

Time (MST) Zone at 1515 MST for a 2-hr. 30-min. flight

would result in arrival in the Pacific Standard Time (PST)

Zone at 1745 MST. Because there is a 1-hr. difference

between MST and PST, 1 hr. must be subtracted from the

1745 MST arrival to determine the 1645 PST estimated time

of arrival at the destination airport.

Figure 21

(Refer to figure 21.) En route to First Flight Airport (area 5), your

flight passes over Hampton Roads Airport (area 2) at 1456 and then

over Chesapeake Municipal at 1501. At what time should your

flight arrive at First Flight?

ANSWER: 1526.

The distance between Hampton

Roads Airport (about 2 in. north of 2) and Chesapeake

Municipal (northeast of 2 on Fig. 21) is 10 NM. It took 5 min.

(1501 - 1456) to go 10 NM, so the airplane is traveling at 2

NM per minute. The distance from Chesapeake Municipal to

First Flight (right of 5) is 50 NM. At 2 NM per minute, it will

take 25 min. 25 min. added to the time you passed

Chesapeake Municipal (1501) is 1526.

Note: There is a discrepancy between this question and the

figure. "Chesapeake Municipal" is labeled "Chesapeake

Regional" on the chart.

Figure 24

(Refer to figure 24.) While en route on Victor 185, a flight crosses

the 248� radial of Allendale VOR at 0953 and then crosses the 216�

radial of Allendale VOR at 1000. What is the estimated time of

arrival at Savannah VORTAC?

ANSWER: 1028.

The first step is to find the three

points involved. V185 runs southeast from the top left of

Fig. 24. The first intersection (V70 and V185) is about 1 in.

from the top of the chart. The second intersection (V157 and

V185) is about 1� in. farther along V185. The Savannah

VORTAC is about 6 in. farther down V185.

Use the sectional scale 1:500,000. From the first intersection

(V70 and V185), it is about 10 NM to the intersection of V185

and V157. From there it is 40 NM to Savannah VORTAC.

On your flight computer, place the 7 min. the first leg took

(1000 - 0953) on the inner scale under 10 NM on the outer

scale. Then find 40 NM on the outer scale. Read 28 min. on

the inner scale, which is the time en route from the V185 and

V157 intersection to the Savannah VORTAC. Arrival time

over Savannah VORTAC is therefore 1028.

Figure 24

(Refer to figure 24.) What is the approximate position of the aircraft

if the VOR receivers indicate the 320� radial of Savannah VORTAC

(area 3) and the 184� radial of Allendale VOR (area 1)?

ANSWER: Town of Springfield.

To locate a position based on VOR

radials, draw the radials on your map or on the plastic

overlay during the FAA pilot knowledge test. Remember

that radials are from the VOR, or leaving the VOR. On Fig. 24,

the 320� radial from Savannah extends northwest, and the

184� radial from Allendale extends south. They intersect

over the town of Springfield.

Figure 27

(Refer to figure 27.) Determine the magnetic course from

Breckheimer (Pvt) Airport (area 1) to Jamestown Airport (area 4).

ANSWER: 180�.

On Fig. 27, you are to find the

magnetic course from Breckheimer Airport (top center) to

Jamestown Airport (below 4). Since Jamestown has a VOR

on the field, a compass rose exists around the Jamestown

Airport symbol on the chart. Compass roses are based on

magnetic courses. Thus, a straight line from Jamestown

Airport to Breckheimer Airport coincides with the compass

rose at 359�. Since the route is south to Jamestown, not

north from Jamestown, compute the reciprocal direction as

179� (359� - 180�). The course, then, is approximately 180�.

Figure 21

(Refer to figure 21.) Determine the magnetic course from First Flight

Airport (area 5) to Hampton Roads Airport (area 2).

ANSWER: 331�.

You are to find the magnetic course

from First Flight Airport (lower right corner) to Hampton

Roads Airport (above 2 on Fig. 21). True course is the

degrees clockwise from true north. Determine the true course

by placing the straight edge of your plotter along the given

route with the grommet at the intersection of your route and

a meridian (the north/south line with crosslines). Here, TC is

321�. To convert this to a magnetic course, add the 10�

westerly variation (indicated by the slanted dashed line

across the upper right of the sectional), and find the

magnetic course of 331�. Remember to subtract easterly

variation and add westerly variation.

Figure 25

(Refer to figure 25.) Determine the magnetic course from Airpark

East Airport (area 1) to Winnsboro Airport (area 2). Magnetic

variation is 6�30'E.

ANSWER: 075�.

To find the magnetic course from

Airpark East Airport (lower left of chart) to Winnsboro

Airport (right of 2 on Fig. 25), you must find true course and

correct it for magnetic variation. Determine the true course

by placing the straight edge of your plotter along the given

route such that the grommet (center hole) is on a meridian

(the north/south line with crosslines). True course of 82� is

the number of degrees clockwise from true north. It is read

on the protractor portion of your plotter at the intersection

of the meridian. To convert this to a magnetic course,

subtract the 6�30'E (or round up to 7�E) easterly variation

and find that the magnetic course is 075�. Remember to

subtract easterly variation and add westerly variation.

Figure 22

(Refer to figure 22.) Determine the magnetic heading for a flight

from Mercer County Regional Airport (area 3) to Minot

International (area 1). The wind is from 330� at 25 knots, the true

airspeed is 100 knots, and the magnetic variation is 10�E.

ANSWER: 352�.

On Fig. 22, begin by computing the

true course (TC) from Mercer Co. Reg. (lower left corner) to

Minot Int'l. (upper left center) by drawing a line between the

two airports. Next, determine the TC by placing the grommet

on the plotter at the intersection of the course line and a

meridian (vertical line with cross-hatchings) and the top of

the plotter aligned with the course line. Note the 012� TC on

the edge of the protractor.

Next, subtract the 10� east magnetic variation from the TC to

obtain a magnetic course (MC) of 002�. Since the wind is

given true, subtract the 10� east magnetic variation to obtain

a magnetic wind direction of 320� (330 - 10).

Now use the wind side of your computer to plot the wind

direction and velocity. Place the magnetic wind direction of

320� on the inner scale on the true index. Mark 25 kt. up from

the grommet with a pencil. Turn the inner scale to the

magnetic course of 002�. Slide the grid up until the pencil

mark lies over the line for true airspeed (TAS) of 100 kt.

Correct for the 10� left wind angle by subtracting from the

magnetic course of 002� to obtain a magnetic heading of

352�. This is intuitively correct because, given the magnetic

course of 002� and a northwesterly wind, you must turn to

the left (crab into the wind) to correct for it.

Figure 23

(Refer to figure 23.) What is the magnetic heading for a flight from

Priest River Airport (area 1) to Shoshone County Airport (area 3)?

The wind is from 030� at 12 knots and the true airspeed is 95 knots.

ANSWER: 118�.

On Fig. 23, begin by computing the

true course from Priest River Airport (upper left corner) to

Shoshone County Airport (just below 3) by laying a flight

plotter between the two airports. The grommet should

coincide with the meridian (vertical line with

cross-hatchings). Note the 143� true course on the edge of

the protractor.

Next, find the magnetic variation which is given by the

dashed line marked 18�E, slanting in a northeasterly fashion

just south of Carlin Bay private airport. Subtract the 18�E

variation from TC to obtain a magnetic course of 125�. Since

the wind is given true, reduce the true wind direction of 30�

by the magnetic variation of 18�E to a magnetic wind

direction of 12�.

Now use the wind side of your computer. Turning the inner

circle to 12� under the true index, mark 12 kt. above the

grommet. Set the magnetic course of 125� under the true

index. Slide the grid so the pencil mark is on 95 kt. TAS. Note

that the pencil mark is 7� left of the center line, requiring you

to adjust the magnetic course to a 118� magnetic heading

(125� - 7�). Subtract left, add right. That is, if you are on an

easterly flight and the wind is from the north, you will want

to correct to the left.

Figure 23

(Refer to figure 23.) Determine the magnetic heading for a flight

from St. Maries Airport (area 4) to Priest River Airport (area 1). The

wind is from 340� at 10 knots and the true airspeed is 90 knots.

ANSWER: 327�.

1. This flight is from St. Maries (just below 4) to Priest River

(upper left corner) on Fig. 23.

2. TC is 346�.

3. MC = 346� - 18�E variation = 328�.

4. Wind magnetic = 340� - 18� = 322�.

5. Mark 10 kt. up when 322� under true index.

6. Put MC 328� under true index.

7. Slide grid so pencil mark is on 90 kt. TAS.

8. Note that the pencil mark is 1� left.

9. Subtract 1� from 328� MC for 327� MH.

Figure 23

(Refer to figure 23.) Determine the magnetic heading for a flight

from Sandpoint Airport (area 1) to St. Maries Airport (area 4). The

wind is from 215� at 25 knots and the true airspeed is 125 knots.

ANSWER: 169�.

1. This flight is from Sandpoint Airport (above 1), to St.

Maries (below 4) on Fig. 23.

2. TC = 181�.

3. MC = 181� - 18�E variation = 163�.

4. Wind magnetic = 215� - 18�E variation = 197�.

5. Mark up 25 kt. with 197� under true index.

6. Put MC 163� under true index.

7. Slide grid so pencil mark is on 125 kt. TAS.

8. Note that the pencil mark is 6� right.

9. Add 6� to 163� MC for 169� MH.

Figure 26

(Refer to figure 26.) Determine the magnetic heading for a flight

from Fort Worth Meacham (area 4) to Denton Muni (area 1). The

wind is from 330� at 25 knots, the true airspeed is 110 knots, and

the magnetic variation is 7�E.

ANSWER: 003�.

1. The flight is from Fort Worth Meacham (southeast of 4) to

Denton Muni (southwest of 1) on Fig. 26.

2. TC = 021�.

3. MC = 021� - 7�E variation = 014�.

4. Wind magnetic = 330� - 7�E variation = 323�.

5. Mark up 25 kt. with 323� under true index.

6. Put MC 014� under true index.

7. Slide grid so pencil mark is on 110 kt. TAS.

8. Note that the pencil mark is 11� left.

9. Subtract 11� from 014� MC for 003� MH.

Figure 24

(Refer to figure 24.) Determine the heading for a flight from

Allendale County Airport (area 1) to Claxton-Evans County Airport

(area 2). The wind is from 090� at 16 knots and the true airspeed is

90 knots.

ANSWER: 208�.

This flight is from Allendale County (above 1) to

Claxton-Evans County Airport (left of 2) on Fig. 24.

TC = 212�.

MC = 212� TC + 5�W variation = 217�.

Wind magnetic = 090� + 5�W variation = 095�.

Mark up 16 kt. with 095� under true index.

Place MC 217� under true index.

Move wind mark to 90 kt. TAS arc.

Note the pencil mark is 9� left.

Subtract 9� from 217� MC for 208� MH.

Figure 24 Figure 59

(Refer to figure 24 and figure 59.) Determine the compass heading

for a flight from Claxton-Evans County Airport (area 2) to Hampton

Varnville Airport (area 1). The wind is from 280� at 8 knots, and the

true airspeed is 85 knots.

ANSWER: 042�.

1. This flight is from Claxton-Evans (left of 2) to Hampton

Varnville (right of 1) on Fig. 24.

2. TC = 045�.

3. MC = 045� TC + 5�W variation = 050�.

4. Wind magnetic = 280� + 5�W variation = 285�.

5. Mark up 8 kt. with 285� under true index.

6. Place MC 050� under true index.

7. Move wind mark to 85 kt. TAS arc.

8. Note the pencil mark is 5� left.

9. Subtract 5� from 050� MC for 045� MH.

10. Subtract 3� compass variation (obtained from Fig. 59)

from 045� to find the compass heading of 042�.

Figure 22

(Refer to figure 22.) What is the estimated time en route from

Mercer County Regional Airport (area 3) to Minot International

(area 1)? The wind is from 330� at 25 knots and the true airspeed is

100 knots. Add 3-1/2 minutes for departure and climb-out.

ANSWER: 48 minutes.

The requirement is time en route and

not magnetic heading, so there is no need to convert TC to

MC.

Using Fig. 22, the time en route from Mercer Co. Reg. Airport

(lower left corner) to Minot (right of 1) is determined by

measuring the distance (60 NM measured with a plotter),

determining the time based on groundspeed, and adding 3.5

min. for takeoff and climb. The TC is 012� as measured with a

plotter. The wind is from 330� at 25 kt.

On the wind side of your flight computer, place the wind

direction 330� under the true index and mark 25 kt. up. Rotate

TC of 012� under the true index. Slide the grid so the pencil

mark is on the arc for TAS of 100 kt. Read 80 kt.

groundspeed under the grommet.

Turn to the calculator side and place the groundspeed of 80

kt. on the outer scale over 60 min. Find 60 NM on outer scale

and note 45 min. on the inner scale. Add 3.5 min. to 45 min.

for climb for en route time of approximately 48 min.

Figure 23

(Refer to figure 23.) Determine the estimated time en route for a

flight from Priest River Airport (area 1) to Shoshone County

Airport (area 3). The wind is from 030 at 12 knots and the true

airspeed is 95 knots. Add 2 minutes for climb-out.

ANSWER: 31 minutes.

The requirement is time en route and

not magnetic heading, so there is no need to convert TC to

MC.

1. To find the en route time from Priest River (upper left

corner) to Shoshone County (southwest of 3), use Fig. 23.

2. Measure the distance with plotter to be 49 NM.

3. TC = 143�.

4. Mark up 12 kt. with 030� under true index.

5. Put TC of 143� under true index.

6. Slide the grid so the pencil mark is on TAS of 95 kt.

7. Read the groundspeed of 99 kt. under the grommet.

8. On the calculator side, place 99 kt. on the outer scale over

60 min.

9. Read 29-1/2 min. on the inner scale below 49 NM on the

outer scale.

10. Add 2 min. for climb-out and the en route time is

approximately 31 min.

Figure 23

(Refer to figure 23.) What is the estimated time en route from

Sandpoint Airport (area 1) to St. Maries Airport (area 4)? The wind

is from 215� at 25 knots, and the true airspeed is 125 knots.

ANSWER: 34 minutes.

The requirement is time en route and

not magnetic heading, so there is no need to convert TC to

MC.

You are to find the time en route from Sandpoint

Airport (north of 1) to St. Maries Airport (southeast

of 4) on Fig. 23.

Measure the distance with plotter to be 59 NM.

TC = 181�.

Mark up 25 kt. with 215� under true index.

Put TC of 181� under true index.

Slide the grid so pencil mark is on TAS of 125 kt.

Read groundspeed of 104 kt. under the grommet.

On the calculator side, place 104 kt. on the outer scale

over 60 min.

Find 59 NM on the outer scale and read 34 min. on the

inner scale.

Figure 23

(Refer to figure 23.) What is the estimated time en route for a flight

from St. Maries Airport (area 4) to Priest River Airport (area 1)? The

wind is from 300� at 14 knots and the true airspeed is 90 knots. Add

3 minutes for climb-out.

ANSWER: 43 minutes.

The requirement is time en route and

not magnetic heading, so there is no need to convert TC to

MC.

1. Time en route from St. Maries Airport (southeast of 4) to

Priest River Airport (upper left corner) on Fig. 23.

2. Measure the distance with plotter to be 54 NM.

3. TC = 346�.

4. Mark up 14 kt. with 300� under true index.

5. Put TC of 346� under true index.

6. Slide the grid so pencil mark is on TAS of 90 kt.

7. Read groundspeed of 80 kt. under the grommet.

8. On the calculator side, place 80 kt. on the outer scale over

60 min.

9. Find 54 NM on the outer scale and read 40 min. on the

inner scale.

10. Add 3 min. for climb-out to get time en route of 43 min.

Figure 24

(Refer to figure 24.) What is the estimated time en route for a flight

from Claxton-Evans County Airport (area 2) to Hampton Varnville

Airport (area 1)? The wind is from 290� at 18 knots and the true

airspeed is 85 knots. Add 2 minutes for climb-out.

ANSWER: 39 minutes.

The distance en route from

Claxton-Evans (southwest of 2) to Hampton Varnville (east

of 1 on Fig. 24) is approximately 57 NM. Also use your

plotter to determine that the TC is 45�. The requirement is

time en route and not magnetic heading, so there is no need

to convert TC to MC.

Using the wind side of your computer, turn your true index

to the wind direction of 290� and mark 18 kt. above the

grommet with your pencil. Then turn the inner scale so that

the true index is above the TC of 45�. Place the pencil mark

on the TAS of 85 kt. and note the groundspeed of 91 kt.

Turn your flight computer over and set the speed of 91 kt.

above the 60-min. index on the inner scale. Then find the

distance of 57 NM on the outer scale to determine a time en

route of 37 min. Add 2 min. for climb-out, and the en route

time is 39 min.

Figure 24

(Refer to figure 24.) What is the estimated time en route for a flight

from Allendale County Airport (area 1) to Claxton-Evans County

Airport (area 2)? The wind is from 100� at 18 knots and the true

airspeed is 115 knots. Add 2 minutes for climb-out.

ANSWER: 30 minutes.

The requirement is time en route and

not magnetic heading, so there is no need to convert TC to

MC.

1. To find the en route time from Allendale County

(northeast of 1) to Claxton-Evans (southeast of 2), use Fig.

24.

2. Measure the distance with plotter to be 55 NM.

3. TC = 212�.

4. Mark up 18 kt. with 100� under true index.

5. Put TC of 212� under true index.

6. Slide the grid so the pencil mark is on TAS of 115 kt.

7. Read groundspeed of 120 kt. under the grommet.

8. On the calculator side, place 120 kt. on the outer scale

over 60 min.

9. Read 28 min. on the inner scale below 55 NM on the outer

scale.

10. Add 2 min. for climb-out and the en route time is 30 min.

Figure 26

(Refer to figure 26.) What is the estimated time en route for a flight

from Denton Muni (area 1) to Addison (area 2)? The wind is from

200� at 20 knots, the true airspeed is 110 knots, and the magnetic

variation is 7� east.

ANSWER: 13 minutes.

The requirement is time en route and

not magnetic heading, so there is no need to convert TC to

MC.

1. To find the en route time from Denton Muni (southwest of

1) to Addison (southwest of 2), use Fig. 26.

2. Measure the distance with plotter to be 23 NM.

3. TC = 128�.

4. Mark up 20 kt. with 200� under true index.

5. Put TC of 128� under true index.

6. Slide the grid so the pencil mark is on TAS of 110 kt.

7. Read groundspeed of 102 kt. under the grommet.

8. On the calculator side, place 102 kt. on the outer scale

over 60 min.

9. Read 13 min. on the inner scale below 23 NM on the outer

scale.

Figure 26

(Refer to figure 26.) Estimate the time en route from Addison (area

2) to Redbird (area 3). The wind is from 300� at 15 knots, the true

airspeed is 120 knots, and the magnetic variation is 7� east.

ANSWER: 8 minutes.

The requirement is time en route and

not magnetic heading, so there is no need to convert TC to

MC.

1. To find the en route time from Addison (southwest of 2)

to Redbird (above 3), use Fig. 26.

2. Measure the distance with plotter to be 18 NM.

3. TC = 186�.

4. Mark up 15 kt. with 300� under true index.

5. Put TC of 186� under true index.

6. Slide the grid so the pencil mark is on TAS of 120 kt.

7. Read groundspeed of 125 kt. under the grommet.

8. On the calculator side, place 125 kt. on the outer scale

over 60 min.

9. Read 8.5 min. on the inner scale below 18 NM on the outer

scale.

Figure 21

(Refer to figure 21.) What is your approximate position on low

altitude airway Victor 1, southwest of Norfolk (area 1), if the VOR

receiver indicates you are on the 340� radial of Elizabeth City VOR

(area 3)?

ANSWER: 18 nautical miles from Norfolk VORTAC.

First find V1 extending SW on the

233� radial from Norfolk VORTAC on Fig. 21. The V1 label

appears just above 2. Then, draw along the 340� radial from

Elizabeth City VOR (southwest of 3). If you are confused

where the exact VOR is (center of compass rose), draw a line

through the entire compass rose so your line coincides with

both your radial (here 340�) and its reciprocal (here 160�).

Note that the intersection with V1 is 18 NM from the Norfolk VORTAC.

Figure 25

(Refer to figure 25.) What is the approximate position of the aircraft

if the VOR receivers indicate the 245� radial of Sulphur Springs

VOR-DME (area 5) and the 140� radial of Bonham VORTAC (area

3)?

ANSWER: Glenmar Airport.

To locate a position based on VOR

radials, draw the radials on your map or on the plastic

overlay during the FAA knowledge test. Remember that

radials are from the VOR, or leaving the VOR.

On Fig. 25, the 245� radial from Sulphur Springs VOR-DME

extends southwest, and the 140� radial from Bonham

VORTAC extends southeast. They intersect about 1 mi. east

of Glenmar Airport.

Figure 26

(Refer to figure 26, area 5.) The VOR is tuned to the Dallas/Fort

Worth VORTAC. The omnibearing selector (OBS) is set on 253�,

with a TO indication, and a right course deviation indicator (CDI)

deflection. What is the aircraft's position from the VORTAC?

ANSWER: East-northeast.

It is not necessary to refer to Fig. 26

to solve this problem. Write the word VOR on a piece of

paper. Now draw a line through it, representing the 253�

radial and its reciprocal. Now imagine you are flying along

this line on a heading of 253�. With a TO indication and a

right CDI deflection, you are northeast of the VOR, but

south of the course.

NOTE: The FAA previously changed the figure to which

this question refers without changing the question. Figure

26 depicts the Dallas-Ft. Worth VOR/DME, not a VORTAC.

Figure 29

(Refer to figures 29, illustration 8.) The VOR receiver has the

indications shown. What radial is the aircraft crossing?

ANSWER: 030�.

The OBS is set on 210� with the

needle centered. The important factor is the (TO) indication

showing. You are thus crossing the 210� inbound bearing

but with a (TO) indication it is the 030� radial. If it was a

(FROM) indication it would be the 210� radial.

Figure 29 Figure 27

(Refer to figures 29 and 27, areas 4 and 3.) The VOR is tuned to

Jamestown VOR, and the aircraft is positioned over Cooperstown

Airport. Which VOR indication is correct?

ANSWER: 6.

Cooperstown Airport (northeast of 2

in Fig. 27) is located on the 028� radial of the Jamestown

VOR (south of 4). With a centered needle, you could have

an OBS setting of 028� and a FROM indication or an OBS

setting of 208� and a TO indication. VOR 6 fits the aircraft's

location over Cooperstown Airport. You have a FROM

indication with an OBS setting of 030� and a half-scale

deflection of the CDI to the right (because Cooperstown

Airport is north of your selected course). You are thus on

approximately the 028� radial.

Figure 29

(Refer to figures 29, illustration 1.) The VOR receiver has the

indications shown. What is the aircraft's position relative to the

station?

ANSWER: South.

The OBS is set to 030�. If the needle

were centered the airplane would be southwest of the

station. The CDI is deflected full scale left so you are right of

course. You are thus south of the VORTAC.

Figure 29

(Refer to figures 29, illustration 3.) The VOR receiver has the

indications shown. What is the aircraft's position relative to the

station?

ANSWER: Southeast.

With no (TO) or (FROM) indications

showing on VOR 3, Fig. 29, you must be flying in the zone of

ambiguity from the VOR which is perpendicular to the OBS

setting, i.e, on the 120� or 300� radials. Since you have a left

deflection, you would be on the 120� radial, or southeast of

the VOR.

Figure 29 Figure 21

(Refer to figures 29 and 21, area 3.) The VOR is tuned to Elizabeth

City VOR, and the aircraft is positioned over Shawboro. Which

VOR indication is correct?

ANSWER: 2.

See Fig. 21, northeast of 3 along the

compass rose.

Shawboro is northeast of the Elizabeth City VOR on the 030�

radial. To be over it, the needle should be centered with

either an OBS setting of 210� and a TO indication, or with an

OBS setting of 030� and a FROM indication. VOR 2 matches

the latter description.

Figure 25 Figure 29

(Refer to figures 25 and 29.) The VOR is tuned to Bonham

VORTAC (area 3) and the aircraft is positioned over the town of

Sulphur Springs (area 5). Which VOR indication is correct?

ANSWER: 7.

The town of Sulpher Springs

(south-southwest of 5) is on the 120� radial of Bonham

VORTAC. Illustration 7 shows the VOR receiver tuned to the

210� radial, which is perpendicular to (90� away from) the

120� radial. This places the aircraft in the zone of ambiguity,

which results in neither a TO nor a FROM indication and an

unstable CDI, which can be deflected left or right.

Figure 30

(Refer to figures 30, illustration 1.) Determine the magnetic bearing

TO the station.

ANSWER: 210�.

Fig. 30 shows movable card ADFs. In

these, the airplane's magnetic heading is always on top and

the needle always indicates the magnetic bearing TO the

station. Thus, the magnetic bearing TO the station in ADF 1

is 210�.

Figure 30

(Refer to figures 30, illustration 2.) What magnetic bearing should

the pilot use to fly TO the station?

ANSWER: 190�.

Fig. 30, illustration 2, is a movable

card ADF. This ADF displays the airplane's magnetic

heading at the top, and the needle always points to the

magnetic bearing TO the station. Thus, the magnetic bearing

TO the station in ADF 2 is 190�.

Figure 30

(Refer to figures 30, illustration 2.) Determine the approximate

heading to intercept the 180� bearing TO the station.

ANSWER: 220�.

A 180� bearing to the station would

put us directly north of the station assuming no wind.

Currently, we are northeast of the station (190� bearing),

proceeding in a northwest direction (magnetic heading of

315�). If we want to intercept the 180� bearing to the station,

we should turn to the southwest, or 220�.

Figure 30

(Refer to figures 30, illustration 3.) What is the magnetic bearing

FROM the station?

ANSWER: 115�.

The tail of the needle of an ADF

indicates the magnetic bearing FROM the station on a

movable card ADF. ADF 3 shows a magnetic bearing of 115�

FROM.

Figure 30

(Refer to figure 30.) Which ADF indication represents the aircraft

tracking TO the station with a right crosswind?

ANSWER: 4.

If you have a crosswind from the

right, you must adjust your heading (crab) to the right to

compensate for the wind. In that case, the needle would

point to the left of the nose, as in ADF 4.

Figure 30

(Refer to figures 30, illustration 1.) What outbound bearing is the

aircraft crossing?

ANSWER: 030�.

The outbound (magnetic) bearing is

the bearing FROM the station, which is represented by the

tail of the needle in a movable card ADF. The airplane in

ADF 1 is crossing the 030� outbound bearing (radial) since

the tail of the needle is pointing to 030�.

Figure 30

(Refer to figures 30, illustration 1.) What is the relative bearing TO

the station?

ANSWER: 240�.

The relative bearing is measured

clockwise from the nose of the airplane to the head of the

needle. From ADF 1, the magnetic heading (MH) is 330� and

the magnetic bearing (MB) TO the station is 210�. Use the

following standard formula to solve for the relative bearing

(RB) TO the station:

MH + RB = MB (TO)

330� + RB = 210�

RB = -120� (210 - 330)

Since it is less than 0�, add 360� to determine the RB of 240�

(-120 + 360).

Figure 30

(Refer to figures 30, illustration 2.) What is the relative bearing TO

the station?

ANSWER: 235�.

The relative bearing is measured

clockwise from the nose of the airplane to the head of the

needle. Use the following standard formula from ADF 2 in

the formula to determine the RB:

MH + RB = MB (TO)

315� + RB = 190�

RB = -125� (190 - 315)

Since it is less than 0�, add 360� to determine the RB of 235�

(-125 + 360) TO the station.

Figure 30

(Refer to figures 30, illustration 4.) What is the relative bearing TO

the station?

ANSWER: 340�.

The relative bearing (RB) is measured

clockwise from the nose of the airplane to the head of the

needle. Use the information from ADF 4 in the formula to

determine the RB:

MH + RB = MB (TO)

220� + RB = 200�

RB = -20� (200 - 220)

Since it is less than 0�, add 360� to determine the RB of 340�

(-20 + 360) TO the station.

Figure 31

(Refer to figures 31, illustration 3.) The relative bearing TO the

station is

ANSWER: 180�.

The relative bearing (RB) is measured

clockwise from the nose of the airplane to the head of the

needle. Since this is a fixed card ADF, the needle points to

the relative bearing TO the station. ADF 3 in Fig. 31 shows a

relative bearing of 180�.

Figure 31

(Refer to figures 31, illustration 1.) The relative bearing TO the

station is

ANSWER: 315�.

On a fixed card ADF, the needle

points to the relative bearing TO the station. ADF 1 in Fig.

31 shows a relative bearing of 315�.

Figure 31

(Refer to figures 31, illustration 2.) The relative bearing TO the

station is

ANSWER: 090�.

On a fixed card ADF, the needle

points to the relative bearing TO the station. ADF 2 in Fig.

31 shows a relative bearing of 090� TO the station.

Figure 31

(Refer to figures 31, illustration 4.) On a magnetic heading of 320�,

the magnetic bearing TO the station is

ANSWER: 185�.

The magnetic bearing TO the station

is required. Use the standard ADF formula.

MH + RB = MB (TO)

320� + 225� = MB (TO)

545� = MB (TO)

Since it is greater than 360�, subtract 360� to determine the

MB (TO) is 185� (545 - 360).

Figure 31

(Refer to figures 31, illustration 5.) On a magnetic heading of 035�,

the magnetic bearing TO the station is

ANSWER: 035�.

The magnetic bearing TO the station

is required. Use the standard ADF formula.

MH + RB = MB (TO)

035� + 0� = MB (TO)

035� = MB (TO)

Figure 31

(Refer to figures 31, illustration 6.) On a magnetic heading of 120�,

the magnetic bearing TO the station is

ANSWER: 165�.

The magnetic bearing TO the station

is required. Use the standard ADF formula.

MH + RB = MB (TO)

120� + 045� = MB (TO)

165� = MB (TO)

Figure 31

(Refer to figures 31, illustration 6.) If the magnetic bearing TO the

station is 240�, the magnetic heading is

ANSWER: 195�.

The magnetic heading is required. Use

the standard ADF formula.

MH + RB = MB (TO)

MH + 045� = 240�

MH = 240� - 045�

MH = 195�

Figure 31

(Refer to figures 31, illustration 7.) If the magnetic bearing TO the

station is 030�, the magnetic heading is

ANSWER: 120�.

The magnetic heading is required. Use

the standard ADF formula.

MH + RB = MB (TO)

MH + 270� = 030�

MH = 030� - 270�

MH = -240� (add 360�)

MH = 120�

Figure 31

(Refer to figures 31, illustration 8.) If the magnetic bearing TO the

station is 135�, the magnetic heading is

ANSWER: 360�.

The magnetic heading is required. Use

the standard ADF formula.

MH + RB = MB (TO)

MH + 135� = 135�

MH = 0� = (360�)

Figure 22

(Refer to figure 22.) What course should be selected on the

omnibearing selector (OBS) to make a direct flight from Mercer

County Regional Airport (area 3) to the Minot VORTAC (area 1)

with a TO indication?

ANSWER: 359�.

Use Fig. 22 to find the course

(omnibearing selector with a "TO" indication) from Mercer

County Regional Airport (lower left corner) to the Minot

VORTAC (right of 1). Note the compass rose (based on

magnetic courses) which indicates the Minot VORTAC. A

straight line from Mercer to Minot Airport coincides the

compass rose at 179�. Since the route is north TO Minot, not

south from Minot, compute the reciprocal direction as 359�

(179� + 180�).

Figure 24

(Refer to figure 24.) On what course should the VOR receiver (OBS)

be set to navigate direct from Hampton Varnville Airport (area 1) to

Savannah VORTAC (area 3)?

ANSWER: 183�.

You are to find the OBS course

setting from Hampton Varnville Airport (right of 1) to

Savannah VORTAC (below 3 on Fig. 24). Since compass

roses are based on magnetic courses, you can find that a

straight line from Hampton Varnville Airport to Savannah

VORTAC coincides the Savannah VORTAC compass rose

at 003�. Since the route is south to (not north from)

Savannah, compute the reciprocal direction as 183� magnetic

(003� + 180�). To use the VOR properly when flying to a VOR

station, the course you select with the OBS should be the

reciprocal of the radial you will be tracking. If this is not

done, reverse sensing occurs.

Figure 25

(Refer to figure 25.) On what course should the VOR receiver (OBS)

be set in order to navigate direct from Majors Airport (area 1) to

Quitman VORTAC (area 2)?

ANSWER: 101�.

You are to find the radial to navigate

direct from Majors Airport (less than 2 in. north and east of

1) to Quitman VORTAC (southeast of 2 on Fig. 25). A

compass rose, based on magnetic course, exists around the

Quitman VORTAC. A straight line from Majors Airport to

Quitman VORTAC coincides with this compass rose at 281�.

Since the route is east to (not west from) Quitman, compute

the reciprocal direction as 101� magnetic (281� - 180�).

Prior to starting each maneuver, pilots should

ANSWER: visually scan the entire area for collision avoidance.

Prior to each maneuver, a pilot should

visually scan the entire area for collision avoidance. Many

maneuvers require a clearing turn which should be used for

this purpose.

When taxiing with strong quartering tailwinds, which aileron

positions should be used?

ANSWER: Ailerons down on the side from which the wind is blowing.

When there is a strong quartering

tailwind, the aileron should be down on the side from which

the wind is blowing (when taxiing away from the wind, turn

away from the wind) to help keep the wind from getting

under that wing and flipping the airplane over.

Which aileron positions should a pilot generally use when taxiing

in strong quartering headwinds?

ANSWER: Aileron up on the side from which the wind is blowing.

When there is a strong quartering

headwind, the aileron should be up on the side from which

the wind is blowing to help keep the wind from getting under

that wing and blowing the aircraft over. (When taxiing into

the wind, turn into the wind.)

Which wind condition would be most critical when taxiing a

nosewheel equipped high-wing airplane?

ANSWER: Quartering tailwind.

The most critical wind condition

when taxiing a nosewheel-equipped high-wing airplane is a

quartering tailwind, which can flip a high-wing airplane over

on its top. This should be prevented by holding the elevator

in the down position, i.e., controls forward, and the aileron

down on the side from which the wind is coming.

Figure 9

(Refer to figure 9, area A.) How should the flight controls be held

while taxiing a tricycle-gear equipped airplane into a left quartering

headwind?

ANSWER: Left aileron up, elevator neutral.

Given a left quartering headwind, the

left aileron should be kept up to spoil the excess lift on the

left wing that the crosswind is creating. The elevator should

be neutral to keep from putting too much or too little weight

on the nosewheel.

Figure 9

(Refer to figure 9, area C.) How should the flight controls be held

while taxiing a tricycle-gear equipped airplane with a left quartering

tailwind?

ANSWER: Left aileron down, elevator down.

With a left quartering tailwind, the left

aileron should be down so the wind does not get under the

left wing and flip the airplane over. Also, the elevator should

be down, i.e., controls forward, so the wind does not get

under the tail and blow the airplane tail over front.

Figure 9

(Refer to figure 9, area B.) How should the flight controls be held

while taxiing a tailwheel airplane into a right quartering headwind?

ANSWER: Right aileron up, elevator up.

When there is a right quartering

headwind, the right aileron should be up to spoil the excess

lift on the right wing that the crosswind is creating. The

elevator should be up to keep weight on the tailwheel to

help maintain maneuverability.

Figure 9

(Refer to figure 9, area C.) How should the flight controls be held

while taxiing a tailwheel airplane with a left quartering tailwind?

ANSWER: Left aileron down, elevator down.

When there is a left quartering

tailwind, the left aileron should be held down so the wind

does not get under the left wing and flip the airplane over.

Also, the elevator should be down, i.e., controls forward, so

the wind does not get under the tail and blow the airplane

tail over front.

What force makes an airplane turn?

ANSWER: The horizontal component of lift.

When the wings of an airplane are

not level, the lift is not entirely vertical and tends to pull the

airplane toward the direction of the lower wing. An airplane

is turned when the pilot coordinates rudder, aileron, and

elevator to bank in order to attain a horizontal component of

lift.

In what flight condition must an aircraft be placed in order to spin?

ANSWER: Stalled.

In order to enter a spin, an airplane

must always first be stalled. Thereafter, the spin is caused

when one wing becomes less stalled than the other wing.

During a spin to the left, which wing(s) is/are stalled?

ANSWER: Both wings are stalled.

In order to enter a spin, an airplane

must always first be stalled. Thereafter, the spin is caused

when one wing is less stalled than the other wing. In a spin

to the left, the right wing is less stalled than the left wing.

The most effective method of scanning for other aircraft for

collision avoidance during nighttime hours is to use

ANSWER: peripheral vision by scanning small sectors and utilizing

offcenter viewing.

At night, collision avoidance

scanning must use the off-center portions of the eyes.

These portions are most effective at seeing objects at night.

Accordingly, peripheral vision should be used, scanning

small sectors and using off-center viewing. This is in

contrast to daytime searching for air traffic, when center

viewing should be used.

What is the most effective way to use the eyes during night flight?

ANSWER: Scan slowly to permit offcenter viewing.

Physiologically, the eyes are most

effective at seeing objects off-center at night. Accordingly,

pilots should scan slowly to permit off-center viewing.

The best method to use when looking for other traffic at night is to

ANSWER: look to the side of the object and scan slowly.

Physiologically, the eyes are most

effective at seeing objects off-center at night. Accordingly,

pilots should scan slowly to permit off-center viewing.

During a night flight, you observe a steady red light and a flashing

red light ahead and at the same altitude. What is the general

direction of movement of the other aircraft?

ANSWER: The other aircraft is crossing to the left.

Airplane position lights consist of a

steady red light on the left wing (looking forward), a green

light on the right wing, and a white light on the tail.

Accordingly, if you observe a steady red light, you are

looking at the tip of a left wing, which means the other plane

is traveling from your right to left (crossing to the left). The

red flashing light is the beacon.

During a night flight, you observe a steady white light and a

flashing red light ahead and at the same altitude. What is the

general direction of movement of the other aircraft?

ANSWER: The other aircraft is flying away from you.

A steady white light (the tail light)

indicates the other airplane is moving away from you. The

flashing red light is the beacon light.

During a night flight, you observe steady red and green lights

ahead and at the same altitude. What is the general direction of

movement of the other aircraft?

ANSWER: The other aircraft is approaching head-on.

If you observe steady red and green

lights at the same altitude, the other airplane is approaching

head-on. You should take evasive action to the right.

Airport taxiway edge lights are identified at night by

ANSWER: blue omnidirectional lights.

Taxiway edge lights are used to

outline the edges of taxiways during periods of darkness or

restricted visibility conditions. These lights are identified at

night by blue omnidirectional lights.

VFR approaches to land at night should be accomplished

ANSWER: the same as during daytime.

Every effort should be made to

execute approaches and landings at night in the same

manner as they are made in the day. Inexperienced pilots

often have a tendency to make approaches and landings at

night with excessive airspeed.

The most important rule to remember in the event of a power failure

after becoming airborne is to

ANSWER: immediately establish the proper gliding attitude and

airspeed.

In the event of a power failure after

becoming airborne, the most important rule to remember is to

maintain best glide airspeed. This will usually require a pitch

attitude slightly higher than level flight. Invariably, with a

power failure, one returns to ground, but emphasis should

be put on a controlled return rather than a crash return.

Many pilots attempt to maintain altitude at the expense of

airspeed, resulting in a stall or stall/spin.

Angle of attack is defined as the angle between the chord line of an

airfoil and the

ANSWER: direction of the relative wind.

The angle of attack is the angle

between the wing chord line and the direction of the relative

wind. The wing chord line is a straight line from the leading

edge to the trailing edge of the wing. The relative wind is the

direction of airflow relative to the wing when the wing is

moving through the air.

What are the standard temperature and pressure values for sea

level?

ANSWER: 15�C and 29.92" Hg.

The standard temperature and

pressure values for sea level are 15�C and 29.92" Hg. This is

equivalent to 59�F and 1013.2 millibars of mercury.

Every physical process of weather is accompanied by, or is the

result of, a

ANSWER: heat exchange.

Every physical process of weather is

accompanied by, or is the result of, a heat exchange. A heat

differential (difference between the temperatures of two air

masses) causes a differential in pressure, which in turn

causes movement of air. Heat exchanges occur constantly,

e.g., melting, cooling, updrafts, downdrafts, wind, etc.

What causes variations in altimeter settings between weather

reporting points?

ANSWER: Unequal heating of the Earth's surface.

Unequal heating of the Earth's

surface causes differences in air pressure, which is reflected

in differences in altimeter settings between weather

reporting points.

The most frequent type of ground or surface-based temperature

inversion is that which is produced by

ANSWER: terrestrial radiation on a clear, relatively still night.

An inversion often develops near the

ground on clear, cool nights when wind is light. The ground

loses heat and cools the air near the ground while the

temperature a few hundred feet above changes very little.

Thus, temperature increases in height, which is an inversion.

A temperature inversion would most likely result in which weather

condition?

ANSWER: An increase in temperature as altitude is increased.

By definition, a temperature inversion

is a situation in which the temperature increases as altitude

increases. The normal situation is that the temperature

decreases as altitude increases.

Which weather conditions should be expected beneath a low-level

temperature inversion layer when the relative humidity is high?

ANSWER: Smooth air, poor visibility, fog, haze, or low clouds.

Beneath temperature inversions, there

is usually smooth air because there is little vertical

movement due to the inversion. There is also poor visibility

due to fog, haze, and low clouds (when there is high relative

humidity).

Under what condition is pressure altitude and density altitude the

same value?

ANSWER: At standard temperature.

Pressure altitude and density altitude

are the same when temperature is standard.

Under which condition will pressure altitude be equal to true

altitude?

ANSWER: When standard atmospheric conditions exist.

Pressure altitude equals true altitude

when standard atmospheric conditions (29.92" Hg and 15�C

at sea level) exist.

If a pilot changes the altimeter setting from 30.11 to 29.96, what is

the approximate change in indication?

ANSWER: Altimeter will indicate 150 feet lower.

Atmospheric pressure decreases

approximately 1" of mercury for every 1,000 ft. of altitude

gained. As an altimeter setting is changed, the change in

altitude indication changes the same way (i.e., approximately

1,000 ft. for every 1" change in altimeter setting) and in the

same direction (i.e., lowering the altimeter setting lowers the

altitude reading). Thus, changing from 30.11 to 29.96 is a

decrease of .15 in., or 150 ft. (.15 x 1,000 ft.) lower.

If a flight is made from an area of low pressure into an area of high

pressure without the altimeter setting being adjusted, the altimeter

will indicate

ANSWER: lower than the actual altitude above sea level.

When an altimeter setting is at a lower

value than the correct setting, the altimeter is indicating less

than it should and thus would be showing lower than the

actual altitude above sea level.

If a flight is made from an area of high pressure into an area of

lower pressure without the altimeter setting being adjusted, the

altimeter will indicate

ANSWER: higher than the actual altitude above sea level.

When flying from higher pressure to

lower pressure without adjusting your altimeter, the altimeter

will indicate a higher than actual altitude. As you adjust an

altimeter barometric setting lower, the altimeter indicates

lower.

Which condition would cause the altimeter to indicate a lower

altitude than true altitude?

ANSWER: Air temperature warmer than standard.

In air that is warmer than standard

temperature, the airplane will be higher than the altimeter

indicates. Said another way, the altimeter will indicate a

lower altitude than actually flown.

Under what condition will true altitude be lower than indicated

altitude?

ANSWER: In colder than standard air temperature.

The airplane will be lower than the

altimeter indicates when flying in air that is colder than

standard temperature. Remember that altimeter readings are

adjusted for changes in barometric pressure but not for

changes in temperature. When one flies from warmer to cold

air and keeps a constant indicated altitude at a constant

altimeter setting, the plane has actually descended.

Which factor would tend to increase the density altitude at a given

airport?

ANSWER: An increase in ambient temperature.

When air temperature increases,

density altitude increases because, at a higher temperature,

the air is less dense.

The wind at 5,000 feet AGL is southwesterly while the surface wind

is southerly. This difference in direction is primarily due to

ANSWER: friction between the wind and the surface.

Winds aloft at 5,000 ft. are largely

affected by Coriolis force, which deflects wind to the right,

in the Northern Hemisphere. But at the surface, the winds

will be more southerly (they were southwesterly aloft)

because Coriolis force has less effect at the surface where

the wind speed is slower. The wind speed is slower at the

surface due to the friction between the wind and the surface.

The presence of ice pellets at the surface is evidence that there

ANSWER: is a temperature inversion with freezing rain at a higher

altitude.

Rain falling through colder air may

freeze during its descent, falling as ice pellets. Ice pellets

always indicate freezing rain at a higher altitude.

What is meant by the term "dewpoint"?

ANSWER: The temperature to which air must be cooled to become

saturated.

Dew point is the temperature to which

air must be cooled to become saturated, or have 100%

humidity.

The amount of water vapor which air can hold depends on the

ANSWER: air temperature.

Air temperature largely determines

how much water vapor can be held by the air. Warm air can

hold more water vapor than cool air.

What are the processes by which moisture is added to unsaturated air?

ANSWER: Evaporation and sublimation.

Evaporation is the process of

converting a liquid to water vapor, and sublimation is the

process of converting ice to water vapor.

Which conditions result in the formation of frost?

ANSWER: The temperature of the collecting surface is at or below

the dewpoint of the adjacent air and the dewpoint is below

freezing.

Frost forms when both the collecting

surface is below the dew point of the adjacent air AND the

dew point is below freezing. Frost is the direct sublimation of

water vapor to ice crystals.

Clouds, fog, or dew will always form when

ANSWER: water vapor condenses.

As water vapor condenses, it

becomes visible as clouds, fog, or dew.

At approximately what altitude above the surface would the pilot

expect the base of cumuliform clouds if the surface air temperature

is 82�F and the dewpoint is 38�F?

ANSWER: 10,000 feet AGL.

The height of cumuliform cloud bases

can be estimated using surface temperature/dew point

spread. Unsaturated air in a convective current cools at

about 5.4�F/1,000 ft., and dew point decreases about

1�F/1,000 ft. In a convective current, temperature and dew

point converge at about 4.4�F/1,000 ft. Thus, if the

temperature/dew point spread is 44� (82� - 38�), divide 44 by

4.4 to obtain 10,000 ft. AGL.

What is the approximate base of the cumulus clouds if the surface

air temperature at 1,000 feet MSL is 70�F and the dewpoint is 48�F?

ANSWER: 6,000 feet MSL.

The height of cumuliform cloud bases

can be estimated using surface temperature/dew point

spread. Unsaturated air in a convective current cools at

about 5.4�F/1,000 ft., and dew point decreases about

1�F/1,000 ft. In a convective current, temperature and dew

point converge at about 4.4�F/1,000 ft. Thus, if the

temperature and dew point are 70�F and 48�F, respectively,

at 1,000 ft. MSL, there would be a 22� spread which, divided

by the lapse rate of 4.4, is approximately 5,000 ft. AGL, or

6,000 ft. MSL (5,000 + 1,000).

What is a characteristic of stable air?

ANSWER: Stratiform clouds.

Characteristics of a stable air mass

include stratiform clouds, continuous precipitation, smooth

air, and fair to poor visibility in haze and smoke.

Moist, stable air flowing upslope can be expected to

ANSWER: produce stratus type clouds.

Moist, stable air flowing upslope can

be expected to produce stratus type clouds as the air cools

adiabatically as it moves up sloping terrain.

If an unstable air mass is forced upward, what type clouds can be expected?

ANSWER: Clouds with considerable vertical development and

associated turbulence.

When unstable air is lifted, it usually

results in considerable vertical development and associated

turbulence, i.e., convective activity.

What are characteristics of unstable air?

ANSWER: Turbulence and good surface visibility.

Characteristics of an unstable air

mass include cumuliform clouds, showery precipitation,

turbulence, and good visibility, except in blowing

obstructions.

A stable air mass is most likely to have which characteristic?

ANSWER: Smooth air.

Characteristics of a stable air mass

include stratiform clouds and fog, continuous precipitation,

smooth air, and fair to poor visibility in haze and smoke.

What are characteristics of a moist, unstable air mass?

ANSWER: Cumuliform clouds and showery precipitation.

Characteristics of an unstable air

mass include cumuliform clouds, showery precipitation,

turbulence, and good visibility, except in blowing

obstructions.

What measurement can be used to determine the stability of the

atmosphere?

ANSWER: Actual lapse rate.

The stability of the atmosphere is

determined by vertical movements of air. Warm air rises

when the air above is cooler. The actual lapse rate, which is

the decrease of temperature with altitude, is therefore a

measure of stability.

What would decrease the stability of an air mass?

ANSWER: Warming from below.

When air is warmed from below, even

though cooling adiabatically, it remains warmer than the

surrounding air. The colder, more dense surrounding air

forces the warmer air upward and an unstable condition

develops.

What feature is associated with a temperature inversion?

ANSWER: A stable layer of air.

A temperature inversion is associated

with an increase in temperature with height, a reversal of

normal decrease in temperature with height. Thus, any warm

air rises to where it is the same temperature and forms a

stable layer of air.

An almond or lens-shaped cloud which appears stationary, but

which may contain winds of 50 knots or more, is referred to as

ANSWER: a lenticular cloud.

Lenticular clouds are lens-shaped

clouds which indicate the crests of standing mountain

waves. They form in the updraft and dissipate in the

downdraft, so they do not move as the wind blows through

them. Lenticular clouds may contain winds of 50 kt. or more

and are extremely dangerous.

Crests of standing mountain waves may be marked by stationary,

lens-shaped clouds known as

ANSWER: standing lenticular clouds.

Lens-shaped clouds, which indicate

crests of standing mountain waves, are called standing

lenticular clouds. They form in the updraft and dissipate in

the downdraft so that they do not move as the wind blows

through them.

Clouds are divided into four families according to their

ANSWER: height range.

The four families of clouds are high

clouds, middle clouds, low clouds, and clouds with

extensive vertical development. Thus, they are based upon

their height range.

The suffix "nimbus," used in naming clouds, means

ANSWER: a rain cloud.

The suffix nimbus or the prefix nimbo

means a rain cloud.

What clouds have the greatest turbulence?

ANSWER: Cumulonimbus.

The greatest turbulence occurs in

cumulonimbus clouds, which are thunderstorm clouds.

What cloud types would indicate convective turbulence?

ANSWER: Towering cumulus clouds.

Towering cumulus clouds are an early

stage of cumulonimbus clouds, or thunderstorms, which are

based on convective turbulence, i.e., an unstable lapse rate.

The boundary between two different air masses is referred to as a

ANSWER: front.

A front is a surface, interface, or

transition zone of discontinuity between two adjacent air

masses of different densities. It is the boundary between

two different air masses.

One weather phenomenon which will always occur when flying

across a front is a change in the

ANSWER: wind direction.

The definition of a front is the zone of

transition between two air masses of different air pressure or

density, e.g., the area separating high and low pressure

systems. Due to the difference in changes in pressure

systems, there will be a change in wind.

One of the most easily recognized discontinuities across a front is

ANSWER: a change in temperature.

Of the many changes which take

place across a front the most easily recognized is the change

in temperature. When flying through a front you will notice a

significant change in temperature, especially at low altitudes.

Steady precipitation preceding a front is an indication of

ANSWER: stratiform clouds with little or no turbulence.

Steady precipitation preceding a front

is usually an indication of a warm front, which results from

warm air being cooled from the bottom by colder air. This

results in stratiform clouds with little or no turbulence.

One in-flight condition necessary for structural icing to form is

ANSWER: visible moisture.

Two conditions are necessary for

structural icing while in flight. First, the airplane must be

flying through visible moisture, such as rain or cloud

droplets. Second, the temperature at the point where the

moisture strikes the airplane must be freezing or below.

Possible mountain wave turbulence could be anticipated when

winds of 40 knots or greater blow

ANSWER: across a mountain ridge, and the air is stable.

Always anticipate possible mountain

wave turbulence when the air is stable and winds of 40 kt. or

greater blow across a mountain or ridge.

Where does wind shear occur?

ANSWER: At all altitudes, in all directions.

Wind shear is the eddies in between

two wind currents of differing velocities, direction, or both.

Wind shear may be associated with either a wind shift or a

wind speed gradient at any level in the atmosphere.

A pilot can expect a wind-shear zone in a temperature inversion

whenever the windspeed at 2,000 to 4,000 feet above the surface is

at least

ANSWER: 25 knots.

When taking off or landing in calm

wind under clear skies within a few hours before or after

sunset, prepare for a temperature inversion near the ground.

You can be relatively certain of a shear zone in the inversion

if you know the wind is 25 kt. or more at 2,000 to 4,000 ft.

Allow a margin of airspeed above normal climb or approach

speed to alleviate the danger of stall in the event of

turbulence or sudden change in wind velocity.

When may hazardous wind shear be expected?

ANSWER: In areas of low-level temperature inversion, frontal zones,

and clear air turbulence.

Wind shear is the abrupt rate of

change of wind velocity (direction and/or speed) per unit of

distance and is normally expressed as vertical or horizontal

wind shear. Hazardous wind shear may be expected in areas

of low-level temperature inversion, frontal zones, and clear

air turbulence.

Why is frost considered hazardous to flight?

ANSWER: Frost spoils the smooth flow of air over the wings,

thereby decreasing lifting capability.

Frost does not change the basic

aerodynamic shape of the wing, but the roughness of its

surface spoils the smooth flow of air, thus causing an

increase in drag and an early airflow separation over the

wing, resulting in a loss of lift.

How does frost affect the lifting surfaces of an airplane on takeoff?

ANSWER: Frost may prevent the airplane from becoming airborne at

normal takeoff speed.

Frost that is not removed from the

surface of an airplane prior to takeoff may make it difficult to

get the airplane airborne at normal takeoff speed. The frost

disrupts the airflow over the wing, which increases drag.

In which environment is aircraft structural ice most likely to have

the highest accumulation rate?

ANSWER: Freezing rain.

Freezing rain usually causes the

highest accumulation rate of structural icing because of the

nature of the supercooled water striking the airplane.

If there is thunderstorm activity in the vicinity of an airport at

which you plan to land, which hazardous atmospheric

phenomenon might be expected on the landing approach?

ANSWER: Wind-shear turbulence.

The most hazardous atmospheric

phenomenon near thunderstorms is wind shear turbulence.

A nonfrontal, narrow band of active thunderstorms that often

develop ahead of a cold front is known as a

ANSWER: squall line.

A nonfrontal, narrow band of active

thunderstorms that often develop ahead of a cold front is

known as a squall line.

What conditions are necessary for the formation of

thunderstorms?

ANSWER: High humidity, lifting force, and unstable conditions.

Thunderstorms form when there is

sufficient water vapor, an unstable lapse rate, and an initial

upward boost (lifting) to start the storm process.

During the life cycle of a thunderstorm, which stage is

characterized predominately by downdrafts?

ANSWER: Dissipating.

Thunderstorms have three life cycles:

cumulus, mature, and dissipating. It is in the dissipating

stage that the storm is characterized by downdrafts as the

storm rains itself out.

Thunderstorms reach their greatest intensity during the

ANSWER: mature stage.

Thunderstorms reach their greatest

intensity during the mature stage, where updrafts and

downdrafts cause a high level of wind shear.

What feature is normally associated with the cumulus stage of a

thunderstorm?

ANSWER: Continuous updraft.

The cumulus stage of a thunderstorm

has continuous updrafts which build the storm. The water

droplets are carried up until they become too heavy. Once

they begin falling and creating downdrafts, the storm

changes from the cumulus to the mature stage.

Which weather phenomenon signals the beginning of the mature

stage of a thunderstorm?

ANSWER: Precipitation beginning to fall.

The mature stage of a thunderstorm

begins when rain begins falling. This means that the

downdrafts are occurring sufficiently to carry water all the

way through the thunderstorm.

Thunderstorms which generally produce the most intense hazard

to aircraft are

ANSWER: squall line thunderstorms.

A squall line is a nonfrontal narrow

band of active thunderstorms. It often contains severe,

steady-state thunderstorms and presents the single most

intense weather hazard to airplanes.

The conditions necessary for the formation of cumulonimbus

clouds are a lifting action and

ANSWER: unstable, moist air.

Unstable moist air in addition to a

lifting action, i.e., convective activity, are needed to form

cumulonimbus clouds.

Upon encountering severe turbulence, which flight condition

should the pilot attempt to maintain?

ANSWER: Level flight attitude.

Attempting to hold altitude and

airspeed in severe turbulence can lead to overstressing the

airplane. Rather, you should set power to what normally will

maintain VA, and simply attempt to maintain a level flight

attitude.

If the temperature/dewpoint spread is small and decreasing, and

the temperature is 62�F, what type weather is most likely to

develop?

ANSWER: Fog or low clouds.

The difference between the air

temperature and dew point is the temperature/dew point

spread. As the temperature/dew point spread decreases, fog

or low clouds tend to develop.

In which situation is advection fog most likely to form?

ANSWER: An air mass moving inland from the coast in winter.

Advection fog forms when moist air

moves over colder ground or water. It is most common in

coastal areas.

What situation is most conducive to the formation of radiation

fog?

ANSWER: Warm, moist air over low, flatland areas on clear, calm

nights.

Radiation fog is shallow fog of which

ground fog is one form. It occurs under conditions of clear

skies, little or no wind, and a small temperature/dew point

spread. The fog forms almost exclusively at night or near

dawn as a result of terrestrial radiation cooling the ground

and the ground cooling the air on contact with it.

What types of fog depend upon wind in order to exist?

ANSWER: Advection fog and upslope fog.

Advection fog forms when moist air

moves over colder ground or water. It is most common in

coastal areas. Upslope fog forms when wind blows moist air

upward over rising terrain and the air cools below its dew

point. Both advection fog and upslope fog require wind to

move air masses.

Low-level turbulence can occur and icing can become hazardous in

which type of fog?

ANSWER: Steam fog.

Steam fog forms in winter when cold,

dry air passes from land areas over comparatively warm

ocean waters, and is composed entirely of water droplets

that often freeze quickly. Low-level turbulence can occur

and icing can become hazardous.

Convective circulation patterns associated with sea breezes are

caused by

ANSWER: cool, dense air moving inland from over the water.

Sea breezes are caused by cool and

more dense air moving inland off the water. Once over the

warmer land, the air heats up and rises. Thus the cooler,

more dense air from the sea forces the warmer air up.

Currents push the hot air over the water where it cools and

descends, starting the cycle over again. This process is

caused by land heating faster than water.

The development of thermals depends upon

ANSWER: solar heating.

Thermals are updrafts in small scale

convective currents. Convective currents are caused by

uneven heating of the earth's surface. Solar heating is the

means of heating the earth's surface.

Which weather phenomenon is always associated with a

thunderstorm?

ANSWER: Lightning.

A thunderstorm, by definition, has

lightning, because lightning causes the thunder.

To get a complete weather briefing for the planned flight, the pilot

should request

ANSWER: a standard briefing.

To get a complete briefing before a

planned flight, the pilot should request a standard briefing.

This will include all pertinent information needed for a safe

flight.

Which type weather briefing should a pilot request, when

departing within the hour, if no preliminary weather information has

been received?

ANSWER: Standard briefing.

A pilot should request a standard

briefing anytime (s)he is planning a flight and has not

received a previous briefing or has not received preliminary

information through mass dissemination media (e.g., TWEB,

PATWAS, etc.).

Which type of weather briefing should a pilot request to

supplement mass disseminated data?

ANSWER: An abbreviated briefing.

An abbreviated briefing will be

provided when the user requests information to supplement

mass disseminated data, update a previous briefing, or to be

limited to specific information.

A weather briefing that is provided when the information requested

is 6 or more hours in advance of the proposed departure time is

ANSWER: an outlook briefing.

An outlook briefing is given when the

briefing is 6 or more hours before the proposed departure

time.

When telephoning a weather briefing facility for preflight weather

information, pilots should state

ANSWER: the aircraft identification or the pilot's name.

When requesting a briefing you

should provide the briefer with the following information:

VFR or IFR, aircraft identification or the pilot's name, aircraft

type, departure point, route of flight, destination, altitude,

estimated time of departure, and time en route or estimated

time of arrival.

To update a previous weather briefing, a pilot should request

ANSWER: an abbreviated briefing.

An abbreviated briefing will be

provided when the user requests information (1) to

supplement mass disseminated data, (2) to update a

previous briefing, or (3) to be limited to specific information.

When requesting weather information for the following morning, a

pilot should request

ANSWER: an outlook briefing.

An outlook briefing should be

requested when the briefing is 6 or more hr. in advance of

the proposed departure.

Transcribed Weather Broadcasts (TWEB's) may be monitored by

tuning the appropriate radio receiver to certain

ANSWER: VOR and NDB frequencies.

Transcribed Weather Broadcasts

(TWEBs) are broadcast on selected VOR and NDB

frequencies.

Individual forecasts for specific routes of flight can be obtained

from which weather source?

ANSWER: Transcribed Weather Broadcasts (TWEB's).

Forecasts for specific routes of flight

should be obtained from Transcribed Weather Broadcasts

(TWEBs) which are based upon specific routes.

For aviation purposes, ceiling is defined as the height above the

Earth's surface of the

ANSWER: lowest broken or overcast layer or vertical visibility into

an obscuration.

A ceiling layer is not designated in

the METAR code. For aviation purposes, the ceiling is the

lowest broken or overcast layer, or vertical visibility into an

obscuration.

Figure 12

(Refer to figure 12.) What are the current conditions depicted for

Chicago Midway Airport (KMDW)?

ANSWER: Sky 700 feet overcast, visibility 1-1/2SM, rain.

At KMDW a special METAR (SPECI)

was taken at 1856Z and reported wind 320� at 5 kt., visibility

1� SM in moderate rain, overcast clouds at 700 ft.,

temperature 17�C, dew point 16�C, altimeter 29.80 in. Hg,

remarks follow, rain began at 35 min. past the hour.

Figure 12

(Refer to figure 12.) Which of the reporting stations have VFR

weather?

ANSWER: KINK, KBOI, and KLAX.

KINK is reporting visibility of 15 SM

and sky clear (15SM SKC); KBOI is reporting visibility of 30

SM and a scattered cloud layer base at 15,000 ft. (30SM

SCT150); and KLAX is reporting visibility of 6SM in mist

(fog) with a scattered cloud layer at 700 ft. and another one

at 25,000 ft. (6SM BR SCT007 SCT250). All of these

conditions are above VFR weather minimums of 1,000-ft.

ceiling and/or 3-SM visibility.

Figure 12

(Refer to figure 12.) The wind direction and velocity at KJFK is

from

ANSWER: 180� true at 4 knots.

The wind group at KJFK is coded as

18004KT. The first three digits are the direction the wind is

blowing from referenced to true north. The next two digits

are the speed in knots. Thus, the wind direction and speed

at KJFK are 180� true at 4 kt.

Figure 12

(Refer to figure 12.) What are the wind conditions at Wink, Texas

(KINK)?

ANSWER: 110� at 12 knots, gusts 18 knots.

The wind group at KINK is coded as

11012G18KT. The first three digits are the direction the wind

is blowing from referenced to true north. The next two digits

are the wind speed in knots. If the wind is gusty, it is

reported as a "G" after the speed followed by the highest (or

peak) gust reported. Thus, the wind conditions at KINK are

110� true at 12 kt., peak gust at 18 kt.

Figure 12

(Refer to figure 12.) The remarks section for KMDW has RAB35

listed. This entry means

ANSWER: rain began at 1835Z.

In the remarks (RMK) section for

KMDW, RAB35 means that rain began at 35 min. past the

hour. Since the report was taken at 1856Z, rain began at 35

min. past the hour, or 1835Z.

Figure 14

(Refer to figure 14.) If the terrain elevation is 1,295 feet MSL, what

is the height above ground level of the base of the ceiling?

ANSWER: 505 feet AGL.

Refer to the PIREP (identified by the

letters UA) in Fig. 14. The base of the ceiling is reported in

the sky cover (SK) section. The first layer is considered a

ceiling (i.e., broken) and the base is 1,800 ft. MSL. The

height above ground of the broken base is 505 ft. AGL (1,800

- 1,295).

Figure 14

(Refer to figure 14.) The base and tops of the overcast layer

reported by a pilot are

ANSWER: 7,200 feet MSL and 8,900 feet MSL.

Refer to the PIREP (identified by the

letters UA) in Fig. 14. The base and tops of the overcast

layer are reported in the sky conditions (identified by the

letters SK). This pilot has reported the base of the overcast

layer at 7,200 ft. and the top of the overcast layer at 8,900 ft.

(072 OVC 089). All altitudes are stated in MSL unless

otherwise noted. Thus, the base and top of the overcast

layer are reported as 7,200 ft. MSL and 8,900 ft. MSL,

respectively.

Figure 14

(Refer to figure 14.) The wind and temperature at 12,000 feet MSL

as reported by a pilot are

ANSWER: 080� at 21 knots and -7�C.

Refer to the PIREP (identified by the

letters UA) in Fig. 14. The wind is reported in the section

identified by the letters WV and is presented in five or six

digits. The temperature is reported in the section identified

by the letters TA in �C, and if below 0�C, prefixed with an

"M." The wind is reported as 080� at 21 kt. with a temperature of -7�C.

Figure 17

(Refer to figure 17.) Determine the wind and temperature aloft

forecast for MKC at 6,000 ft.

ANSWER: 200� true at 6 knots, temperature +3�C.

Refer to the FD forecast in Fig. 17.

Locate MKC on the left side of the chart and move to the

right to the 6,000-ft. column. The wind and temperature

forecast is coded as 2006+03, which translates as the

forecast wind at 200� true at 6 kt. and a temperature of 3�C.

Figure 17

(Refer to figure 17.) What wind is forecast for STL at 9,000 feet?

ANSWER: 230� true at 32 knots.

Refer to the FD forecast in Fig. 17.

Locate STL on the left side of the chart and move right to

the 9,000-ft. column. The coded wind forecast (first four

digits) is 2332. Thus, the forecast wind is 230� true at 32 kt.

To obtain a continuous transcribed weather briefing, including

winds aloft and route forecasts for a cross-country flight, a pilot

should monitor a

ANSWER: Transcribed Weather Broadcast (TWEB) on an NDB or a

VOR facility.

To obtain a continuous transcribed

weather briefing, including winds aloft and route forecasts

for a cross-country flight, a pilot should monitor a TWEB on

the ADF (low-frequency) radio receiver and/or the VOR.

SIGMET's are issued as a warning of weather conditions hazardous

to which aircraft?

ANSWER: All aircraft.

SIGMETs (significant meteorological

information) warn of weather considered potentially

hazardous to all aircraft. SIGMET advisories cover severe

and extreme turbulence; severe icing; and widespread

duststorms, sandstorms, or volcanic ash that reduce

visibility to less than 3 SM.

AIRMETs are advisories of significant weather phenomena but of

lower intensities than SIGMETs and are intended for dissemination

to

ANSWER: all pilots.

AIRMETs are advisories of

significant weather phenomena that describe conditions at

intensities lower than those which require the issuance of

SIGMETs. They are intended for dissemination to all pilots.

Which in-flight advisory would contain information on severe icing

not associated with thunderstorms?

ANSWER: SIGMET.

SIGMET advisories cover severe icing

not associated with thunderstorms; severe or extreme

turbulence or clear air turbulence not associated with

thunderstorms; dust-storms, sandstorms, or volcanic ash

that reduce visibility to less than 3 SM; and volcanic

eruption.

What information is contained in a CONVECTIVE SIGMET?

ANSWER: Tornadoes, embedded thunderstorms, and hail 3/4 inch or

greater in diameter.

Convective SIGMETs are issued for

tornadoes, lines of thunderstorms, embedded thunderstorms

of any intensity level, areas of thunderstorms greater than or

equal to VIP level 4 with an area coverage of 40% or more,

and hail � in. or greater.

What is indicated when a current CONVECTIVE SIGMET forecasts

thunderstorms?

ANSWER: Thunderstorms obscured by massive cloud layers.

Convective SIGMETs are issued for

tornadoes, lines of thunderstorms, embedded (i.e., obscured

by massive cloud layers) thunderstorms of any intensity

level, areas of thunderstorms greater than or equal to VIP

level 4 with an area coverage of 40% or more, and hail � in.

or greater.

Figure 18

(Refer to figure 18.) The IFR weather in northern Texas is due to

ANSWER: low ceilings.

Refer to the Weather Depiction Chart

in Fig. 18. The shaded area around northern Texas and

central Oklahoma indicates that IFR conditions exist. The

symbols "3=S" and "3=T" mean that the visibility is 3 SM in

fog (3=), and the sky is overcast at 600 ft. ( S ) to 800 ft. ( T )

AGL. Thus, low ceilings between 600-800 ft. are the source

of IFR weather conditions.

Figure 18

(Refer to figure 18.) What is the status of the front that extends

from Nebraska through the upper peninsula of Michigan?

ANSWER: Cold.

Refer to the Weather Depiction Chart

in Fig. 18. The front that extends from Nebraska through the

upper peninsula of Michigan is a cold front, as shown by

the pointed scallops on the southern side of the frontal line.

Figure 18

(Refer to figure 18.) Of what value is the Weather Depiction Chart

to the pilot?

ANSWER: For determining general weather conditions on which to

base flight planning.

The weather depiction chart is

prepared from surface aviation weather reports giving a

quick picture of weather conditions as of the time stated on

the chart. Thus, it presents general weather conditions on

which to base flight planning.

Figure 18

(Refer to figure 18.) What weather phenomenon is causing IFR

conditions in central Oklahoma?

ANSWER: Low ceilings and visibility.

Refer to the Weather Depiction Chart

in Fig. 18. In central Oklahoma, the IFR conditions are

caused by low ceilings and visibility. In the shaded area

over central Oklahoma and northern Texas, there are six

darkened circles with numbers ranging from one to eight

below them, signifying overcast skies with ceilings at 100 to

800 ft. The circles also have numbers ranging from 3/4 to 3

beside them, signifying visibilities between 3/4 and 3 statute

miles. The IFR conditions are therefore due to low ceilings

and visibility.

Figure 18

(Refer to figure 18.) According to the Weather Depiction Chart, the

weather for a flight from southern Michigan to north Indiana is

ceilings

ANSWER: greater than 3,000 feet and visibility greater than 5 miles.

Refer to the Weather Depiction Chart

in Fig. 18. The weather from southern Michigan to north

Indiana is shown by the lack of shading or contours to have

ceilings greater than 3,000 ft. and visibilities greater than 5

miles.

Figure 18

(Refer to figure 18.) The marginal weather in central Kentucky is

due to low

ANSWER: ceiling.

Refer to the Weather Depiction Chart

in Fig. 18. The MVFR weather in central Kentucky is

indicated by the contour line without shading. The station

symbol indicates an overcast ceiling at 3,000 ft. MVFR is

ceiling 1,000 ft. to 3,000 ft. and/or visibility 3 to 5 SM. Thus,

the marginal weather is due to a low ceiling.

Figure 19

(Refer to figure 19, area B.) What is the top for precipitation of the

radar return?

ANSWER: 24,000 feet MSL.

Refer to the Radar Summary Chart in

Fig. 19. The radar return at B (northern Nevada) has a "240"

with a line under it. This means the maximum top of the

precipitation is 24,000 ft. MSL.

What does the heavy dashed line that forms a large rectangular

box on a radar summary chart refer to?

ANSWER: Severe weather watch area.

On a Radar Summary Chart, severe

weather watch areas are outlined by heavy dashed lines.

Figure 19

(Refer to figure 19, area B.) What type of weather is occurring in

the radar return?

ANSWER: Continuous rain.

Refer to the Radar Summary Chart in

Fig. 19. The radar return around point B is labeled with an

"R." This means rain (R) that is light to moderate due to the

single contour, and is steady or continuous, due to the lack

of an intensity symbol (+ or -).

Figure 19

(Refer to figure 19, area D.) What is the direction and speed of

movement of the cell?

ANSWER: North at 17 knots.

Refer to the Radar Summary Chart in

Fig. 19. The radar return at D (Virginia) has an arrow pointing

north with "17" at the point. The movement is thus north at

17 kt.

Figure 19

(Refer to figure 19, Area E.) The top of the precipitation of the cell

is

ANSWER: 16,000 feet MSL.

Refer to the Radar Summary Chart in

Fig. 19. The cell <189> in. below point E (Virginia/North

Carolina) has a "160" with a line under it. This means the

maximum top of the precipitation is 16,000 ft. MSL.

What information is provided by the Radar Summary Chart that is

not shown on other weather charts?

ANSWER: Lines and cells of hazardous thunderstorms.

The Radar Summary Charts show

lines of thunderstorms and hazardous cells that are not

shown on other weather charts.

Radar weather reports are of special interest to pilots because they

indicate

ANSWER: location of precipitation along with type, intensity, and

cell movement of precipitation.

Radar weather reports are of special

interest to pilots because they report the location of

precipitation along with type, intensity, and cell movement.

Figure 20

(Refer to figure 20.) What weather is forecast for the Florida area

just ahead of the stationary front during the first 12 hours?

ANSWER: Ceiling 1,000 to 3,000 feet and/or visibility 3 to 5 miles with

continuous precipitation.

Refer to the Significant Weather

Prognostic Chart in Fig. 20. During the first 12 hr. (bottom

and top left panels), the weather just ahead of the stationary

front which extends from coastal Virginia into the Gulf of

Mexico is forecast to have ceilings from 1,000 to 3,000 ft.

and/or visibility 3 to 5 SM (as indicated by the scalloped

lines) with continuous light to moderate rain covering more

than half the area (as indicated by the shading).

Figure 20

(Refer to figure 20.) Interpret the weather symbol depicted in Utah

on the 12-hour Significant Weather Prognostic Chart.

ANSWER: Moderate turbulence, surface to 18,000 feet.

Refer to the upper left panel of the

Significant Weather Prog Chart in Fig. 20. In Utah, the

weather symbol indicates moderate turbulence as

designated by the symbol of a small peaked hat. Note that

the broken line indicates moderate or greater turbulence. The

peaked hat is the symbol for moderate turbulence. The 180

means the moderate turbulence extends from the surface

upward to 18,000 ft.

Figure 20

(Refer to figure 20.) At what altitude is the freezing level over the

middle of Florida on the 12-hour Significant Weather Prognostic

Chart?

ANSWER: 12,000 feet.

Refer to the upper left panel of the

Significant Weather Prog Chart in Fig. 20. On prog charts,

the freezing level is indicated by a dashed line, with the

height given in hundreds of feet MSL. In Fig. 20, there is a

dashed line across the middle of Florida, marked with "120"

just off the coast. This signifies that the freezing level is

12,000 ft. MSL.

The vertical limit of Class C airspace above the primary airport is

normally

ANSWER: 4,000 feet AGL.

The vertical limit (ceiling) of Class C

airspace is normally 4,000 ft. above the primary airport

elevation.

Under what condition may an aircraft operate from a satellite airport

within Class C airspace?

ANSWER: The pilot must contact ATC as soon as practicable after

takeoff.

Aircraft departing from a satellite

airport within Class C airspace with an operating control

tower must establish and maintain two-way radio

communication with the control tower and thereafter as

instructed by ATC. When departing a satellite airport

without an operating control tower, the pilot must contact

and maintain two-way radio communication with ATC as

soon as practicable after takeoff.

Figure 26

(Refer to figure 26, area 2.) The floor of Class B airspace at Addison

Airport is

ANSWER: 3,000 feet MSL.

Addison Airport (Fig. 26, area 2) has a

segmented blue circle around it depicting Class D airspace.

Addison Airport also underlies Class B airspace as depicted

by solid blue lines. The altitudes of the Class B airspace are

shown as

110

30

to the east of the airport. The bottom number denotes the

floor of the Class B airspace to be 3,000 ft. MSL.

Figure 26

(Refer to figure 26, area 4.) The floor of Class B airspace overlying

Hicks Airport (T67) north-northwest of Fort Worth Meacham Field is

ANSWER: 4,000 feet MSL.

Hicks Airport (T67) on Fig. 26 is

northeast of 4. Class B airspace is depicted by a solid blue

line, as shown just west of the airport. Follow the blue line

toward the bottom of the chart until you find a number over

a number in blue,

110

40

The bottom number denotes the floor of the Class B airspace

as 4,000 ft. MSL.

Responsibility for collision avoidance in an alert area rests with

ANSWER: all pilots.

Alert areas may contain a high volume

of pilot training or other unusual activity. Pilots using the

area as well as pilots crossing the area are equally

responsible for collision avoidance.

Under what condition, if any, may pilots fly through a restricted

area?

ANSWER: With the controlling agency's authorization.

An aircraft may not be operated

within a restricted area unless permission has been obtained

from the controlling agency. Frequently, the ATC within the

area acts as the controlling agent's authorization; e.g., an

approach control in a military restricted area can permit

aircraft to enter it when the restricted area is not active.

Figure 27

(Refer to figure 27.) What hazards to aircraft may exist in areas such

as Devils Lake East MOA?

ANSWER: Military training activities that necessitate acrobatic or

abrupt flight maneuvers.

Military Operations Areas (MOAs)

such as Devils Lake East in Fig. 27 consist of defined lateral

and vertical limits that are designated for the purpose of

separating military training activities from IFR traffic. Most

training activities necessitate acrobatic or abrupt flight

maneuvers. Therefore, the likelihood of a collision is

increased inside an MOA. VFR traffic is permitted, but extra

vigilance should be exercised in seeing and avoiding military

aircraft.

What action should a pilot take when operating under VFR in a

Military Operations Area (MOA)?

ANSWER: Exercise extreme caution when military activity is being

conducted.

Military operations areas consist of

airspace established for separating military training activities

from IFR traffic. VFR traffic should exercise extreme caution

when flying within an MOA. Information regarding MOA

activity can be obtained from flight service stations (FSSs)

within 100 mi. of the MOA.

Figure 21

(Refer to figure 21.) What hazards to aircraft may exist in restricted

areas such as R-5302B?

ANSWER: Unusual, often invisible, hazards such as aerial gunnery

or guided missiles.

The question asks what may exist in

restricted areas such as R-5302B (Fig. 21). Restricted areas

denote the existence of unusual, often invisible hazards to

aircraft such as military firing, aerial gunnery, or guided

missiles.

A non-tower satellite airport, within the same Class D airspace as

that designated for the primary airport, requires radio

communications be established and maintained with the

ANSWER: primary airport's control tower.

Each pilot departing a non-tower

satellite airport, within Class D airspace, must establish and

maintain two-way radio communications with the primary

airport's control tower as soon as practicable after departing.

The lateral dimensions of Class D airspace are based on

ANSWER: the instrument procedures for which the controlled

airspace is established.

The lateral dimensions of Class D

airspace are based upon the instrument procedures for

which the controlled airspace is established.

Prior to entering an Airport Advisory Area, a pilot should

ANSWER: contact the local FSS for airport and traffic advisories.

Airport Advisory Areas exist at

noncontrolled airports that have a Flight Service Station

(FSS) located on that airport. The FSS provides advisory

(not control) information on traffic, weather, etc., to

requesting aircraft. Accordingly, pilots should (not must)

contact FSSs for advisory services.

Figure 22

(Refer to figure 22, area 3.) What type military flight operations

should a pilot expect along IR 644?

ANSWER: IFR training flights above 1,500 feet AGL at speeds in

excess of 250 knots.

In Fig. 22, IR 644 is below area 3.

Military training flights are established to promote

proficiency of military pilots in the interest of national

defense. Military flight routes below 1,500 ft. are charted

with four-digit numbers; those above 1,500 ft. have

three-digit numbers. IR means the flights are made in

accordance with IFR. (VR would mean they use VFR.) Thus,

IR 644, a three-digit number, is above 1,500 ft., and flights

will be flown under IFR rules.

Automatic Terminal Information Service (ATIS) is the continuous

broadcast of recorded information concerning

ANSWER: noncontrol information in selected high-activity terminal

areas.

The continuous broadcast of

recorded noncontrol information is known as the Automatic

Terminal Information Service (ATIS). ATIS includes

weather, active runway, and other information that arriving

and departing pilots need to know.

Which initial action should a pilot take prior to entering Class C

airspace?

ANSWER: Contact approach control on the appropriate frequency.

Prior to entering Class C airspace, a

pilot must contact and establish communication with

approach control on the appropriate frequency.

TRSA Service in the terminal radar program provides

ANSWER: sequencing and separation for participating VFR aircraft.

TRSA service in the terminal radar

program provides sequencing and separation for all

participating VFR aircraft within the airspace defined as a

Terminal Radar Service Area (TRSA). Pilot participation is

urged but is not mandatory.

From whom should a departing VFR aircraft request radar traffic

information during ground operations?

ANSWER: Ground control, on initial contact.

Pilots of departing VFR aircraft are

encouraged to request radar traffic information by notifying

ground control on initial contact with their request and

proposed direction of flight.

Basic radar service in the terminal radar program is best described as

ANSWER: safety alerts, traffic advisories, and limited vectoring to

VFR aircraft.

Basic radar service in the terminal

radar program provides safety alerts, traffic advisories, and

limited vectoring (on a workload-permitting basis) to VFR

aircraft.

If Air Traffic Control advises that radar service is terminated when

the pilot is departing Class C airspace, the transponder should be

set to code

ANSWER: 1200.

The code 1200 designates VFR

operations when another number is not assigned by ATC.

When making routine transponder code changes, pilots should

avoid inadvertent selection of which codes?

ANSWER: 7500, 7600, 7700.

Some special codes set aside for

emergencies should be avoided during routine VFR flights.

They are 7500 for hijacking, 7600 for lost radio

communications, and 7700 for a general emergency.

Additionally, you should know that code 7777 is reserved

for military interceptors.

When operating under VFR below 18,000 feet MSL, unless

otherwise authorized, what transponder code should be selected?

ANSWER: 1200.

The standard VFR transponder code

is 1200. Since all flight operations above 18,000 ft. MSL are

to be IFR, code 1200 is not used above that height.

An ATC radar facility issues the following advisory to a pilot

flying on a heading of 090�:

"TRAFFIC 3 O'CLOCK, 2 MILES, WESTBOUND..."

Where should the pilot look for this traffic?

ANSWER: South.

If you receive traffic information

service from radar and are told you have traffic at the 3

o'clock position, traffic is in the direction of the right

wingtip, or to the south.

An ATC radar facility issues the following advisory to a pilot

flying on a heading of 360�:

"TRAFFIC 10 O'CLOCK, 2 MILES, SOUTHBOUND..."

Where should the pilot look for this traffic?

ANSWER: Northwest.

The controller is telling you that

traffic is at 10 o'clock and 2 mi. 9 o'clock is the left wingtip,

and 10 o'clock is 2/3 of the way from the nose of the airplane

(12 o'clock) to the left wingtip. Thus, you are looking

northwest.

An ATC radar facility issues the following advisory to a pilot

during a local flight:

"TRAFFIC 2 O'CLOCK, 5 MILES, NORTHBOUND..."

Where should the pilot look for this traffic?

ANSWER: Between directly ahead and 90� to the right.

The right wingtip is 3 o'clock, and the

nose is 12 o'clock. A controller report of traffic 2 o'clock, 5

mi., northbound indicates that the traffic is to the right of the

airplane's nose, just ahead of the right wingtip.

Figure 51

(Refer to figure 51.) Which runway and traffic pattern should be

used as indicated by the wind cone in the segmented circle?

ANSWER: Left-hand traffic on Runway 36.

The appropriate traffic pattern and

runway, given a wind from the northwest (Fig. 51), is

left-hand traffic on Runway 36, which would have a

quartering headwind.

Figure 50

(Refer to figure 50.) If the wind is as shown by the landing direction

indicator, the pilot should land on

ANSWER: Runway 18 and expect a crosswind from the right.

Given a wind as shown by the

landing direction indicator in Fig. 50, the pilot should land to

the south on Runway 18 and expect a crosswind from the

right. The tetrahedron points to the wind which is from the

southwest.

Figure 50

(Refer to figure 50.) Select the proper traffic pattern and runway for

landing.

ANSWER: Right-hand traffic and Runway 18.

The tetrahedron indicates wind

direction by pointing into the wind. On Fig. 50, Runways 4

and 22 are closed, as indicated by the X at each end of the

runway. Accordingly, with the wind from the southwest, the

landing should be made on Runway 18. Runway 18 has

right-hand traffic, as indicated by the traffic pattern indicator

at a 90� angle to the landing runway indicator in the

segmented circle.

After landing at a tower-controlled airport, when should the pilot

contact ground control?

ANSWER: When advised by the tower to do so.

After landing at a tower-controlled

airport, you should contact ground control on the

appropriate frequency only when instructed by the tower.

If instructed by ground control to taxi to Runway 9, the pilot may

proceed

ANSWER: via taxiways and across runways to, but not onto,

Runway 9.

A clearance to taxi to the active

runway means a pilot has been given permission to taxi via

taxiways and across intersecting runways to, but not onto,

the active runway.

Who should not participate in the Land and Hold Short Operations

(LAHSO) program?

ANSWER: Student pilots.

Land and hold short operations

(LAHSO) take place at some airports with an operating

control tower in order to increase the total capacity and

improve the flow of traffic. LAHSO requires that a pilot not

use the full length of the runway but, rather, that (s)he stop

and hold short before reaching an intersecting runway,

taxiway, or other specified point on the landing runway.

Student pilots or pilots who are not familiar with LAHSO

should not participate in the program.

Who has final authority to accept or decline any land and hold

short (LAHSO) clearance?

ANSWER: Pilot-in-command.

Land and hold short operations

(LAHSO) take place at some airports with an operating

control tower in order to increase the total capacity and

improve the flow of traffic. LAHSO requires that a pilot not

use the full length of the runway but, rather, that (s)he stop

and hold short before reaching an intersecting runway,

taxiway, or other specified point on the landing runway.

LAHSO requires familiarity with the available landing

distance (ALD) for given LAHSO combinations and with the

landing performance of the aircraft. The pilot in command

has the final authority to accept or decline any land and hold

short clearance.

When should pilots decline a land and hold short (LAHSO)

clearance?

ANSWER: When it will compromise safety.

Land and hold short operations

(LAHSO) take place at some airports with an operating

control tower in order to increase the total capacity and

improve the flow of traffic. LAHSO requires that a pilot not

use the full length of the runway but, rather, that (s)he stop

and hold short before reaching an intersecting runway,

taxiway, or other specified point on the landing runway.

LAHSO requires familiarity with the available landing

distance (ALD) for given LAHSO combinations and with the

landing performance of the aircraft. Pilots are expected to

decline a land and hold short clearance if they determine that

it will compromise safety.

Where is the "Available Landing Distance" (ALD) data published

for an airport that utilizes Land and Hold Short Operations

(LAHSO) published?

ANSWER: Airport/Facility Directory (A/FD).

Land and hold short operations

(LAHSO) take place at some airports with an operating

control tower in order to increase the total capacity and

improve the flow of traffic. LAHSO requires that a pilot not

use the full length of the runway but, rather, that (s)he stop

and hold short before reaching an intersecting runway,

taxiway, or other specified point on the landing runway.

LAHSO requires familiarity with the available landing

distance (ALD) for given LAHSO combinations and with the

landing performance of the aircraft. ALD data are published

in the special notices section of the Airport/Facility

Directory.

What is the minimum visibility for a pilot to receive a land and hold

short (LAHSO) clearance?

ANSWER: 3 statute miles.

You should receive a land and hold

short (LAHSO) clearance only when there is a minimum

ceiling of 1,000 ft. and visibility of 3 SM. The intent of

having basic VFR weather conditions is to allow pilots to

maintain visual contact with other aircraft and ground

vehicle operations.

What procedure is recommended when climbing or descending

VFR on an airway?

ANSWER: Execute gentle banks, left and right for continuous visual

scanning of the airspace.

When climbing (descending) VFR on

an airway, you should execute gentle banks left and right to

facilitate scanning for other aircraft. Collision avoidance is a

constant priority and especially pertinent to climbs and

descents on airways where other traffic is expected.

What ATC facility should the pilot contact to receive a special VFR

departure clearance in Class D airspace?

ANSWER: Air Traffic Control Tower.

When special VFR is needed, the pilot

should contact the Air Traffic Control Tower to receive a

departure clearance in Class D airspace.

Figure 52

(Refer to figure 52.) If more than one cruising altitude is intended,

which should be entered in block 7 of the flight plan?

ANSWER: Initial cruising altitude.

Use only your initial requested

altitude on your VFR flight plan to assist briefers in

providing weather and wind information.

Figure 52

(Refer to figure 52.) What information should be entered in block 9

for a VFR day flight?

ANSWER: The name of destination airport if no stopover for more

than 1 hour is anticipated.

In Block 9 of the flight plan form in

Fig. 52, enter the name of the airport of last intended landing

for that flight, as long as no stopover exceeds 1 hr.

Figure 52

(Refer to figure 52.) What information should be entered in block 12

for a VFR day flight?

ANSWER: The amount of usable fuel on board expressed in time.

Block 12 of the flight plan requires the

amount of usable fuel in the airplane at the time of departure.

It should be expressed in hours and minutes of flying time.

How should a VFR flight plan be closed at the completion of the

flight at a controlled airport?

ANSWER: The pilot must close the flight plan with the nearest FSS

or other FAA facility upon landing.

A pilot is responsible for ensuring

that the VFR or DVFR flight plan is canceled (FAR 91.153).

You should close your flight plan with the nearest FSS or, if

one is not available, you may request any ATC facility to

relay your cancellation to the FSS.

When activated, an emergency locator transmitter (ELT) transmits

on

ANSWER: 121.5 and 243.0 MHz.

When activated, an emergency

locator transmitter (ELT) transmits simultaneously on the

international distress frequencies of 121.5 and 243.0 MHz.

Which procedure is recommended to ensure that the emergency

locator transmitter (ELT) has not been activated?

ANSWER: Monitor 121.5 before engine shutdown.

To ensure that your ELT has not been

activated, you can monitor 121.5 MHz or 243.0 MHz in flight

when a receiver is available and prior to engine shut-down at

the end of each flight.

Pilots are more subject to spatial disorientation if

ANSWER: body signals are used to interpret flight attitude.

Spatial disorientation is a state of

temporary confusion resulting from misleading information

being sent to the brain by various sensory organs. Thus the

pilot should ignore sensations of muscles and inner ear and

kinesthetic senses (those which sense motion).

If a pilot experiences spatial disorientation during flight in a

restricted visibility condition, the best way to overcome the effect

is to

ANSWER: rely upon the aircraft instrument indications.

The best way to overcome the effects

of spatial disorientation is to rely entirely on the aircraft's

instrument indications and not upon body sensations. Sight

of the horizon also overrides inner ear sensations. Thus, in

areas of poor visibility, especially, such bodily signals

should be ignored.

The danger of spatial disorientation during flight in poor visual

conditions may be reduced by

ANSWER: having faith in the instruments rather than taking a

chance on the sensory organs.

Various complex motions and forces

and certain visual scenes encountered in flight can create

illusions of motion and position. Spatial disorientation from

these illusions can be prevented only by visual reference to

reliable fixed points on the ground and horizon or to flight

instruments.

A state of temporary confusion resulting from misleading

information being sent to the brain by various sensory organs is

defined as

ANSWER: spatial disorientation.

A state of temporary confusion

resulting from misleading information being sent to the brain

by various sensory organs is defined as vertigo (spatial

disorientation). Put simply, the pilot cannot determine

his/her relationship to the earth's horizon.

Which technique should a pilot use to scan for traffic to the right

and left during straight-and-level flight?

ANSWER: Systematically focus on different segments of the sky for

short intervals.

Due to the fact that eyes can focus

only on a narrow viewing area, effective scanning is

accomplished with a series of short, regularly spaced eye

movements that bring successive areas of the sky into the

central vision field.

What effect does haze have on the ability to see traffic or terrain

features during flight?

ANSWER: All traffic or terrain features appear to be farther away

than their actual distance.

Atmospheric haze can create the

illusion of being at a greater distance from traffic or terrain

than you actually are. This is especially prevalent on

landings.

What preparation should a pilot make to adapt the eyes for night

flying?

ANSWER: Avoid bright white lights at least 30 minutes before the

flight.

Prepare for night flying by letting

your eyes adapt to darkness, including avoiding bright

white light for at least 30 min. prior to night flight.

Large accumulations of carbon monoxide in the human body result

in

ANSWER: loss of muscular power.

Carbon monoxide reduces the ability

of the blood to carry oxygen. Large accumulations result in

loss of muscular power.

Susceptibility to carbon monoxide poisoning increases as

ANSWER: altitude increases.

Carbon monoxide poisoning results in

an oxygen deficiency. Since there is less oxygen available at

higher altitudes, carbon monoxide poisoning can occur with

lesser amounts of carbon monoxide as altitude increases.

An ATC clearance provides

ANSWER: authorization to proceed under specified traffic conditions

in controlled airspace.

A clearance issued by ATC is

predicated on known traffic and known physical airport

conditions. An ATC clearance means an authorization by

ATC, for the purpose of preventing collision between

known airplanes, for an airplane to proceed under specified

conditions within controlled airspace.

The letters VHF/DF appearing in the Airport/Facility Directory for a

certain airport indicate that

ANSWER: the Flight Service Station has equipment with which to

determine your direction from the station.

The VHF/Direction Finder (DF) facility

is a ground operation that displays the magnetic direction of

the airplane from the station each time the airplane

communication radio transmits a signal to it. It is used by

ATC and FSS to assist lost pilots by telling them which

direction they are from the receiving station.

Figure 23

(Refer to figure 23, area 2, and Legend 1.) For information about the

parachute jumping and glider operations at Silverwood Airport,

refer to

ANSWER: the Airport/Facility Directory.

The miniature parachute near the

Silverwood Airport (at 2 on Fig. 23) indicates a parachute

jumping area. In Legend 1, the symbol for a parachute

jumping area instructs you to see the Airport/Facility

Directory (A/FD) for more information. The A/FD will also

have information on the glider operations at Silverwood

Airport.

Figure 53

(Refer to figure 53.) When approaching Lincoln Municipal from the

west at noon for the purpose of landing, initial communications

should be with

ANSWER: Lincoln Approach Control on 124.0 MHz.

Fig. 53 contains the A/FD excerpt for

Lincoln Municipal. Locate the section titled Airspace and

note that Lincoln Municipal is located in Class C airspace.

The Class C airspace is in effect from 0530-0030 local time

(1130-0630Z). You should contact approach control (app

con) during that time before entering. Move up three lines to

App/Dep Con and note that aircraft arriving from the west of

Lincoln (i.e., 170� - 349�) at noon should initially contact

Lincoln Approach Control on 124.0.

Figure 53

(Refer to figure 53.) Which type radar service is provided to VFR

aircraft at Lincoln Municipal?

ANSWER: Sequencing to the primary Class C airport, traffic

advisories, conflict resolution, and safety alerts.

Fig. 53 contains the A/FD excerpt for

Lincoln Municipal. Locate the section titled Airspace to

determine that Lincoln Municipal is located in Class C

airspace. Once communications and radar contact are

established, VFR aircraft are provided the following services:

1. Sequencing to the primary airport

2. Approved separation between IFR and VFR aircraft

3. Basic radar services, i.e., safety alerts, limited vectoring,

and traffic advisories.

The FAA should change "conflict resolution" to "limited

vectoring" in the future.

Figure 53

(Refer to figure 53.) Traffic patterns in effect at Lincoln Municipal

are

ANSWER: to the left on Runway 17L and Runway 35L; to the right

on Runway 17R and Runway 35R.

Fig. 53 contains the A/FD excerpt for

Lincoln Municipal. For this question, you need to locate the

runway end data elements, i.e., Rwy 17R, Rwy 35L, Rwy 14,

Rwy 32, Rwy 17L, and Rwy 35R. Traffic patterns are to the

left unless right traffic is noted by the contraction Rgt tfc.

The only runways with right traffic are Rwy 17R and Rwy

35R.

Figure 53

(Refer to figure 53.) Where is Loup City Municipal located with

relation to the city?

ANSWER: Northwest approximately 1 mile.

Fig. 53 contains the A/FD excerpt for

Loup City Municipal. On the first line, the third item listed, 1

NW, means that Loup City Municipal is located

approximately 1 NM northwest of the city.

Figure 53

(Refer to figure 53.) What is the recommended communications

procedure for landing at Lincoln Municipal during the hours when

the tower is not in operation?

ANSWER: Monitor airport traffic and announce your position and

intentions on 118.5 MHz.

When the Lincoln Municipal tower is

closed, you should monitor airport traffic and announce

your position and intentions on the CTAF. Fig. 53 contains

the A/FD excerpt for Lincoln Municipal. Locate the section

titled Communications and note that on that same line the

CTAF frequency is 118.5.

Figure 26

(Refer to figure 26, area 7.) The airspace overlying Mc Kinney

(TKI) is controlled from the surface to

ANSWER: 2,900 feet MSL.

The airspace overlying Mc Kinney

airport (TKI) (Fig. 26, northeast of 7) is Class D airspace as

denoted by the segmented blue lines. The upper limit is

depicted in a broken box in hundreds of feet MSL to the left

of the airport symbol. The box contains the number "29,"

meaning that the vertical limit of the Class D airspace is 2,900

feet MSL.

Figure 26

(Refer to figure 26, area 4.) The airspace directly overlying Fort

Worth Meacham is

ANSWER: Class D airspace to 3,200 feet MSL.

The airspace overlying Fort Worth

Meacham (Fig. 26, southeast of 4) is Class D airspace as

denoted by the segmented blue lines. The upper limit is

depicted in a broken box in hundreds of feet MSL northeast

of the airport. Thus, the Class D airspace extends from the

surface to 3,200 ft. MSL.

Figure 26

(Refer to figure 26, area 8.) What minimum altitude is required to fly

over the Cedar Hill TV towers in the congested area south of NAS

Dallas?

ANSWER: 3,449 feet MSL.

The Cedar Hill TV towers (Fig. 26,

west of 8) have an elevation of 2,449 ft. MSL. The minimum

safe altitude over a congested area is 1,000 ft. above the

highest obstacle within a horizontal radius of 2,000 ft. of the

aircraft. Thus, to vertically clear the towers, the minimum

altitude is 3,449 ft. MSL (2,449 + 1,000).

Figure 22

(Refer to figure 22.) The terrain elevation of the light tan area

between Minot (area 1) and Audubon Lake (area 2) varies from

ANSWER: 2,000 feet to 2,500 feet MSL.

The requirement is the terrain

elevation in the tan area between 1 and 2 in Fig. 22. The tan

area indicates terrain between 2,000 ft. and 3,000 ft. The

elevation contours on sectionals vary by 500 ft. increments.

The 2,000-ft. contour line is located where the color changes

from light green to light tan. Since there is no other contour

line in the light tan area, the terrain elevation is between

2,000 ft. and 2,500 ft. MSL. Also, Poleschook Airport

(halfway between 1 and 2) indicates an elevation above MSL

of 2,245.

Figure 26

(Refer to figure 26, area 5.) The navigation facility at Dallas-Ft.

Worth International (DFW) is a

ANSWER: VOR/DME.

On Fig. 26, DFW is located at the

center of the chart and the navigation facility is 1 NM south

of the right set of parallel runways. The symbol is a hexagon

with a dot in the center within a square. This is the symbol

for a VOR/DME navigation facility.

Figure 21

(Refer to figure 21, area 2.) The flag symbol at Lake Drummond

represents a

ANSWER: visual checkpoint used to identify position for initial

callup to Norfolk Approach Control.

The magenta (reddish) flag (Fig. 21,

north of 2) at Lake Drummond signifies that the lake is a

visual checkpoint that can be used to identify the position

for initial callup to the Norfolk approach control.

Figure 21

(Refer to figure 21, area 5.) The CAUTION box denotes what hazard

to aircraft?

ANSWER: Unmarked balloon on cable to 3,000 feet MSL.

On Fig. 21, northwest of 5, find

"CAUTION: UNMARKED BALLOON ON CABLE TO 3,000

MSL." This is self-explanatory.

Figure 22

(Refer to figure 22.) Which public use airports depicted are

indicated as having fuel?

ANSWER: Minot Int'l (area 1) and Mercer County Regional Airport

(area 3).

On Fig. 22, the requirement is to

identify the airports having fuel available. Airports having

fuel available are designated by small squares extending

from the top, bottom, and both sides of the airport symbol.

Only Minot (area 1) and Mercer County Regional Airport

(area 3) have such symbols.

Figure 24

(Refer to figure 24.) The flag symbols at Statesboro Bullock County

Airport, Claxton-Evans County Airport, and Ridgeland Airport are

ANSWER: visual checkpoints to identify position for initial callup

prior to entering Savannah Class C airspace.

On Fig. 24, note the flag symbols at

Claxton-Evans County Airport (1 in. to the left of 2), at

Statesboro Bullock County Airport (2 in. above 2), and at

Ridgeland Airport (2 in. above 3). These airports are visual

checkpoints to identify position for initial callup prior to

entering the Savannah Class C airspace.

Figure 22

(Refer to figure 22.) On what frequency can a pilot receive

Hazardous Inflight Weather Advisory Service (HIWAS) in the

vicinity of area 1?

ANSWER: 117.1 MHz.

On Fig. 22, 1 is on the upper left and

the Minot VORTAC information box is 1 in. below 1.

Availability of Hazardous Inflight Weather Advisory Service

(HIWAS) will be indicated by a circle which contains an

"H," found in the upper right corner of a navigation

frequency box. Note that the Minot VORTAC information

box has such a symbol. Accordingly, a HIWAS can be

obtained on the VOR frequency of 117.1.

Figure 26

(Refer to figure 26, area 2.) The control tower frequency for

Addison Airport is

ANSWER: 126.0 MHz.

Addison Airport (Fig. 26, area 2)

control tower frequency is given as the first item in the

second line of the airport data to the right of the airport

symbol. The control tower (CT) frequency is 126.0 MHz.

What is it often called when a pilot pushes his or her capabilities

and the aircraft's limits by trying to maintain visual contact with the

terrain in low visibility and ceiling?

ANSWER: Scud running.

Scud running refers to a pilot's

pushing his/her capabilities and the aircraft's limits by trying

to maintain visual contact with the terrain while flying with a

low visibility or ceiling. Scud running is a dangerous (and

often illegal) practice that may lead to a mishap. This

dangerous tendency must be identified and eliminated.

What often leads to spatial disorientation or collision with

ground/obstacles when flying under Visual Flight Rules (VFR)?

ANSWER: Continual flight into instrument conditions.

Continuing VFR flight into instrument

conditions often leads to spatial disorientation or collision

with ground/obstacles due to the loss of outside visual

references. It is even more dangerous if the pilot is not

instrument qualified or current.

What is one of the neglected items when a pilot relies on short and

long term memory for repetitive tasks?

ANSWER: Checklists.

Neglect of checklists, flight planning,

preflight inspections, etc., is an indication of a pilot's

unjustified reliance on his/her short- and long-term memory

for repetitive flying tasks.

What is the antidote when a pilot has a hazardous attitude, such as

"Antiauthority"?

ANSWER: Follow the rules.

When you recognize a hazardous

thought, you should correct it by stating the corresponding

antidote. The antidote for the antiauthority ("Do not tell

me!") hazardous attitude is "Follow the rules; they are

usually right."

What is the antidote when a pilot has a hazardous attitude, such as

"Impulsivity"?

ANSWER: Not so fast, think first.

When you recognize a hazardous

thought, you should correct it by stating the corresponding

antidote. The antidote for the impulsivity ("Do something

quickly!") hazardous attitude is "Not so fast, think first."

What is the antidote when a pilot has a hazardous attitude, such as

"Invulnerability"?

ANSWER: It could happen to me.

When you recognize a hazardous

thought, you should correct it by stating the corresponding

antidote. The antidote for the invulnerability ("It will not

happen to me") hazardous attitude is "It could happen to

me."

What is the antidote when a pilot has a hazardous attitude, such as

"Macho"?

ANSWER: Taking chances is foolish.

When you recognize a hazardous

thought, you should correct it by stating the corresponding

antidote. The antidote for the macho ("I can do it")

hazardous attitude is "Taking chances is foolish."

What is the antidote when a pilot has a hazardous attitude, such as

"Resignation"?

ANSWER: I am not helpless.

When you recognize a hazardous

thought, you should correct it by stating the corresponding

antidote. The antidote for the resignation ("What is the

use?") hazardous attitude is "I am not helpless. I can make a

difference."

Who is responsible for determining whether a pilot is fit to fly for a

particular flight, even though he or she holds a current medical

certificate?

ANSWER: The pilot.

A number of factors, from lack of

sleep to an illness, can reduce a pilot's fitness to make a

particular flight. It is the responsibility of the pilot to

determine whether (s)he is fit to make a particular flight, even

though (s)he holds a current medical certificate.

Additionally, FAR 61.53 prohibits a pilot who possesses a

current medical certificate from acting as pilot in command,

or in any other capacity as a required pilot flight

crewmember, while the pilot has a known medical condition

or an aggravation of a known medical condition that would

make the pilot unable to meet the standards for a medical

certificate.

What is the one common factor which affects most preventable

accidents?

ANSWER: Human error.

Most preventable accidents, such as

fuel starvation or exhaustion, VFR flight into IFR conditions

leading to disorientation, and flight into known icing, have

one common factor: human error. Pilots who are involved in

accidents usually know what went wrong. In the interest of

expediency, cost savings, or other often irrelevant factors,

the wrong course of action (decision) was chosen.

FAA advisory circulars (some free, others at cost) are available to

all pilots and are obtained by

ANSWER: ordering those desired from the Government Printing

Office.

FAA Advisory Circulars are issued

with the purpose of informing the public of nonregulatory

material of interest. Free advisory circulars can be ordered

from the FAA, while those at cost can be ordered from the

Government Printing Office.

FAA advisory circulars containing subject matter specifically

related to Air Traffic Control and General Operations are issued

under which subject number?

ANSWER: 90.

FAA advisory circulars are numbered

based on the numbering system used in the FARs

60 Airmen

70 Airspace

90 Air Traffic Control and General Operation

FAA advisory circulars containing subject matter specifically

related to Airmen are issued under which subject number?

ANSWER: 60.

FAA advisory circulars are numbered

based on the numbering system used in the FARs

60 Airmen

70 Airspace

90 Air Traffic Control and General Operation

FAA advisory circulars containing subject matter specifically

related to Airspace are issued under which subject number?

ANSWER: 70.

FAA advisory circulars are numbered

based on the numbering system used in the FARs

60 Airmen

70 Airspace

90 Air Traffic Control and General Operation